# Table of Contents

Page

Section 4.46.	Berry Baskets and Boxes	4-39

A.	Application	4-39

A.1.	General.	4-39

A.2.	Additional Code Requirements.	4-39

S.	Specifications	4-39

S.1.	Units.	4-39

S.2.	Materials.	4-39

S.3.	Capacity Point.	4-39

N.	Notes	4-39

N.1.	Method of Test.	4-39

T.	Tolerances	4-39

T.1.	Tolerances on Individual Measures.	4-39

T.2.	Tolerances on Average Capacity.	4-39

THIS PAGE INTENTIONALLY LEFT BLANK

# Section 4.46.	Berry Baskets and Boxes

## A. Application

### A.1. General.

– This code applies to baskets and boxes for berries and small fruits in capacities of 1 dry quart and less.

### A.2. Additional Code Requirements.

– In addition to the requirements of this code, berry baskets and boxes shall meet the requirements of Section 1.10. General Code.

## S. Specifications

### S.1. Units.

– The capacity of a berry basket or box shall be ½ dry pint, 1 dry pint, or 1 dry quart.

### S.2. Materials.

– A berry basket or box shall be made of any suitable materials that will retain its shape during normal filling, storage, and handling.

### S.3. Capacity Point.

– The capacity of a berry basket or box shall be determined by its top edges.

## N. Notes

### N.1. Method of Test.

– A berry basket or box may be tested either volumetrically, using rape seed as the testing medium, or geometrically through accurate inside dimension measurement and calculation.

## T. Tolerances

### T.1. Tolerances on Individual Measures.

– Maintenance and acceptance tolerances in excess and deficiency on an individual measure shall be as shown in Table 1. Maintenance and Acceptance Tolerances in Excess and in Deficiency.

### T.2. Tolerances on Average Capacity.

– The average capacity on a random sample of 10 measures selected from a lot of 25 or more shall be equal to or greater than the nominal capacity.

(Amended 1979)

| Table 1. Maintenance and Acceptance Tolerances in Excess and in Deficiency |  |  |
| --- | --- | --- |
| Nominal Capacity | Tolerance |  |
|  | In Excess Cubic Inches | In Deficiency Cubic Inches |
| ½ pint | 1 | 0.5 |
| 1 pint | 2 | 1.0 |
| 1 quart | 3 | 1.5 |

THIS PAGE INTENTIONALLY LEFT BLANK