Table of Contents

Appendix C. General Tables of Units of Measurement	C-3

1.	Tables of Metric Units of Measurement	C-3

Units of Length	C-3

Units of Area	C-3

Units of Liquid Volume	C-4

Units of Volume	C-4

Units of Mass	C-4

2.	Tables of U.S. Customary Units of Measurement	C-4

Units of Length	C-4

Units of Area	C-5

Units of Volume	C-5

Gunter’s or Surveyors Chain Units of Measurement	C-5

Units of Liquid Volume	C-5

Apothecaries Units of Liquid Volume	C-5

Units of Dry Volume	C-6

Avoirdupois Units of Mass	C-6

Troy Units of Mass	C-6

Apothecaries Units of Mass	C-7

3.	Notes on British Units of Measurement	C-7

4.	Tables of Units of Measurement	C-8

Units of Length - International Measure	C-8

Units of Length - Survey Measure	C-8

Units of Area - International Measure	C-8

Units of Area - Survey Measure9	C-9

Units of Volume	C-10

Units of Capacity or Volume - Dry Volume Measure	C-10

Units of Capacity or Volume - Liquid Volume Measure	C-11

Units of Mass Not Less Than Avoirdupois Ounces	C-12

Units of Mass Not Greater Than Pounds and Kilograms	C-13

5.	Tables of Equivalents	C-14

Units of Length	C-14

Units of Area	C-16

Units of Capacity or Volume	C-16

Units of Mass	C-18

THIS PAGE INTENTIONALLY LEFT BLANK

# Appendix C. General Tables of Units of Measurement

These tables have been prepared for the benefit of those requiring tables of units for occasional ready reference.  In Section 4 of this Appendix, the tables are carried out to a large number of decimal places and exact values are indicated by underlining.  In most of the other tables, only a limited number of decimal places are given, therefore making the tables better adapted to the average user.

## Tables of Metric Units of Measurement

In the metric system of measurement, designations of multiples and subdivisions of any unit may be arrived at by combining with the name of the unit the prefixes deka, hecto, and kilo meaning, respectively, 10, 100, and 1000, and deci, centi, and milli, meaning, respectively, one-tenth, one-hundredth, and one-thousandth.  In some of the following metric tables, some such multiples and subdivisions have not been included for the reason that these have little, if any currency in actual usage.

In certain cases, particularly in scientific usage, it becomes convenient to provide for multiples larger than 1000 and for subdivisions smaller than one-thousandth.  Accordingly, the following prefixes have been introduced and these are now generally recognized:

yotta,	(Y)	meaning 1024	deci,	(d),	meaning 10-1

zetta,	(Z),	meaning 1021	centi,	(c),	meaning 10-2

exa,	(E),	meaning 1018	milli,	(m),	meaning 10-3

peta,	(P),	meaning 1015	micro,	(µ),	meaning 10-6

tera,	(T),	meaning 1012	nano,	(n),	meaning 10-9

giga,	(G),	meaning 109	pico,	(p),	meaning 10-12

mega,	(M),	meaning 106	femto,	(f),	meaning 10-15

kilo,	(k),	meaning 103	atto,	(a),	meaning 10-18

hecto,	(h),	meaning 102	zepto,	(z),	meaning 10-21

deka,	(da),	meaning 101	yocto,	(y),	meaning 10-24

Thus a kilometer is 1000 meters and a millimeter is 0.001 meter.

| ### Units of Length |  |
| --- | --- |
| 10 millimeters (mm) | = 1 centimeter (cm) |
| 10 centimeters | = 1 decimeter (dm) = 100 millimeters |
| 10 decimeters | = 1 meter (m) = 1000 millimeters |
| 10 meters | = 1 dekameter (dam) |
| 10 dekameters | = 1 hectometer (hm) = 100 meters |
| 10 hectometers | = 1 kilometer (km) = 1000 meters |

| ### Units of Area |  |
| --- | --- |
| 100 square millimeters (mm2) | = 1 square centimeter (cm2) |
| 100 square centimeters | = 1 square decimeter (dm2) |
| 100 square decimeters | = 1 square meter (m2) |
| 100 square meters | = 1 square dekameter (dam2) = 1 are |
| 100 square dekameters | = 1 square hectometer (hm2) = 1 hectare (ha) |
| 100 square hectometers | = 1 square kilometer (km2) |

| ### Units of Liquid Volume |  |
| --- | --- |
| 10 milliliters (mL) | = 1 centiliter (cL) |
| 10 centiliters | = 1 deciliter (dL) = 100 milliliters |
| 10 deciliters | = 1 liter = 1000 milliliters |
| 10 liters | = 1 dekaliter (daL) |
| 10 dekaliters | = 1 hectoliter (hL) = 100 liters |
| 10 hectoliters | = 1 kiloliter (kL) = 1000 liters |

| ### Units of Volume |  |
| --- | --- |
| 1000 cubic millimeters (mm3) | = 1 cubic centimeter (cm3) |
| 1000 cubic centimeters | = 1 cubic decimeter (dm3) |
|  | = 1 000 000 cubic millimeters |
| 1000 cubic decimeters | = 1 cubic meter (m3) |
|  | = 1 000 000 cubic centimeters |
|  | = 1 000 000 000 cubic millimeters |

| ### Units of Mass |  |
| --- | --- |
| 10 milligrams (mg) | = 1 centigram (cg) |
| 10 centigrams | = 1 decigram (dg) = 100 milligrams |
| 10 decigrams | = 1 gram (g) = 1000 milligrams |
| 10 grams | = 1 dekagram (dag) |
| 10 dekagrams | = 1 hectogram (hg) = 100 grams |
| 10 hectograms | = 1 kilogram (kg) = 1000 grams |
| 1000 kilograms | = 1 megagram (Mg) or 1 metric ton (t) |

## Tables of U.S. Customary Units of Measurement

In these tables where foot or mile is underlined, it is survey foot or U.S. statute mile rather than international foot or mile that is meant.

| ### Units of Length |  |
| --- | --- |
| 12 inches (in) | = 1 foot (ft) |
| 3 feet | = 1 yard (yd) |
| 16½ feet | = 1 rod (rd), pole, or perch |
| 40 rods | = 1 furlong (fur) = 660 feet |
| 8 furlongs | = 1 U.S. statute mile (mi) = 5280 feet |
| 1852 meters (m) | = 6076.115 49 feet (approximately) |
|  | = 1 international nautical mile |

| ### Units of Area |  |
| --- | --- |
| 144 square inches (in2) | = 1 square foot (ft2) |
| 9 square feet | = 1 square yard (yd2) |
|  | = 1296 square inches |
| 272¼ square feet | = 1 square rod (rd2) |
| 160 square rods | = 1 acre = 43 560 square feet |
| 640 acres | = 1 square mile (mi2) |
| 1 mile square | = 1 section of land |
| 6 miles square | = 1 township |
|  | = 36 sections = 36 square miles |

| Units of Volume3 |  |
| --- | --- |
| 1728 cubic inches (in3) | = 1 cubic foot (ft3) |
| 27 cubic feet | = 1 cubic yard (yd3) |

| ### Gunter’s or Surveyors Chain Units of Measurement |  |
| --- | --- |
| 0.66 foot (ft) | = 1 link (li) |
| 100 links | = 1 chain (ch) |
|  | = 4 rods = 66 feet |
| 80 chains | = 1 U.S. statute mile (mi) |
|  | = 320 rods = 5280 feet |

| ### Units of Liquid Volume |  |
| --- | --- |
| 4 gills (gi) | = 1 pint (pt) = 28.875 cubic inches (in3) |
| 2 pints | = 1 quart (qt) = 57.75 cubic inches |
| 4 quarts | = 1 gallon (gal) = 231 cubic inches |
|  | = 8 pints = 32 gills |

| ### Apothecaries Units of Liquid Volume |  |
| --- | --- |
| 60 minims | = 1 fluid dram (fl dr or Ӡ) |
|  | = 0.225 6 cubic inch (in3) |
| 8 fluid drams | = 1 fluid ounce (fl oz or ℥) |
|  | = 1.804 7 cubic inches |
| 16 fluid ounces | = 1 pint (pt) |
|  | = 28.875 cubic inches |
|  | = 128 fluid drams |
| 2 pints | = 1 quart (qt) = 57.75 cubic inches |
|  | = 32 fluid ounces = 256 fluid drams |
| 4 quarts | = 1 gallon (gal) = 231 cubic inches |
|  | = 128 fluid ounces = 1024 fluid drams |

| ### Units of Dry Volume |  |
| --- | --- |
| 2 pints (pt) | = 1 quart (qt) = 67.200 6 cubic inches (in3) |
| 8 quarts | = 1 peck (pk) = 537.605 cubic inches |
|  | = 16 pints |
| 4 pecks | = 1 bushel (bu) = 2150.42 cubic inches |
|  | = 32 quarts |

### Avoirdupois Units of Mass

[The “grain” is the same in avoirdupois, troy, and apothecaries units of mass.]

| 1 µlb | = 0.000 001 pound (lb) |
| --- | --- |
| 2711/32 grains (gr) | = 1 dram (dr) |
| 16 drams | = 1 ounce (oz) |
|  | = 437½ grains |
| 16 ounces | = 1 pound (lb) |
|  | = 256 drams |
|  | = 7000 grains |
| 100 pounds | = 1 hundredweight (cwt) |
| 20 hundredweights | = 1 ton (tn) |
|  | = 2000 pounds7 |

In “gross” or “long” measure, the following values are recognized:

| 112 pounds (lb) | = 1 gross or long hundredweight (cwt)7 |
| --- | --- |
| 20 gross or long hundredweights | = 1 gross or long ton |
|  | = 2240 pounds7 |

### Troy Units of Mass

[The “grain” is the same in avoirdupois, troy, and apothecaries units of mass.]

| 24 grains (gr) | = 1 pennyweight (dwt) |
| --- | --- |
| 20 pennyweights | = 1 ounce troy (oz t) = 480 grains |
| 12 ounces troy | = 1 pound troy (lb t) |
|  | = 240 pennyweights = 5760 grains |

### Apothecaries Units of Mass

[The “grain” is the same in avoirdupois, troy, and apothecaries units of mass.]

| 20 grains (gr) | = 1 scruple (s ap or ℈) |
| --- | --- |
| 3 scruples | = 1 dram apothecaries (dr ap or Ӡ) |
|  | = 60 grains |
| 8 drams apothecaries | = 1 ounce apothecaries (oz ap or ℥) |
|  | = 24 scruples = 480 grains |
| 12 ounces apothecaries | = 1 pound apothecaries (lb ap) |
|  | = 96 drams apothecaries |
|  | = 288 scruples = 5760 grains |

## Notes on British Units of Measurement

In Great Britain, the yard, the avoirdupois pound, the troy pound, and the apothecaries pound are identical with the units of the same names used in the United States.  The tables of British linear measure, troy mass, and apothecaries mass are the same as the corresponding United States tables, except for the British spelling “drachm” in the table of apothecaries mass.  The table of British avoirdupois mass is the same as the United States table up to 1 pound; above that point the table reads:

| 14 pounds | = 1 stone |
| --- | --- |
| 2 stones | = 1 quarter = 28 pounds |
| 4 quarters | = 1 hundredweight = 112 pounds |
| 20 hundredweight | = 1 ton = 2240 pounds |

The present British gallon and bushel – known as the “Imperial gallon” and “Imperial bushel” – are, respectively, about 20 % and 3 % larger than the United States gallon and bushel.  The Imperial gallon is defined as the volume of 10 avoirdupois pounds of water under specified conditions, and the Imperial bushel is defined as 8 Imperial gallons.  Also, the subdivision of the Imperial gallon as presented in the table of British apothecaries fluid measure differs in two important respects from the corresponding United States subdivision, in that the Imperial gallon is divided into 160 fluid ounces (whereas the United States gallon is divided into 128 fluid ounces), and a “fluid scruple” is included.  The full table of British measures of capacity (which are used alike for liquid and for dry commodities) is as follows:

| 4 gills | = 1 pint |
| --- | --- |
| 2 pints | = 1 quart |
| 4 quarts | = 1 gallon |
| 2 gallons | = 1 peck |
| 8 gallons (4 pecks) | = 1 bushel |
| 8 bushels | = 1 quarter |

The full table of British apothecaries measure is as follows:

| 20 minims | = 1 fluid scruple |
| --- | --- |
| 3 fluid scruples | = 1 fluid drachm |
|  | = 60 minims |
| 8 fluid drachms | = 1 fluid ounce |
| 20 fluid ounces | = 1 pint |
| 8 pints | = 1 gallon (160 fluid ounces) |

## Tables of Units of Measurement

(all underlined figures are exact)

### Units of Length - International Measure

| Units | Inches | Feet | Yards | Miles | Centimeters | Meters |
| --- | --- | --- | --- | --- | --- | --- |
| 1 inch = | 1 | 0.083 333 33 | 0.027 777 78 | 0.000 015 782 83 | 2.54 | 0.025 4 |
| 1 foot = | 12 | 1 | 0.333 333 3 | 0.000 189 393 9 | 30.48 | 0.304 8 |
| 1 yard = | 36 | 3 | 1 | 0.000 568 181 8 | 91.44 | 0.914 4 |
| 1 mile = | 63 360 | 5 280 | 1 760 | 1 | 160 934.4 | 1609.344 |
| 1 centimeter = | 0.393 700 8 | 0.032 808 40 | 0.010 936 13 | 0.000 006 213 712 | 1 | 0.01 |
| 1 meter = | 39.370 08 | 3.280 840 | 1.093 613 | 0.000 621 371 2 | 100 | 1 |

Units of Length - Survey Measure9

| Units | Links | Feet | Rods | Chains | Miles | Meters |
| --- | --- | --- | --- | --- | --- | --- |
| 1 link = | 1 | 0.66 | 0.04 | 0.01 | 0.000 125 | 0.201 168 4 |
| 1 foot = | 1.515 152 | 1 | 0.060 606 06 | 0.015 151 52 | 0.000 189 393 9 | 0.304 800 6 |
| 1 rod = | 25 | 16.5 | 1 | 0.25 | 0.003 125 | 5.029 210 |
| 1 chain = | 100 | 66 | 4 | 1 | 0.0125 | 20.116 84 |
| 1 mile = | 8 000 | 5 280 | 320 | 80 | 1 | 1609.347 |
| 1 meter = | 4.970 960 | 3.280 833 | 0.198 838 4 | 0.049 709 60 | 0.000 621 369 9 | 1 |

### Units of Area - International Measure

(all underlined figures are exact)

| Units | Square Inches | Square Feet | Square Yards |
| --- | --- | --- | --- |
| 1 square inch = | 1 | 0.006 944 444 | 0.000 771 604 9 |
| 1 square foot = | 144 | 1 | 0.111 111 1 |
| 1 square yard = | 1 296 | 9 | 1 |
| 1 square mile = | 4 014 489 600 | 27 878 400 | 3 097 600 |
| 1 square centimeter = | 0.155 000 3 | 0.001 076 391 | 0.000 119 599 0 |
| 1 square meter = | 1550.003 | 10.763 91 | 1.195 990 |
| Note: 1 survey foot = 1200/3937 meter (exactly) 1 international foot = 12 x 0.0254 meter (exactly) 1 international foot = 0.0254 x 39.37 survey foot (exactly) |  |  |  |

| Units | Square Miles | Square Centimeters | Square Meters |
| --- | --- | --- | --- |
| 1 square inch = | 0.000 000 000 249 097 7 | 6.451 6 | 0.000 645 16 |
| 1 square foot = | 0.000 000 035 870 06 | 929.030 4 | 0.092 903 04 |
| 1 square yard = | 0.000 000 322 830 6 | 8361.273 6 | 0.836 127 36 |
| 1 square mile = | 1 | 25 899 881 103.36 | 2 589 988.110 336 |
| 1 square centimeter = | 0.000 000 000 038 610 22 | 1 | 0.0001 |
| 1 square meter = | 0.000 000 386 102 2 | 10 000 | 1 |

### Units of Area - Survey Measure10,

| Units | Square Feet | Square Rods | Square Chains | Acres |
| --- | --- | --- | --- | --- |
| 1 square foot = | 1 | 0.003 673 095 | 0.000 229 568 4 | 0.000 022 956 84 |
| 1 square rod = | 272.25 | 1 | 0.062 5 | 0.006 25 |
| 1 square chain = | 4 356 | 16 | 1 | 0.1 |
| 1 acre = | 43 560 | 160 | 10 | 1 |
| 1 square mile = | 27 878 400 | 102 400 | 6 400 | 640 |
| 1 square meter = | 10.763 87 | 0.039 536 70 | 0.002 471 044 | 0.000 247 104 4 |
| 1 hectare = | 107 638.7 | 395.367 0 | 24.710 44 | 2.471 044 |

| Units | Square Miles | Square Meters | Hectares |
| --- | --- | --- | --- |
| 1 square foot = | 0.000 000 035 870 06 | 0.092 903 41 | 0.000 009 290 341 |
| 1 square rod = | 0.000 009 765 625 | 25.292 95 | 0.002 529 295 |
| 1 square chain = | 0.000 156 25 | 404.687 3 | 0.040 468 73 |
| 1 acre = | 0.001 562 5 | 4 046.873 | 0.404 687 3 |
| 1 square mile = | 1 | 2 589 998 | 258.999 8 |
| 1 square meter = | 0.000 000 386 100 6 | 1 | 0.000 1 |
| 1 hectare = | 0.003 861 006 | 10 000 | 1 |

Units of Volume

(all underlined figures are exact)

| Units | Cubic Inches | Cubic Feet | Cubic Yards |
| --- | --- | --- | --- |
| 1 cubic inch = | 1 | 0.000 578 703 7 | 0.000 021 433 47 |
| 1 cubic foot = | 1 728 | 1 | 0.037 037 04 |
| 1 cubic yard = | 46 656 | 27 | 1 |
| 1 cubic centimeter = | 0.061 023 74 | 0.000 035 314 67 | 0.000 001 307 951 |
| 1 cubic decimeter = | 61.023 74 | 0.035 314 67 | 0.001 307 951 |
| 1 cubic meter = | 61 023.74 | 35.314 67 | 1.307 951 |

| Units | Milliliters (Cubic Centimeters) | Liters (Cubic Decimeters) | Cubic Meters |
| --- | --- | --- | --- |
| 1 cubic inch = | 16.387 064 | 0.016 387 064 | 0.000 016 387 064 |
| 1 cubic foot = | 28 316.846 592 | 28.316 846 592 | 0.028 316 846 592 |
| 1 cubic yard = | 764 554.857 984 | 764.554 857 984 | 0.764 554 857 984 |
| 1 cubic centimeter = | 1 | 0.001 | 0.000 001 |
| 1 cubic decimeter = | 1 000 | 1 | 0.001 |
| 1 cubic meter = | 1 000 000 | 1000 | 1 |

### Units of Capacity or Volume - Dry Volume Measure

| Units | Dry Pints | Dry Quarts | Pecks | Bushels |
| --- | --- | --- | --- | --- |
| 1 dry pint = | 1 | 0.5 | 0.062 5 | 0.015 625 |
| 1 dry quart = | 2 | 1 | 0.125 | 0.031 25 |
| 1 peck = | 16 | 8 | 1 | 0.25 |
| 1 bushel = | 64 | 32 | 4 | 1 |
| 1 cubic inch = | 0.029 761 6 | 0.014 880 8 | 0.001 860 10 | 0.000 465 025 |
| 1 cubic foot = | 51.428 09 | 25.714 05 | 3.214 256 | 0.803 563 95 |
| 1 liter = | 1.816 166 | 0.908 083 0 | 0.113 510 4 | 0.028 377 59 |
| 1 cubic meter = | 1 816.166 | 908.083 0 | 113.510 4 | 28.377 59 |

| Units | Cubic Inches | Cubic Feet | Liters | Cubic Meters |
| --- | --- | --- | --- | --- |
| 1 dry pint = | 33.600 312 5 | 0.019 444 63 | 0.550 610 5 | 0.000 550 610 5 |
| 1 dry quart = | 67.200 625 | 0.038 889 25 | 1.101 221 | 0.001 101 221 |
| 1 peck = | 537.605 | 0.311 114 | 8.809 768 | 0.008 809 768 |
| 1 bushel = | 2 150.42 | 1.244 456 | 35.239 070 166 88 | 0.035 239 070 166 88 |
| 1 cubic inch = | 1 | 0.000 578 703 7 | 0.016 387 064 | 0.000 016 387 064 |
| 1 cubic foot = | 1728 | 1 | 28.316 846 592 | 0.028 316 846 592 |
| 1 liter = | 61.023 74 | 0.035 314 67 | 1 | 0.001 |
| 1 cubic meter = | 61 023.74 | 35.314 67 | 1000 | 1 |

### Units of Capacity or Volume - Liquid Volume Measure

(All underlined figures are exact)

| Units | Minims | Fluid Drams | Fluid Ounces | Gills |
| --- | --- | --- | --- | --- |
| 1 minim = | 1 | 0.016 666 67 | 0.002 083 333 | 0.000 520 833 3 |
| 1 fluid dram = | 60 | 1 | 0.125 | 0.031 25 |
| 1 fluid ounce = | 480 | 8 | 1 | 0.25 |
| 1 gill = | 1 920 | 32 | 4 | 1 |
| 1 liquid pint = | 7 680 | 128 | 16 | 4 |
| 1 liquid quart = | 15 360 | 256 | 32 | 8 |
| 1 gallon = | 61 440 | 1024 | 128 | 32 |
| 1 cubic inch = | 265.974 0 | 4.432 900 | 0.554 112 6 | 0.138 528 1 |
| 1 cubic foot = | 459 603.1 | 7660.052 | 957.506 5 | 239.376 6 |
| 1 milliliter = | 16.230 73 | 0.270 512 2 | 0.033 814 02 | 0.008 453 506 |
| 1 liter = | 16 230.73 | 270.512 2 | 33.814 02 | 8.453 506 |

| Units | Liquid Pints | Liquid Quarts | Gallons | Cubic Inches |
| --- | --- | --- | --- | --- |
| 1 minim = | 0.000 130 208 3 | 0.000 065 104 17 | 0.000 016 276 04 | 0.003 759 766 |
| 1 fluid dram = | 0.007 812 5 | 0.003 906 25 | 0.000 976 562 5 | 0.225 585 94 |
| 1 fluid ounce = | 0.062 5 | 0.031 25 | 0.007 812 5 | 1.804 687 5 |
| 1 gill = | 0.25 | 0.125 | 0.031 25 | 7.218 75 |
| 1 liquid pint = | 1 | 0.5 | 0.125 | 28.875 |
| 1 liquid quart = | 2 | 1 | 0.25 | 57.75 |
| 1 gallon = | 8 | 4 | 1 | 231 |
| 1 cubic inch = | 0.034 632 03 | 0.017 316 02 | 0.004 329 004 | 1 |
| 1 cubic foot = | 59.844 16 | 29.922 08 | 7.480 519 | 1 728 |
| 1 milliliter = | 0.002 113 376 | 0.001 056 688 | 0.000 264 172 1 | 0.061 023 74 |
| 1 liter = | 2.113 376 | 1.056 688 | 0.264 172 1 | 61.023 74 |

| Units | Cubic Feet | Milliliters | Liters |
| --- | --- | --- | --- |
| 1 minim = | 0.000 002 175 790 | 0.061 611 52 | 0.000 061 611 52 |
| 1 fluid dram = | 0.000 130 547 4 | 3.696 691 | 0.003 696 691 |
| 1 fluid ounce = | 0.001 044 379 | 29.573 53 | 0.029 573 53 |
| 1 gill = | 0.004 177 517 | 118.294 1 | 0.118 294 1 |
| 1 liquid pint = | 0.016 710 07 | 473.176 5 | 0.473 176 5 |
| 1 liquid quart = | 0.033 420 14 | 946.352 9 | 0.946 352 9 |
| 1 gallon = | 0.133 680 6 | 3785.411 784 | 3.785 411 784 |
| 1 cubic inch = | 0.000 578 703 7 | 16.387 06 | 0.016 387 06 |
| 1 cubic foot = | 1 | 28 316.85 | 28.316 85 |
| 1 milliliter = | 0.000 035 314 67 | 1 | 0.001 |
| 1 liter = | 0.035 314 67 | 1 000 | 1 |

### Units of Mass Not Less Than Avoirdupois Ounces

(all underlined figures are exact)

| Units | Avoirdupois Ounces | Avoirdupois Pounds | Short Hundred-weights | Short Tons |
| --- | --- | --- | --- | --- |
| 1 avoirdupois ounce = | 1 | 0.0625 | 0.000 625 | 0.000 031 25 |
| 1 avoirdupois pound = | 16 | 1 | 0.01 | 0.000 5 |
| 1 short hundredweight = | 1 600 | 100 | 1 | 0.05 |
| 1 short ton = | 32 000 | 2 000 | 20 | 1 |
| 1 long ton = | 35 840 | 2 240 | 22.4 | 1.12 |
| 1 kilogram = | 35.273 96 | 2.204 623 | 0.022 046 23 | 0.001 102 311 |
| 1 metric ton = | 35 273.96 | 2204.623 | 22.046 23 | 1.102 311 |

| Units | Long Tons | Kilograms | Metric Tons |
| --- | --- | --- | --- |
| 1 avoirdupois ounce = | 0.000 027 901 79 | 0.028 349 523 125 | 0.000 028 349 523 125 |
| 1 avoirdupois pound = | 0.000 446 428 6 | 0.453 592 37 | 0.000 453 592 37 |
| 1 short hundredweight = | 0.044 642 86 | 45.359 237 | 0.045 359 237 |
| 1 short ton = | 0.892 857 1 | 907.184 74 | 0.907 184 74 |
| 1 long ton = | 1 | 1016.046 908 8 | 1.016 046 908 8 |
| 1 kilogram = | 0.000 984 206 5 | 1 | 0.001 |
| 1 metric ton = | 0.984 206 5 | 1 000 | 1 |

### Units of Mass Not Greater Than Pounds and Kilograms

(all underlined figures are exact)

| Units | Grains | Apothecaries Scruples | Pennyweights | Avoirdupois Drams |
| --- | --- | --- | --- | --- |
| 1 grain = | 1 | 0.05 | 0.041 666 67 | 0.036 571 43 |
| 1 apoth. scruple = | 20 | 1 | 0.833 333 3 | 0.731 428 6 |
| 1 pennyweight = | 24 | 1.2 | 1 | 0.877 714 3 |
| 1 avdp. dram = | 27.343 75 | 1.367 187 5 | 1.139 323 | 1 |
| 1 apoth. dram = | 60 | 3 | 2.5 | 2.194 286 |
| 1 avdp. ounce = | 437.5 | 21.875 | 18.229 17 | 16 |
| 1 apoth. or troy oz. = | 480 | 24 | 20 | 17.554 29 |
| 1 apoth. or troy pound = | 5 760 | 288 | 240 | 210.651 4 |
| 1 avdp. pound = | 7 000 | 350 | 291.666 7 | 256 |
| 1 milligram = | 0.015 432 36 | 0.000 771 617 9 | 0.000 643 014 9 | 0.000 564 383 4 |
| 1 gram = | 15.432 36 | 0.771 617 9 | 0.643 014 9 | 0.564 383 4 |
| 1 kilogram = | 15432.36 | 771.617 9 | 643.014 9 | 564.383 4 |

| Units | Apothecaries Drams | Avoirdupois Ounces | Apothecaries or Troy Ounces | Apothecaries or Troy Pounds |
| --- | --- | --- | --- | --- |
| 1 grain = | 0.016 666 67 | 0.002 285 714 | 0.002 083 333 | 0.000 173 611 1 |
| 1 apoth. scruple = | 0.333 333 3 | 0.045 714 29 | 0.041 666 67 | 0.003 472 222 |
| 1 pennyweight = | 0.4 | 0.054 857 14 | 0.05 | 0.004 166 667 |
| 1 avdp. dram = | 0.455 729 2 | 0.062 5 | 0.56 966 15 | 0.004 747 179 |
| 1 apoth. dram = | 1 | 0.137 142 9 | 0.125 | 0.010 416 67 |
| 1 avdp. ounce = | 7.291 667 | 1 | 0.911 458 3 | 0.075 954 86 |
| 1 apoth. or troy ounce = | 8 | 1.097 143 | 1 | 0.083 333 333 |
| 1 apoth. or troy pound = | 96 | 13.165 71 | 12 | 1 |
| 1 avdp. pound = | 116.666 7 | 16 | 14.583 33 | 1.215 278 |
| 1 milligram = | 0.000 257 206 0 | 0.000 035 273 96 | 0.000 032 150 75 | 0.000 002 679 229 |
| 1 gram = | 0.257 206 0 | 0.035 273 96 | 0.032 150 75 | 0.002 679 229 |
| 1 kilogram = | 257.206 0 | 35.273 96 | 32.150 75 | 2.679 229 |

| Units | Avoirdupois Pounds | Milligrams | Grams | Kilograms |
| --- | --- | --- | --- | --- |
| 1 grain = | 0.000 142 857 1 | 64.798 91 | 0.064 798 91 | 0.000 064 798 91 |
| 1 apoth. scruple = | 0.002 857 143 | 1 295.978 2 | 1.295 978 2 | 0.001 295 978 2 |
| 1 pennyweight = | 0.003 428 571 | 1 555.173 84 | 1.555 173 84 | 0.001 555 173 84 |
| 1 avdp. dram = | 0.003 906 25 | 1 771.845 195 312 5 | 1.771 845 195 312 5 | 0.001 771 845 195 312 5 |
| 1 apoth. dram = | 0.008 571 429 | 3 887.934 6 | 3.887 934 6 | 0.003 887 934 6 |
| 1 avdp. ounce = | 0.062 5 | 28 349.523 125 | 28.349 523 125 | 0.028 349 523 125 |
| 1 apoth. or troy ounce = | 0.068 571 43 | 31 103.476 8 | 31.103 476 8 | 0.031 103 476 8 |
| 1 apoth. or troy pound = | 0.822 857 1 | 373 241.721 6 | 373.241 721 6 | 0.373 241 721 6 |
| 1 avdp. pound = | 1 | 453 592.37 | 453.592 37 | 0.453 592 37 |
| 1 milligram = | 0.000 002 204 623 | 1 | 0.001 | 0.000 001 |
| 1 gram = | 0.002 204 623 | 1 000 | 1 | 0.001 |
| 1 kilogram = | 2.204 623 | 1 000 000 | 1 000 | 1 |

## Tables of Equivalents

In these tables it is necessary to differentiate between the “international foot” and the “survey foot.”  Therefore, the survey foot is underlined.

When the name of a unit is enclosed in brackets (thus, [1 hand] . . . ), this indicates (1) that the unit is not in general current use in the United States, or (2) that the unit is believed to be based on “custom and usage” rather than on formal authoritative definition.

Equivalents involving decimals are, in most instances, rounded off to the third decimal place except where they are exact, in which cases these exact equivalents are so designated.  The equivalents of the imprecise units “tablespoon” and “teaspoon” are rounded to the nearest milliliter.

| ### Units of Length |  |
| --- | --- |
| angstrom () | 0.1 nanometer (exactly) 0.000 1 micrometer (exactly) 0.000 000 1 millimeter (exactly) 0.000 000 004 inch |
| 1 cable’s length | 120 fathoms (exactly) 720 feet (exactly) 219 meters |
| 1 centimeter (cm) | 0.393 7 inch |
| 1 chain (ch) (Gunter’s or surveyors) | 66 feet (exactly) 20.116 8 meters |
| 1 decimeter (dm) | 3.937 inches |
| 1 dekameter (dam) | 32.808 feet |
| 1 fathom | 6 feet (exactly) 1.828 8 meters |
| 1 foot (ft) | 0.304 8 meter (exactly) |
| 1 furlong (fur) | 10 chains (surveyors) (exactly) 660 feet (exactly) 1/8 U.S. statute mile (exactly) 201.168 meters |
| [1 hand] | 4 inches |
| 1 inch (in) | 2.54 centimeters (exactly) |
| 1 kilometer (km) | 0.621 mile |
| 1 league (land) | 3 U.S. statute miles (exactly) 4.828 kilometers |
| 1 link (li) (Gunter’s or surveyors) | 0.66 foot (exactly) 0.201 168 meter |
| 1 meter (m) | 39.37 inches 1.094 yards |
| 1 micrometer | 0.001 millimeter (exactly) 0.000 039 37 inch |
| 1 mil | 0.001 inch (exactly) 0.025 4 millimeter (exactly) |
| 1 mile (mi) (U.S. statute) | 5280 feet survey (exactly) 1.609 kilometers |
| 1 mile (mi) (international) | 5280 feet international (exactly) |
| 1 mile (mi) (international nautical) | 1.852 kilometers (exactly) 1.151 survey miles |
| 1 millimeter (mm) | 0.039 37 inch 0.001 meter (exactly) |
| 1 nanometer (nm) | 0.000 000 039 37 inch |
| 1 Point (typography) | 0.013 837 inch (exactly) 1/72 inch (approximately) 0.351 millimeter |
| 1 rod (rd), pole, or perch | 16½ feet (exactly) 5.029 2 meters |
| 1 yard (yd) | 0.914 4 meter (exactly) |

| ### Units of Area |  |
| --- | --- |
| 1 acre | 43 560 square feet (exactly) 0.405 hectare |
| 1 are | 119.599 square yards 0.025 acre |
| 1 hectare | 2.471 acres |
| [1 square (building)] | 100 square feet |
| 1 square centimeter (cm2) | 0.155 square inch |
| 1 square decimeter (dm2) | 15.500 square inches |
| 1 square foot (ft2) | 929.030 square centimeters |
| 1 square inch (in2) | 6.451 6 square centimeters (exactly) |
| 1 square kilometer (km2) | 247.104 acres 0.386 square mile |
| 1 square meter (m2) | 1.196 square yards 10.764 square feet |
| 1 square mile (mi2) | 258.999 hectares |
| 1 square millimeter (mm2) | 0.002 square inch |
| 1 square rod (rd2), sq pole, or sq perch | 25.293 square meters |
| 1 square yard (yd2) | 0.836 square meter |

| ### Units of Capacity or Volume |  |
| --- | --- |
| 1 barrel (bbl), liquid | 31 to 42 gallons |
| 1 barrel (bbl), standard for fruits, vegetables, and other dry commodities, except cranberries | 7056 cubic inches 105 dry quarts 3.281 bushels, struck measure |
| 1 barrel (bbl), standard, cranberry | 5826 cubic inches 8645/64 dry quarts 2.709 bushels, struck measure |
| 1 bushel (bu) (U.S.) struck measure | 2150.42 cubic inches (exactly) 35.238 liters |
| [1 bushel, heaped (U.S.)] | 2747.715 cubic inches 1.278 bushels, struck measure |
| [1 bushel (bu) (British Imperial) (struck measure)] | 1.032 U.S. bushels, struck measure 2219.36 cubic inches |
| 1 cord (cd) (firewood) | 128 cubic feet (exactly) |
| 1 cubic centimeter (cm3) | 0.061 cubic inch |
| 1 cubic decimeter (dm3) | 61.024 cubic inches |
| 1 cubic foot (ft3) | 7.481 gallons 28.316 cubic decimeters |
| 1 cubic inch (in3) | 0.554 fluid ounce 4.433 fluid drams 16.387 cubic centimeters |
| 1 cubic meter (m3) | 1.308 cubic yards |
| 1 cubic yard (yd3) | 0.765 cubic meter |
| 1 cup, measuring | 8 fluid ounces (exactly) 237 milliliters ½ liquid pint (exactly) |
| 1 dekaliter (daL) | 2.642 gallons 1.135 pecks |
| 1 dram, fluid (or liquid) (fl dr) or Ӡ) (U.S.) | 1/8 fluid ounce (exactly) 0.226 cubic inch 3.697 milliliters 1.041 British fluid drachms |
| [1 drachm, fluid (fl dr) (British)] | 0.961 U.S. fluid dram 0.217 cubic inch 3.552 milliliters |
| 1 gallon (gal) (U.S.) | 231 cubic inches (exactly) 3.785 liters 0.833 British gallon 128 U.S. fluid ounces (exactly) |
| [1 gallon (gal) (British Imperial)] | 277.42 cubic inches 1.201 U.S. gallons 4.546 liters 160 British fluid ounces (exactly) |
| 1 gill (gi) | 7.219 cubic inches 4 fluid ounces (exactly) 0.118 liter |
| 1 hectoliter (hL) | 26.418 gallons 2.838 bushels |
| 1 liter (1 cubic decimeter exactly) | 1.057 liquid quarts 0.908 dry quart 61.024 cubic inches |
| 1 milliliter (mL) | 0.271 fluid dram 16.231 minims 0.061 cubic inch |
| 1 ounce, fluid (or liquid) (fl oz) or ) (U.S.) | 1.805 cubic inches 29.573 milliliters 1.041 British fluid ounces |
| [1 ounce, fluid (fl oz) (British)] | 0.961 U.S. fluid ounce 1.734 cubic inches 28.412 milliliters |
| 1 peck (pk) | 8.810 liters |
| 1 pint (pt), dry | 33.600 cubic inches 0.551 liter |
| 1 pint (pt), liquid | 28.875 cubic inches exactly 0.473 liter |
| 1 quart (qt), dry (U.S.) | 67.201 cubic inches 1.101 liters 0.969 British quart |
| 1 quart (qt), liquid (U.S.) | 57.75 cubic inches (exactly) 0.946 liter 0.833 British quart |
| [1 quart (qt) (British)] | 69.354 cubic inches 1.032 U.S. dry quarts 1.201 U.S. liquid quarts |
| 1 tablespoon, measuring | 3 teaspoons (exactly) 15 milliliters 4 fluid drams ½ fluid ounce (exactly) |
| 1 teaspoon, measuring | ⅓ tablespoon (exactly) 5 milliliters 1⅓ fluid drams |
| 1 water ton (English) | 270.91 U.S. gallons 224 British Imperial gallons (exactly) |

| ### Units of Mass |  |
| --- | --- |
| 1 assay ton (AT) | 29.167 grams |
| 1 carat (c) | 200 milligrams (exactly) 3.086 grains |
| 1 dram apothecaries (dr ap or 3) | 60 grains (exactly) 3.888 grams |
| 1 dram avoirdupois (dr avdp) | 2711/32 (= 27.344) grains 1.772 grams |
| 1 gamma (γ) | 1 microgram (exactly) |
| 1 grain | 64.798 91 milligrams (exactly) |
| 1 gram (g) | 15.432 grains 0.035 ounce, avoirdupois |
| 1 hundredweight, gross or long (gross cwt) | 112 pounds (exactly) 50.802 kilograms |
| 1 hundredweight, gross or short (cwt or net cwt) | 100 pounds (exactly) 45.359 kilograms |
| 1 kilogram (kg) | 2.205 pounds |
| 1 milligram (mg) | 0.015 grain |
| 1 ounce, avoirdupois (oz avdp) | 437.5 grains (exactly) 0.911 troy or apothecaries ounce 28.350 grams |
| 1 ounce, troy or apothecaries (oz t or oz ap or ℥) | 480 grains (exactly) 1.097 avoirdupois ounces 31.103 grams |
| 1 pennyweight (dwt) | 1.555 grams |
| 1 point | 0.01 carat 2 milligrams |
| 1 pound, avoirdupois (lb avdp) | 7000 grains (exactly) 1.215 troy or apothecaries pounds 453.592 37 grams (exactly) |
| 1 micropound (µlb) [the Greek letter mu in combination with the letters lb] | 0.000 001 pound (exactly) |
| 1 pound, troy or apothecaries (lb t or lb ap) | 5760 grains (exactly) 0.823 avoirdupois pound 373.242 grams |
| 1 scruple (s ap or ℈) | 20 grains (exactly) 1.296 grams |
| 1 ton, gross or long | 2240 pounds (exactly) 1.12 net tons (exactly) 1.016 metric tons |
| 1 ton, metric (t) | 2204.623 pounds 0.984 gross ton 1.102 net tons |
| 1 ton, net or short (tn)21 | 2000 pounds (exactly) 0.893 gross ton 0.907 metric ton |

THIS PAGE INTENTIONALLY LEFT BLANK

By action of the 12th General Conference on Weights and Measures (1964), the liter is a special name for the cubic decimeter.

This section lists units of measurement that have traditionally been used in the United States.  In keeping with the Omnibus Trade and Competitiveness Act of 1988, the ultimate objective is to make the International System of Units the primary measurement system used in the United States.

Squares and cubes of customary but not of metric units are sometimes expressed by the use of abbreviations rather than symbols.  For example, sq ft means square foot, and cu ft means cubic foot.

When necessary to distinguish the liquid pint or quart from the dry pint or quart, the word “liquid” or the abbreviation “liq” should be used in combination with the name or abbreviation of the liquid unit.

When necessary to distinguish dry pint or quart from the liquid pint or quart, the word “dry” should be used in combination with the name or abbreviation of the dry unit.

6 When necessary to distinguish the avoirdupois dram from the apothecaries dram, or to distinguish the avoirdupois dram or ounce from the fluid dram or ounce, or to distinguish the avoirdupois ounce or pound from the troy or apothecaries ounce or pound, the word “avoirdupois” or the abbreviation “avdp” should be used in combination with the name or abbreviation of the avoirdupois unit.

7 When the terms “hundredweight” and “ton” are used unmodified, they are commonly understood to mean the 100-pound hundredweight and the 2000-pound ton, respectively; these units may be designated “net” or “short” when necessary to distinguish them from the corresponding units in gross or long measure.

As of January 1, 2014, “tn” is the required abbreviation for “short ton.”  Devices manufactured between January 1, 2008, and December 31, 2013, may use an abbreviation other than “tn” to specify “short ton.”

One international foot		= 0.999 998 survey foot (exactly)

One international mile		= 0.999 998 survey mile (exactly)

One square survey foot				= 1.000 004 square international feet

One square survey mile				= 1.000 004 square international miles

10 One square survey foot				= 1.000 004 square international feet

One square survey mile		= 1.000 004 square international miles

One international foot		= 0.999 998 survey foot (exactly)

One international mile		= 0.999 998 survey mile (exactly)

The angstrom is basically defined as 10−10 meter.

The term “statute mile” originated with Queen Elizabeth I who changed the definition of the mile from the Roman mile of 5000 feet to the statute mile of 5280 feet.  The international mile and the U.S. statute mile differ by about 3 millimeters although both are defined as being equal to 5280 feet.  The international mile is based on the international foot (0.3048 meter) whereas the U.S. statute mile is based on the survey foot (1200/3937 meter).

The international nautical mile of 1852 meters (6076.115 49 feet) was adopted effective July 1, 1954, for use in the United States.  The value formerly used in the United States was 6080.20 feet = 1 nautical (geographical or sea) mile.

The question is often asked as to the length of a side of an acre of ground.  An acre is a unit of area containing 43 560 square feet.  It is not necessarily square, or even rectangular.  But, if it is square, then the length of a side is equal to (not exact).

There are a variety of “barrels” established by law or usage.  For example, federal taxes on fermented liquors are based on a barrel of 31 gallons; many state laws fix the “barrel for liquids” as 31½ gallons; one state fixes a 36-gallon barrel for cistern measurement; federal law recognizes a 40-gallon barrel for “proof spirits;” by custom, 42 gallons comprise a barrel of crude oil or petroleum products for statistical purposes, and this equivalent is recognized “for liquids” by four states.

Frequently recognized as 1¼ bushels, struck measure.

The equivalent “1 teaspoon = 1⅓ fluid drams” has been found by the Bureau to correspond more closely with the actual capacities of “measuring” and silver teaspoons than the equivalent “1 teaspoon = 1 fluid dram,” which is given by a number of dictionaries.

Used in assaying.  The assay ton bears the same relation to the milligram that a ton of 2000 pounds avoirdupois bears to the ounce troy; hence the mass in milligrams of precious metal obtained from one assay ton of ore gives directly the number of troy ounces to the net ton.

The gross or long ton and hundredweight are used commercially in the United States to only a very limited extent, usually in restricted industrial fields.  The units are the same as the British “ton” and the “hundredweights.”

21 As of January 1, 2014, “tn” is the required abbreviation for “short ton.”  Devices manufactured between January 1, 2008, and December 31, 2013, may use an abbreviation other than “tn” to specify “short ton.”