FS-TST: Forensic Software Testing Support Tools

Requirements, Design Notes and User Manual

Version 1.0

May 2002

# Abstract†

This document describes Release 1.0 of a software package developed to aid the testing of disk imaging tools typically used in forensic investigations. The package includes programs that use the interrupt 13h BIOS disk interface to initialize disk drives, detect changes in disk content, compare pairs of disks, and simulate bad sectors on a disk. Most of the software is written in Borland C++ 4.5 with a few parts written in Borland Assembler. The software can be used in the DOS 6.3 environment to setup disk drives for tests, measure the results of a test and aid in documenting test runs.

The intended audience for this document should be familiar with the DOS operating system, computer operation, computer hardware components such as hard drives, hard drive interfaces (e.g., IDE or SCSI) and computer forensics. A working knowledge of C and Assembly programming is not necessary for understanding but would be helpful.

Turbo Assembler™ is a trademark of Borland International, Inc.

Borland® is a registered trademark of Borland International, Inc.

Linux™ is a trademark of Linus Torvalds.

MS-DOS® is a registered trademark of Microsoft Corporation, Inc.

Pentium® is a registered trademark of Intel, Inc.

All other products mentioned herein may be trademarks of their respective companies.

Abstract	iii

Table of Figures	vii

List of Tables	vii

1	Introduction	1

1.1	How to read this document	1

1.2	Capabilities Required to Support Testing Disk Imaging Tools	2

1.3	Summary of Required Capabilities	3

1.4	Test Case Structure	4

1.5	Software Overview	4

2	Requirements	5

2.1	Requirements for Common Functionality	5

2.2	Individual Program Requirements	7

3	Design Notes	12

3.1	Data Structures	12

3.2	Library Functions Designs	14

3.3	Individual Program Designs	17

4	User Manual	19

4.1	System Environment	19

4.2	Log files	19

4.3	Error Messages	21

4.4	Disk initialization: DISKWIPE	21

4.5	Faulty Disk Simulation	22

4.6	Corrupt an image file: CORRUPT	23

4.7	Comparison Software: ADJCMP, DISKCMP and PARTCMP	25

4.8	Verify no changes to a disk: DISKHASH	30

4.9	Verify no changes to a block of sectors: SECHASH	31

4.10	Log a test case: LOGCASE	33

4.11	Log Disk Setup: LOGSETUP	33

4.12	Print a partition table: PARTAB	33

4.13	Examine or Change a Disk: DISKCHG	34

4.14	Compare two disk sectors: SECCMP	35

4.15	Copy a block of sectors: SECCOPY	36

5	Example Test Case	37

5.1	Pre-test preparations	38

5.2	Setup	39

5.3	Execute	41

5.4	Measure	42

# Table of Figures

Figure 1 General Structure of a Test Case	4

Figure 2 List of Support Programs	5

Figure 3  Data Structures for Describing a Hard Drive	12

Figure 4 Data Structures for Describing a Partition Table	13

Figure 5 Data Structures for Describing a Range of Sectors	14

Figure 6 General Program Design for Support Software	17

Figure 7 TSR Program Design	17

Figure 8 ADJCMP Summary Report Example	26

# List of Tables

Table 1 Reader's Guide	2

Table 2 DISKWIPE Pattern Specification	8

Table 3 Program Log Files	20

Table 4 DISKWIPE Command Line Parameters	22

Table 5 BADDISK Command Line Parameters	23

Table 6 BADX13 Command Line Parameters	23

Table 7 CORRUPT Command Line Parameters	24

Table 8 Comparison Programs	25

Table 9ADJCMP Command Line Parameters	27

Table 10 DISKCMP Command Line Parameters	28

Table 11 PARTCMP Command Line Parameters	29

Table 12 DISKHASH Command Line Parameters	30

Table 13 SECHASH Command Line Parameters	32

Table 14 LOGCASE Command Line Parameters	33

Table 15 LOGSETUP Command Line Parameters	33

Table 16 PARTAB Command Line Parameters	34

Table 17 DISKCHG Command Line Parameters	35

Table 18 SECCMP Command Line Parameters	35

Table 19 SECCOPY Command Line Parameters	37

Table 20 Example Test Case Specification	38

Table 21 Commands Used During Test Setup	39

Table 22 Example Test Case Setup	40

Table 23 Commands Used During Tool Execution	41

Table 24 Example Test Case Execution	41

Table 25 Commands Used to Measure Test Results	42

Table 26 Example Test Case Measurement	42

# Introduction

The Computer Forensics Tool Testing project at the National Institute of Standards and Technology (NIST), an agency of the United States Department of Commerce, provides a measure of confidence in the software tools used in computer forensics investigations. This work stems from an agreement between NIST Office of Law Enforcement Standards (OLES), NIST Software Diagnostics and Conformance Testing Division, the National Institute of Justice, Federal Bureau of Investigation, DoD Computer Forensics Lab, United States Customs Service and the Technical Support Working Group (TSWG).

This document describes Release 1.0 of FS-TST, a software package that supports the testing of disk imaging tools. The package includes programs that use the interrupt 13h BIOS disk interface to initialize disk drives, detect changes in disk content, compare pairs of disks, and simulate bad sectors on a disk. Most of the software is written in Borland C++ 4.5 with a few parts written in Borland Assembler. The software can be used in the DOS 6.3 environment to setup disk drives for tests, measure the results of a test and aid in documenting test runs. A set of test cases for disk imaging tools is described in Disk Imaging Tool Specification, Version 3.1.6 (see http://www.cftt.nist.gov/DI-spec-3-1-6.doc).

## How to read this document

The intended audience for this document should be familiar with the DOS operating system, computer operation, computer hardware components such as hard drives, hard drive interfaces (e.g., IDE or SCSI) and computer forensics. A working knowledge of C and Assembly programming is not necessary for understanding but would be helpful. We expect that the primary objectives of the reader to be some combination of the following:

- The reader wants a general understanding of FS-TST.

- The reader wants to use FS-TST as is.

- The reader wants to use FS-TST after enhancement or modification.

- The reader wants to verify or test the operation of FS-TST

The remainder of this section describes the capabilities required to test disk imaging tools and lists the components of FS-TST. The goal is to identify capabilities required for testing disk imaging tools and allocate the required capabilities to the programs of the FS-TST package.

Section 2 documents the requirements of each program. This section gives a concise description of what each program is supposed to do.

The program designs are discussed in Section 3. This section should be read in conjunction with the program listings. The objective of this section is to give the reader a high level view of the program design to help understand the source code.

Section 4 is a user manual and Section 5 describes using the FS-TST tools on an example test case.

Not all the document is of interest to every reader. The goals of the reader determine the relevant sections of the document. A bullet (●) in Table 1 indicates the relevant sections given the goals of the reader.

Table 1 Reader's Guide

| Reader’s Goal | Section |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 |
| General understanding of FS-TST | ● |  |  |  | ● |
| Use FS-TST | ● |  |  | ● | ● |
| Enhance or modify FS-TST | ● |  | ● | ● |  |
| Verify or test the operation of FS-TST | ● | ● |  | ● |  |

## Capabilities Required to Support Testing Disk Imaging Tools

The usual testing paradigm is a three step process.

- Setup the test.

- Execute the test

- Measure the results.

This section establishes the capabilities required to setup and measure each test. This is accomplished by first determining what must be measured to confirm compliance with the tool requirements and second what conditions must be created to allow an accurate measurement of compliance.

The basic function of a disk imaging tool is to create a duplicate of either a hard disk drive or a partition from a hard disk drive. A disk imaging tool copies disk sectors from a source to a destination such that the destination is identical or nearly identical to the source. In the ideal situation, the destination is identical to the original. However, forensically valid differences do occur when the source and destination are not the same size or if partitions from the source disk must be relocated on the destination drive to keep the destination partitions accessible. The critical disk imaging requirements are the following:

- The tool shall not alter the original disk.

- The tool shall make a bit-stream duplicate or an image of an original disk or partition.

- The tool shall log errors (or other conditions of note).

### Source Disk is Unchanged

The source disk must not be altered by running the disk imaging tool. One approach for detecting any changes to a disk is to compute a hash function of the entire contents of a disk both before and after the disk imaging tool is run. The likelihood that two disk drives with different content would produce the same hash value is related to the length of the hash function value. The secure hash algorithm (SHA-1) produces a 160 bit hash. This gives a 1 in 280 chance that any change to the source by the tool would be undetected. The hashing technique can also be used to verify that a block of sectors has not been changed by some operation.

### Duplicate Creation

A program to compare the corresponding sectors of two disks can determine the accuracy or point of failure of the duplication process. If the destination is larger than the source, then there will be excess sectors on the destination. An analysis of the excess destination sectors is useful to determine tool behavior if the destination is larger than the source. However, to do such an analysis, the source and destination sectors need to be uniquely recognizable from the sector contents. If every sector of each disk used in a test is initialized to a known unique value then the task of determining the origin of each sector of the destination during the measurement of the results of a test run is simplified. When a destination sector is examined there are three possibilities for the contents of the sector: the original destination contents are unchanged by the test, the contents of a sector from the source disk is written to the destination or the imaging tool has generated new and unexpected contents for the sector.

Another factor to consider in duplicate creation is the accessibility of the duplicate. It may be desirable to ensure that the information copied to a disk with a different number of cylinders and heads can be accessed or booted. To create a bootable copy of the source, the partitions of the destination need to be aligned on cylinder boundaries. If there is more than one partition on the source then the destination partitions may be padded with extra sectors to create a bootable destination.

### Errors

Since forensic evidence may need to be obtained from a faulty disk, the behavior and capabilities of disk imaging tools in the presence of disk I/O errors must be evaluated. The problem is how to have a reliably bad disk (i.e., a disk that can produce a variety of failure modes and I/O errors useful to a planned test). One solution is to use a correctly functioning disk, but intercept disk I/O requests such that a faulty disk can be simulated to the disk imaging tool according to each test case specification.

## Summary of Required Capabilities

The minimum set of basic capabilities required to support testing a disk imaging tool is the following:

- Initialize each sector of a disk to a known unique value.

- Compute a SHA-1 hash of an entire disk or a block of disk sectors.

- Simulate a faulty disk drive.

- Compare two disks of either equal or unequal size.

- Compare two disk partitions of either equal or unequal size.

- Compare two disks where the destination disk has had partitions aligned to cylinder boundaries.

In addition to these basic capabilities there are several additional capabilities that are helpful. These include the following:

- Compare two sectors.

- Modify or examine the contents of a sector.

- Relocate a block of sectors.

- Examine a partition table.

## Test Case Structure

Each disk imaging test case is designed to follow a sequence of steps that setup the test, execute the disk imaging tool under test and measure the results. The general structure of a test case is outlined in Figure 1. Usually a source drive is setup (steps 1-3) once and the used in more than one test case.

Figure 1 General Structure of a Test Case

Setup for the test case: - Record details of source disk setup. - Initialize the source disk to a known value. - Hash the source disk and save the hash value. - Record details of test case setup. - Initialize a destination disk. - If the test requires a partition on the destination, create and format a partition on the destination disk. - If the test uses an image file, partition and format a media disk. Execute the tool being tested: - If the test requires a disk I/O error, setup disk error simulation. - If the test requires an image file, use the tool to create an image file of the source on the media disk. - If the test requires a corrupted image file, corrupt the image file. - Use the disk imaging tool to create the destination disk either from a copy of the source disk or by restoring an image file of the source to the destination. Measure the results: - Compare the source to the destination. - Compute a hash of the source disk, compare with the saved hash value.

## Software Overview

To support the capabilities necessary for testing disk imaging tools the programs listed in Figure 2 were developed. The support software can be grouped into four categories: programs to setup something required for the test, programs to measure the results of a test, programs to document some aspect of a test, and utility programs to provide capabilities outside the scope of the usual test case.

Figure 2 List of Support Programs

| Program | Category | Function |
| --- | --- | --- |
| DISKWIPE | Setup | Write a unique pattern to each sector of a disk. |
| BADDISK | Setup | Simulate a bad disk sector for legacy BIOS access. |
| BADX13 | Setup | Simulate a bad disk sector for extended BIOS access. |
| CORRUPT | Setup | Change one byte in a file (corrupt an image file). |
| ADJCMP | Measure | Compare two disks by partitions (allowing for cylinder alignment). |
| DISKCMP | Measure | Compare two disks. |
| PARTCMP | Measure | Compare two partitions. |
| DISKHASH | Measure | Compute a SHA-1 for an entire disk. |
| SECHASH | Measure | Compute a SHA-1 for part of a disk. |
| LOGCASE | Document | Log information about the test run, e.g., disks used. |
| LOGSETUP | Document | Log information about source hard drive setup. |
| PARTAB | Document | Log partition tables. |
| DISKCHG | Utility | Change or examine the content of a single disk sector. |
| SECCMP | Utility | Compare two disk sectors. |
| SECCOPY | Utility | Copy a block of sectors from one location to another. |

# Requirements

This section gives a concise description of what each program is supposed to do. The section is intended to serve as a reference for development of test cases to test the support programs. A reader with other goals could skip this section on first reading. The first subsection describes requirements for common functionality shared by most of the programs. The second subsection lists specific requirements for each program.

## Requirements for Common Functionality

Most of the support programs share common functionality. For example, most support programs work with one or two disks drives and are required to log descriptive information about any disks drives used by the program. Rather than describe these common requirements with each program, they are described here once and then referenced as needed.

### Program Execution Logging

These requirements specify what identifying information a program records in a log file each time the program is executed. A program required to do program execution logging must record:

- The program name, version number, source file creation date & time, and compile date & time.

- The support library name, version number, source file creation date & time, and compile date & time.

- The header file name, version number, and source file creation date & time.

- The command line (including command line options).

- The date and time program execution begins, ends and the elapsed time.

- The test case ID

- The name of the computer where the program is executed.

- A user supplied comment.

In addition, the program must provide command line options to:

- Either start a new log file or append to an existing log file.

- Print a summary of the program command line and command line options, then exit.

### Disk Logging

Each hard disk drive used by a program is described in the program log file.

A program required to do disk loging must record the following information in the specified log file for the specified disk drive:

- The type of BIOS access (extended or legacy) supported for the disk drive.

- The disk geometry as reported by interrupt 13h function 08h, i.e., maximum allowed cylinder value, maximum allowed head value, number of sectors per track and total number of sectors.

- If the interrupt 13h BIOS extensions are supported for the disk drive, then record the disk geometry as reported by the interrupt 13h function 48h, i.e., number of cylinders, number of heads, number of sectors per track and number of sectors.

- If the interrupt 13h BIOS extensions are not supported for the disk drive, then record disk geometry values for number of cylinders, number of heads, number of sectors per track and number of sectors that are consistent with the values reported by interrupt 13h function 08h.

- For IDE disk drives, record the model number and serial number as reported by the ATA identify device command.

- For IDE disk drives that support LBA, record the total number of user addressable sectors as reported by the ATA identify device command.

### Partition Table Logging

A program required to do partition table logging must record the following information for the partition table of the specified disk drive in the specified log file:

- For each partition table entry in either the master boot record partition table and each partition table in any extended partition, print the following: starting LBA address, partition length, starting cylinder/head/sector address, ending cylinder/head/sector address, bootable flag and partition type code (in hexadecimal).

- For common partition types (FAT12, FAT16, FAT32, extended, Linux EXT2, Linux swap, and NTFS) print a descriptive string, e.g., Fat32 for type code 0x0B.

### Comparison Logging

These requirements specify the information logged by a program required to do comparison logging when comparing a source to a destination. A source or destination is defined to be a block of contiguous disk sectors. A source or destination can be an entire disk, a disk partition or a block of sectors located between two partitions. The source (original) is assumed to have been initialized by diskwipe with the source fill byte and the destination is assumed to have been initialized by diskwipe with the destination fill byte.

- Summarize corresponding sectors of the source and destination with counts of the sectors compared, sectors matching (have the same content), sectors differing and the total number of bytes that are different. Note that if large disk drives with few matching bytes are compared then the total number of matching bytes may exceed the maximum integer that can be represented by a single variable. In this case, overflow is permitted without notification.

- If the source and destination are not the same size, log the size of each and the difference in size.

- If the destination is larger than the source, categorize the excess sectors according to the following: zero fill (every byte is zero), diskwipe style fill, and other contents. The diskwipe style fill is actually three categories: source fill byte, destination fill byte and any other fill byte.

- For each category, the first few (up to some arbitrary limit) sectors belonging to the category are logged. A contiguous block of sectors is logged as a hyphen separated pair of integers (start sector - last sector).

### Error Reporting

The following requirements apply to all programs except as noted under each program. In general, the support programs should not encounter any I/O errors during a valid test run. If an I/O error occurs then the support program results are corrupt and cannot be used*.

- If the command line parameters are not valid, print an error message indicating the problem, print a summary of the program command line and command line options, then the program exits.

- If any I/O operation fails, print a diagnostic message and then the program exits.

- If any I/O operation fails, the content of the log file is undefined (the log file should be considered corrupt).

## Individual Program Requirements

This section presents the specific requirements for each support program.

### DISKWIPE

For a designated hard drive, the diskwipe program writes unique contents to each disk sector. The first 25 bytes of each disk sector contains an ASCII string representing both the C/H/S and LBA address of the sector. The remaining bytes of each disk sector are set to a specified hexadecimal fill byte value in the range 0x00 to 0xFF.

- Write the specified content from Table 2 to each disk sector on the specified drive.

- Allow specification of at least three log file names, one for a source disk, one for a destination disk, and one for a media disk.

- Log the specified hard drive.

- Log the program execution.

- By default, use the number of heads obtained from the BIOS extensions, however optionally allow specification of the number of heads to override value from the BIOS.

Table 2 DISKWIPE Pattern Specification

| Sector Contents |  |
| --- | --- |
| Bytes | Contents |
| 0-11 | The C/H/S address of the sector %05d/%03d/%02d |
| 12 | Blank (space character) |
| 13-23 | LBA address of the sector: %011d |
| 24 | Null byte (zero) |
| 25-511 | Specified hexadecimal fill byte in the range 0x00 to 0xFF |

### BADDISK

Baddisk is used to simulate a faulty hard disk accessed by the legacy interrupt 13 BIOS.

- For a specified disk address, specified disk I/O command, and error code value, monitor disk I/O requests. Intercept any attempts to execute the specified disk I/O command on the specified disk address and return the specified error code value.

- Log the command parameters and the current date and time to stdout.

### BADX13

Badx13 is used to simulate a faulty hard disk accessed by the extended interrupt 13 BIOS.

- For a specified disk address, specified disk I/O command, and error code value, monitor disk I/O requests. Intercept any attempts to execute the specified disk I/O command on the specified disk address and return the specified error code value.

- Log the command parameters and the current date and time to stdout.

### CORRUPT

Corrupt is used to change a single byte of an image file.

- Log the program execution.

- Change a specified byte at a specified location in a specified file to a specified value.

- Log the original value at the specified location.

- Log the new value at the specified location.

### ADJCMP

Adjcmp is used to compare two disks where the partitions copied to the destination are adjusted so that the copy is aligned on a cylinder boundary.

- Log the specified source drive.

- Log the specified destination drive.

- Log the program execution.

- Log the partition table for the specified hard drive.

- For each disk, assign each sector to a contiguous block of sectors, called a disk chunk, such that each disk chunk is assigned to one of the following chunk categories: a sector contained within a partition, a sector contained within a partition boot track, the unallocated sectors between two partitions or unallocated sectors after the last partition on the disk.

- Record the location of each disk chunk in the log file.

- Allow specification of corresponding disk chunks between the source hard drive and the destination hard drive. (A disk chunk on the source drive is compared to the corresponding disk chunk on the destination drive.)

- Log the correspondence between source disk chunks and destination disk chunks, i.e. for each disk chunk on the source drive, log the disk chunk on the destination that the source disk is to be compared to.

- Log the comparison between each pair of corresponding disk chunks.

- For any destination disk chunks that have no corresponding source chunk categorize the sectors of the disk chunk according to the following: zero fill (every byte is zero), diskwipe style fill, and other contents. The diskwipe style fill is actually three categories: source fill byte, destination fill byte and any other fill byte. For each category, the first few (up to some arbitrary limit) sectors belonging to the category are logged. A contiguous block of sectors is logged as a hyphen separated pair of integers (start sector - last sector).

- Log a summary as follows:

- Number of boot tracks, total number of sectors assigned to boot tracks, and number of boot track sectors that do not compare equal.

- Number of partitions, total number of sectors assigned to some partition, and number of corresponding partition sectors that do not compare equal.

- Number of unallocated chunks with a corresponding unallocated chunk, number of sectors in this category and number of corresponding sectors that do not compare equal.

- Number of excess sectors in destination chunks that have a corresponding source chunk, number of sectors that have every byte set to zero, and number of remaining sectors.

- Number of sectors in destination chunks that do not have a corresponding source chunk, number of sectors that have every byte set to zero, and number of remaining sectors.

- Total number of source sectors and total number of destination sectors.

### DISKCMP

Diskcmp is used to evaluate the accuracy of a disk duplication operation.

- Log the specified source drive.

- Log the specified destination drive.

- Log the program execution.

- Log the comparison between the source drive and the destination drive.

- If there is a read error the comparison results are undefined.

- If there are any read errors, then continue scanning the disk and log a count of the number of tracks with read errors on each disk.

### PARTCMP

Partcmp is used to evaluate the accuracy of a partition duplication operation.

- Log the specified source drive.

- Log the specified destination drive.

- Log the program execution.

- Log the comparison between the source partition and the destination partition.

### DISKHASH

For a designated hard drive, the diskhash program computes a SHA-1 hash on a specified hard disk drive.

- Compute a SHA-1 hash for the designated hard drive.

- Allow specification of at least two log file names, one for reference before a tool is run and one for comparison after a tool is run.

- Log the specified hard drive.

- Log the program execution.

### SECHASH

For a designated hard drive, the sechash program computes a SHA-1 on a block of contiguous sectors from specified hard disk drive.

- Compute a SHA-1 for a specified block of continuous sectors from the designated hard drive.

- Log the computed hash value.

- Allow specification of at least two log file names, one for reference before a tool is run and one for comparison after a tool is run.

- Log the specified hard drive.

- Log the program execution.

### LOGCASE

Logcase is used to create a log file recording basic information about the test case.

- Record the following: Test case ID, host computer, operator, source disk drive, destination disk drive, other disk drive, date and time.

### LOGSETUP

Logsetup is used to log information about the setup of a source disk,

- Record the following: disk label, host computer, operator, operating systems loaded, date and time.

### PARTAB

Partab is used to print the partition table for a hard drive.

- Log the specified hard drive.

- Log the program execution.

- Log the partition table for the specified hard drive.

- Use a different log file name for each hard drive.

- Log (optionally by command line control) a unique identification for each partition that can be used by the partcmp program to select partitions for comparison.

- Log (optionally by command line control) empty partition table entries.

### DISKCHG

Diskchg is used to setup hard disk drives for the testing of the other support programs. Diskchg can be used to set a single byte of a specified sector to a given value or to set an entire sector either to all zero bytes or to diskwipe style fill. Diskchg can also be used to examine the contents of a given sector.

- Log the specified hard drive.

- Log the program execution.

- Allow specification of disk sector addresses in either CHS or LBA format.

- Set every byte of a specified sector to zero.

- For a specified sector, s, a specified address, a, (possibly not the same as the specified sector), a specified disk geometry and a specified fill value, fill sector s with the contents of a diskwipe style fill using a as the address value for the fill. In other words, set sector s to the contents that diskwipe would use for the sector at location a on a disk with the specified geometry using the specified fill value.

- For a specified sector, a specified offset within the sector and a specified value, set the byte at the offset within the sector to the specified value.

- For a specified hard drive, a specified sector, a specified offset within the sector and a specified count, log the contents count bytes from the specified sector starting at the specified offset.

- Allow interactive examination of sector contents.

- Use a different log file name for each function.

### SECCMP

Seccmp is used to compare two disk sectors.

- Log the specified source drive.

- Log the specified destination drive.

- Log the program execution.

- If the sectors to compare are not diskwipe style filled or zero filled, log any differences between the source sector and the destination sector.

- Diskwipe style filled sectors or zero filled sectors are logged with no need for comparison.

- Allow specification of an alternate log file name.

### SECCOPY

Seccopy is used to copy blocks of sectors from one hard disk drive to another. Seccopy is used to setup hard disk drives for the testing of the other support programs.

- Log the specified source drive.

- Log the specified destination drive.

- Log the program execution.

- For a specified length, source disk starting address, and a destination disk starting address, copy length sectors from the source to the destination.

# Design Notes

This section describes the design of the support software. The logical structure of most of the support software is usually a simple loop. The program determines the number of sectors in the selected object (disk or partition) and then performs some operation on each sector from the first to the last. The ADJCMP program is the most complex of all the support programs, the disk is divided into regions (partitions and unallocated space) that are each processed in turn via a nested loop (for each region do the operation for each sector of each region). A few programs only operate on a single sector or follow the linked list of a partition table.

The physical structure of the support programs is also simple. Except for DISKHASH and SECHASH, each program is compiled and linked from two source files and a header file. Each program has a main source file that contains the main procedure (e.g., DISKWIPE.CPP) and the essential functionality of the program. The second file, the library (ZBIOS.CPP), contains functions that are commonly used by the support programs, such as functions to read or write disk sectors by absolute disk address. In general, the requirements listed in section 2.1 Requirements for Common Functionality, such as logging, are implemented in the library. The two hashing programs also link with routines that do the actual SHA-1 hash calculations.

The remainder of this section is organized as follows: the next subsection discusses data structures defined in ZBIOS.H, subsection 3.2 discusses the functions defined in ZBIOS.CPP, and the individual programs are discussed in subsection 3.3.

## Data Structures

The data structures defined in ZBIOS.H are used to keep track of information about disk drives. Each data structure is defined in a typedef as a block of storage and as a pointer to a block of storage. There are three major data structures defined. Data structures to describe a disk drive, data structures for handling partition tables and data structures for tracking lists of sectors with a common property.

### Describing a disk drive

The disk_control_block is the data structure for describing a disk drive. Several other structures defined in ZBIOS.H are components of the disk_control_block. These data structures are: chs_addr, ide_rec, ide_ptr, physical_sector, and physical_track.

Figure 3  Data Structures for Describing a Hard Drive

typedef struct {unsigned long cylinder,head,sector; } chs_addr; typedef unsigned char physical_sector[512]; /* a sector of 512 bytes */ typedef physical_sector physical_track[63]; /* a track is an array of 63 sectors */ typedef struct { unsigned long max_user_sectors; char serial_no[21]; char model_no[41]; } ide_rec, *ide_ptr; struct disk_struct { unsigned long n_sectors; chs_addr disk_max; /* number of cyl, number of head */ chs_addr logical_disk; /* max cyl, max head */ chs_addr this_track; /* cyl and head of track in buffer */ int drive; /* drive number (starts at 0x80) */ int use_bios_x; /* is disk > 8.4GB */ int valid_sectors; /* track buffer has data */ ide_ptr ide_info; /* Model#, Serial# & max sectors */ physical_track buffer; /* 63 sectors (1 track) buffer[sector][byte]*/ }; typedef struct disk_struct disk_control_block, *disk_control_ptr;

Any I/O to or from the disk takes place one track at a time. The data buffer is built up as an array of 63 sectors of 512 bytes per sector. If the BIOS extensions are supported, the n_sectors field is set to the value returned by the extended command (interrupt 13h function 48h), otherwise the n_sectors field is set to a value computed from values returned by interrupt 13h function 08h. The field logical_disk contains the disk geometry returned by interrupt 13h function 08h. The field disk_max contains the disk geometry returned by interrupt 13h function 48h. If the BIOS extensions are not supported, then the values from logical_disk are copied to disk_max and one is added to the cylinder and head fields of disk_max (to simulate the values that would be returned by function 48h). The disk address of the buffer field contents is this_track. The BIOS assigned drive number is in the drive field. The field use_bios_x field is true if the BIOS extensions are supported. The valid_sectors field is a flag indicating the presence of data (if true) in the buffer field. If the disk drive is attached to either the primary or secondary IDE interface, then the ide_info field contains the results of an ATA identify device command for the drive. The value of the max_user_sectors field overrides the n_sectors field for any support programs that operate on an entire disk.

### Describing a partition table

Partition tables are stored on a hard drive as a linked list with a root node in the master boot record of the drive. The layout of the master boot record is in the mbr_sector structure. The first 446 bytes are not part of the partition table. The four partition table entries are found in the pe field. The partition table is followed by a signature byte in the sig field. A partition table entry is described by the partition_table_rec data structure. It should be noted that the two high order bits of the 10 bit cylinder number are located in the two high order bits of the sector number field. After the partition table is read from the disk, the each partition table entry is placed in a linked list of pte_rec structures.

Figure 4 Data Structures for Describing a Partition Table

/****************************************************************************** Partition table entry layout on disk ******************************************************************************/ typedef struct pts { /* partition table layout */ unsigned char bootid, start_head, start_sector, start_cylinder, type_code, end_head, end_sector, end_cylinder; unsigned long starting_lba_sector, n_sectors; } partition_table_rec, *partition_table_pointer; /****************************************************************************** Layout of a partition table in a boot sector ******************************************************************************/ typedef struct { /* partition table in Master Boot Record */ char fill[446]; /* MBR boot code */ partition_table_rec pe[4]; /* master partition table */ unsigned int sig; /* partition table signature word 0xAA55 */ }mbr_sector,*mbr_ptr; /****************************************************************************** Data structure to keep partition table information ******************************************************************************/ typedef struct pte_struct pte_rec, *pte_ptr; struct pte_struct { pte_ptr next; unsigned char is_boot,type; chs_addr start,end; unsigned long lba_start,lba_length; };

### Describing a range of sectors

The compare programs examine each disk sector in LBA address sequence and assign each sector to a catagory, e.g., dst sector that differs from corresponding src sector, zero filled, src filled, dst filled, etc. This data structure is used to track blocks of disk sector LBA addresses for sectors that are classified in the same catagory. A range is recorded as a pair, from and to. The n field counts the number of pairs in the r field. The is_more field is true if there are more ranges than r can contain.

Figure 5 Data Structures for Describing a Range of Sectors

/****************************************************************************** Data structure to track ranges of integers. The compare programs examine each disk sector in LBA address sequence and assign each sector to a catagory, e.g., dst sector that differs from corresponding src sector, zero filled, src filled, dst filled, etc. This data structure is used to track blocks of disk sector LBA addresses for sectors that are classified in the same catagory. ******************************************************************************/ typedef struct {unsigned long from,to;} lba_range; typedef struct { /* structure to keep a list of ranges */ int n; long is_more; /* more than N_RANGE ranges present (i.e., some not recorded) */ lba_range r[N_RANGE]; } range_list,*range_ptr;

## Library Functions Designs

The library functions can be grouped into six categories: disk drive information, disk drive I/O, logging, partition tables, range lists and debugging. The functions are usually short with the longest about 60 C code statements. Several of the functions use brief sections of inline assembly code to access the BIOS extensions.

### Disk Drive Information Functions

The functions that obtain information about disk drives use three separate methods to gather information. The first approach is to use the interrupt 13h function 08h (legacy) interface to the BIOS. The disk geometry returned from this interface is stored in the logical_disk field of the disk_control_block. The values for the cylinder and head are the maximum allowed values. Since cylinders and heads begin at zero, these values are one less that the number of cylinders and heads. The second approach is to use the interrupt 13h function 48h interface to the BIOS. This is part of the BIOS extensions and may not be supported. If the BIOS extensions are supported, then the values returned for disk geometry are stored in the disk_max field of the disk_control_block. The third approach, only appropriate for IDE disk drives, uses I/O ports to send the ATA identify device command directly to the disk drive. Three fields are extracted from the results to obtain the model number, serial number and maximum number of user addressable sectors.

The following functions obtain information about disk drives:

- c13 This function uses the interrupt 13h function 41h to test if the BIOS extensions are supported.

- Decimal_Number This function is used to assemble two unsigned int values into an unsigned long value.

- Get_Hard_Drive_Parameters This function uses interrupt 13h function 48h interface to obtain the disk geometry and number of sectors on a hard drive.

- probe_ide_adapter This function uses the port functions (outportb, inp and inport) to probe a given port for an IDE disk drive.

- probe_ide This function returns information about an IDE disk drive. The first time this function is called it uses probe_ide_adapter to test for disks on the primary and secondary IDE controllers.

- trim This function removes excess blanks from the model number and serial number obtained by probe_ide_adapter.

- open_disk This function calls the other disk information functions to create a disk_control_block for a specified drive.

### Disk Drive I/O Functions

These functions handle the actual I/O to the disk. All operations are performed on an entire track of 63 sectors at a time.

- lba_to_crs This is a utility function to convert an LBA address to CHS notation.

- disk_write This function writes an entire track to a specified disk.

- disk_read This function reads an entire track from the specified disk.

- read_lba This function returns a pointer to the disk sector at the specified address. If the requested sector is not in the disk_control_block buffer, then disk_read is called to read the appropriate track into the buffer.

### Logging Functions

These functions write selected information to a log file.

- log_open This function creates a log file and writes descriptive information about the program to the log.

- log_close This function calculates the elapsed program run time and writes the value to the log file.

- log_disk This function logs information about the selected disk drive to the log file.

- feedback This function provides progress feedback to the program operator for a program iterating through a loop. An estimate of the program run time remaining is printed to the console based on how much time has elapsed, the number of iterations executed so far and the total number of iteration to execute.

### Partition Tables Functions

These functions fetch the partition table from a specified disk drive to create a linked list of partition table entries that can be examined or printed.

- get_sub_part This function is called to follow the linked list of partition table entries in an extended partition.

- get_partition_table This function reads the master boot record partition table and builds a linked list of partition table entries. If there are any extended partitions then get_sub_part is called.

- print_partition_table This function is called to print the partition table linked list created by get_partition_table. The function partition_type is called to map partition type codes to ASCII strings.

- partition_type This function maps common partition type codes to an ASCII string.

### Range Lists Functions

The range list functions manage lists of ranges. Several of the programs categorize disk sectors according to content criteria and report on the number of sectors in each category. It is also useful to get a general idea of where and how each category is distributed on a disk. By tracking the first few contiguous groups of sectors in each category a general indication of the distribution can be obtained.

The lba_range data structure stores a range as two values, from and to, indicating the lower and upper end points of a range.

- create_range_list This function returns an empty range list.

- add_to_range This function adds a new value to a range. The list of ranges is searched and if a range is found such the new value is one step outside an existing range then that range is expanded by one. Otherwise a new range is created with the new value as the upper and lower end points. The add_to_range function assumes that the sectors are scanned in sequence so that a range can be built up as the scan progresses.

- print_range_list This function prints a range list.

### Debugging Functions

There are two debugging functions that print intermediate data structures: print_dcb and dap_print.

## Individual Program Designs

This section gives a general outline of each program. Most of the programs are small ( less than 200 statements) and follow the same general plan (Figure 6). The first_sector and last_sector might encompass the entire disk, a single partition or only one sector depending on the actual program.

Figure 6 General Program Design for Support Software

main() { decode command line parameters if (error) print message & quit open log file open disks log disks for (i = first_sector; i < last_sectors; i++) { do something if (error) print message & quit } log results close log file }

The programs DISKHASH, DISKWIPE, DISKCMP, DISKCHG, PARTCMP, PARTAB, ADJCMP, SECCMP, SECCOPY, SECHASH, and CORRUPT all follow the general support software design. The remaining subsections of this section discuss any deviation from the general program design and the do something aspect of the general program design in Figure 6.

### DISKWIPE

The diskwipe program sets up the known unique content for each sector of a disk track and then when the contents for the last sector of a track is setup, writes the content for the given track to the disk.

### BADDISK

This program is a terminate and stay resident (TSR) written in assembly language to simulate a disk with a bad sector by replacing the original interrupt 13h service routine. A command, an address in cylinder/head/sector notation and a error code are specified. If a program issues the command for the specified address then the error code is returned otherwise the command is passed to the original interrupt 13h service routine. BADDISK operates on a legacy BIOS.

Figure 7 TSR Program Design

Extract and save command line parameters If parameters not valid, print a message and exit else activate resident part and exit Resident portion Entry via interrupt 13 If target command and address then return error code pass command to original service routine

### BADX13

This program is a version of BADDISK for systems with an extended BIOS. The major difference is the format of disk sector addresses. Sector addresses are specified in LBA format.

### CORRUPT

The corrupt program only operates on a single byte of the specified file. The byte, indicated by an offset from the beginning of the file, is read, logged and a new value is written back to the file.

### ADJCMP

The adjcmp program adds an outer loop that indexes each disk chunk. The inner loop runs from the first sector to the last sector of each disk chunk.

### DISKCMP

The diskcmp program reads corresponding sectors from the source and destination, checks for an exact match and tallies the result. If the destination is smaller than the source, no more sectors are examined, the results are logged and the program exits. If the destination is larger than the source, the excess destination sectors are classified, the results are logged and the program exits.

### PARTCMP

The partcmp program reads corresponding sectors from the source and destination, checks for an exact match and tallies the result. If the destination is smaller than the source, no more sectors are examined, the results are logged and the program exits. If the destination is larger than the source, the excess destination sectors are classified, the results are logged and the program exits.

### DISKHASH

The diskhash program reads each disk sector and calls a SHA-1 function to add each sector of data to the hash. After all the sectors have been added to the hash, the final SHA-1 hash value is extracted and logged.

### SECHASH

The sechash program reads each disk sector from a range of sectors specified on the command line and calls a SHA-1 function to add each sector of data to the hash. After all the sectors have been added to the hash, the final SHA-1 hash value is extracted and logged.

### LOGCASE

LOGCASE writes the command line parameters to a file to document disks used in a test case.

### LOGSETUP

LOGCASE writes the command line parameters to a file to document setup of a source disk.

### PARTAB

Print the partition table from the boot sector. For each extended partition, follow the links to each extended table entry and print the entries.

### DISKCHG

Decode the command line options and do the requested operation.

### SECCMP

Read the two specified sectors, compare the data, and log the results.

### SECCOPY

For each source sector in the specified range of addresses, read the sector and write the contents back at the corresponding destination location.

# User Manual

This section describes in detail how to use each program. The organization of this section is as follows. Section 4.1 describes the expected environment, Section 4.2 is a general discussion of the program log files, Section 4.3 describes the format of error messages. Section 4.6 gives an overview of features common to the comparison programs. Each of the remaining sections describes one of the support programs. For each program there is a general description of the program followed by subsections devoted to command line parameters, error messages, and other topics as needed.

## System Environment

The expected operating environment is an Intel X86 (or Pentium) architecture PC running DOS 6.3 with a floppy disk and at least two hard disk drives. The hard drives may use either an IDE or a SCSI interface, however all disks must have 63 sectors per track.. The system BIOS may or may not support the ATA interrupt 13 extensions for large (more than 8GB) disk access. If the system has both IDE and SCSI disks, the BIOS must be configured to access the IDE disks first, i.e., the IDE disks must be assigned drive numbers before the SCSI disks. Otherwise, the support software will erroneously try to use the size of the first IDE disk (primary master) as the size of the first SCSI disk.

## Log files

Each program keeps a log file of results. The log files are named so that all the log files for a given test case can exist in the same file system directory. This is the rationale for some programs having options to set the log file name. For example, a test may use one, two or three disks. Each disk must be uniquely initialized. If each log file had the same name there would be a conflict. All support programs (except baddisk and badx13) log the following information:

- Program version, date compiled, time compiled.

- Support library version, date compiled, time compiled.

- Test case ID (from command line)

- Host computer (from command line)

- A user supplied comment

- List of command line options specified

- Run start date and time

- Run finish date and time

- Elapsed time

The following information is logged for each disk used by the program:

- Drive number

- Legacy BIOS geometry: C/H/S

- XBIOS geometry: C/H/S

- Total sectors reported by the BIOS

- For IDE drives: Model and serial numbers and number of sectors accessible in LBA mode

Each program also logs any information specific to the program.

Table 3 Program Log Files

| Program Log Files |  |  |
| --- | --- | --- |
| Program | Logfile name | Comments |
| diskwipe | WIPEDLOG.TXT | Default log name |
| diskwipe | WIPESLOG.TXT | /src option selected. Used to wipe a src disk |
| diskwipe | WIPEMLOG.TXT | /media option selected. Used to wipe a media disk |
| baddisk | BD-XX-NN.TXT | Program does not create the log file. To create the log file use command line output redirection. Naming convention: replace XX with RD for read commands, RL for read long and WT for write commands. Replace NN with the test case number. |
| badx13 | BD-XX-NN.TXT | Program does not create the log file. To create the log file use command line output redirection. Naming convention: replace XX with RD for read commands, RL for read long and WT for write commands. Replace NN with the test case number. |
| corrupt | CORLOG.TXT | Old and new values of the selected byte |
| adjcmp | CMPALOG.TXT | Compare two disk drives, allowing for cylinder alignment |
| diskcmp | CMPLOG.TXT | Compare two disk drives (physical drives) |
| partcmp | CMPPTLOG.TXT | Compare two partitions (logical drives) |
| diskhash | HASHBLOG.TXT | /before option selected – hash before running tool |
| diskhash | HASHALOG.TXT | /after option selected – hash after running tool |
| sechash | HASHBSEC.TXT | /before option selected. |
| sechash | HASHASEC.TXT | /after option selected. |
| logcase | CASE.TXT | Information about the test case |
| logsetup | SETUP.TXT | Information about a disk setup |
| partab | PTXXLOG.TXT | XX is replaced by the drive number. |
| diskchg | CGXXFLOG.TXT | XX is replaced by the drive number. F is determined by the options selected. Option; Logfile name. /read; CGXXRLOG.TXT. /write; CGXXWLOG.TXT. /fill; CGXXFLOG.TXT. /zero; CGXXZLOG.TXT. /exam; CGXXXLOG.TXT. |
| seccmp | SECLOG.TXT | Default log file name. /log name overrides. |
| seccopy | COPYLOG.TXT | Information about the sectors copied |

## Error Messages

The major error messages for each program are noted. Location of information specific to the error instance is indicated by the C/printf code used to format the information. Examples of such specific information include disk drive number (%X), status codes (%d) and disk logical block address (%ld). The code %X indicates a hexadecimal value, %d indicates an integer, %ld indicates a long (very large) integer and %s indicates a text string.

For the comparison programs, the same message may be possible for both the source and

the destination. This is indicated by [src/dst] rather than repeating the explanation of the error message twice.

## Disk initialization: DISKWIPE

DISKWIPE initializes a disk to a known value. DISKWIPE uses a two digit hexadecimal value as a fill byte on each sector of the disk. DISKWIPE writes the sector address in both CHS and LBA style to the first 25 bytes of each sector (see Table 2 DISKWIPE Pattern Specification).

DISKWIPE wipes all the user addressable sectors reported. DISKWIPE may wipe sectors beyond the last sector address reported by the BIOS. DISKWIPE logs the number of sectors wiped and the hexadecimal fill value.

A disk used in testing disk imaging tools may be used in one of three roles: setup as a known source for copy or image operations, setup with a file system to contain an image file, or as setup as a destination for a copy or image restore operation. The role of the disk in the test case determines the selection of /src, /media or /dst options.

A suggested method for assigning hexadecimal fill values is to give each disk drive a unique label of two hexadecimal digits. The disk label can then be used as the fill value for any operations on the disk.

### DISKWIPE command line

diskwipe test-case host drive fill [/opts]

Table 4 DISKWIPE Command Line Parameters

| Parameter | Description |
| --- | --- |
| test-case | A test case identifier |
| host | The name of the computer running the test |
| drive | The drive number in hex. Possible values begin at 80 |
| fill | A two digit hex value, used to fill each sector. |
| /comment “…” | A comment for the test log file. If this option is not used, the program prompts for a log floppy to be inserted. If this option is used, then a log floppy is assumed to be already loaded. |
| /src | Set log file name to: wipeslog.txt (defaults to wipedlog.txt). Use this option if the disk is used as a source. |
| /media | Set log file name to: wipemlog.txt (defaults to wipedlog.txt). Use this option if the disk is used to contain an image file. |
| /dst | Use this option for disks used as a destination. Set log file name to: wipedlog.txt |
| /heads nnn | Override the number of heads reported by the BIOS. This only changes the pattern of addresses written to each sector. |
| /noask | Skip the confirmation dialog asking if you are really sure you want to wipe out a disk. This option is intended for running DISKWIPE from a script (unattended). |
| /new_log | Start a new log file rather than append to an existing log file. |
| /? | Print a summary of command options and exit |

### DISKWIPE error messages

- /comment option requires a comment

This option must be followed by the text of the comment on the command line. If the comment includes any space characters then the entire comment must begin and end with a double quote character (“). For example,

a:\diskwipe DI-01 lab 80 B4 /comment “Wipeout disk labeled B4”

- /heads option requires a value

This option must be followed on the command line with a decimal integer (the number of heads per track).

- could not access drive %x status code %d

The specified disk drive could not be accessed. The status code indicates the type of problem. 3000 indicates the program ran out of memory. 2000 indicates that the disk is too large. Other values indicate a problem with the drive. Most likely cause is the drive number does not refer to an attached drive or the indicated drive may not be properly installed.

- At sector (%s) write error\n error code %d

The disk has not been completely wiped. There is likely a hardware problem that must be resolved.

## Faulty Disk Simulation

BADDISK is an assembler terminate and stay resident (TSR) that monitors the interrupt 13 disk interface for a given disk address and command. BADDISK usually passes an I/O request on to the BIOS interface, however if the I/O request is for the given address and command a specified error result code is returned instead. BADX13 is a version for use on large disks that require the interrupt 13 extensions. These programs echo the command line to stdout and do not write a log file. The usual execution protocol is to redirect stdout to a log file.

### BADDISK Command Line

baddisk drive cyl head sec command error_code > log_file

Table 5 BADDISK Command Line Parameters

| Parameter | Description |
| --- | --- |
| drive | Indicates the hard drive, drive numbers start at 80 |
| cyl head sec | The disk address of a target sector |
| command | The command code of a target command (2 is read, 3 is write, 10 is long read). Note that to completely block disk reads, the TSR must be run twice, once for the read command and once with the read long command. |
| error_code | The error code to return if command is requested for sector cyl/head/sec on drive |
| > log_file | Create a log file by I/O redirection. |

### BADX13 Command Line

badx13 drive command error_code sector_LBA > log_file

Table 6 BADX13 Command Line Parameters

| Parameter | Description |
| --- | --- |
| drive | Indicates the hard drive, 80=>master, 81=>slave |
| command | The command code of a target command (42 is read, 43 is write) |
| error_code | The error code to return if command is requested for sector sector_LBA on drive |
| sector_LBA | The disk address of a target sector |
| > log_file | Create a log file by I/O redirection. |

## Corrupt an image file: CORRUPT

CORRUPT is used to corrupt an image file by changing a single selected byte in an image file. CORRUPT logs the original content of the selected byte, the replacement value, the index of the byte within the file and the file name.

### CORRUPT command line

corrupt test-case host image-file index value [/opts]

Table 7 CORRUPT Command Line Parameters

| Parameter | Description |
| --- | --- |
| test-case | A test case identifier |
| host | The name of the computer running the test |
| image-file | Full path name of the image file to be corrupted. |
| index | Relative index position of the byte to be changed. |
| value | A two digit hex value, used to replace the selected byte. |
| /comment “…” | A comment for the test log file. If this option is not used, the program prompts for a log floppy to be inserted. If this option is used, then a log floppy is assumed to be already loaded. |
| /new_log | Start a new log file rather than append to an existing log file. |
| /? | Print a summary of command options and exit |

### CORRUPT error messages

- /comment option requires a comment

This option must be followed by the text of the comment on the command line. If the comment includes any space characters then the entire comment must begin and end with a double quote character (“). For example,

a:\corrupt DI-01 lab image 123456 AE /comment “Corrupt byte 123456”

- Open %s failed

CORRUPT could not open the specified file. The file name or path may be incorrect. The open would also fail if the file is marked read only.

- Offset value (%s) is not valid

CORRUPT could not convert the offset to an integer. A likely cause would be if CORRUPT tried to convert a string that contained characters other than digits.

- Replacement value (%s) is not valid

CORRUPT could not convert the replacement value to an integer. A likely cause would be if CORRUPT tried to convert a string that contained characters other than digits.

- Offset %ld is not valid for %s

CORRUPT could not seek to the specified offset location in the file. A likely cause would be if an offset value larger than the file size were specified.

- Read failed\n

CORRUPT failed to read the byte designated for replacement.

- Seek to %ld on %s after read failed

Should not happen unless the disk fails.

- Write failed

Should not happen unless the disk fails.

- Seek to %ld on %s after write failed

Should not happen unless the disk fails.

- Verify read failed

Should not happen unless the disk fails.

- verify failed: char read back (0x%02X) should be (0x%02x)

Should not happen unless the disk fails.

## Comparison Software: ADJCMP, DISKCMP and PARTCMP

The comparison software is a group of five programs. The program selected depends on the requirements of the test case.

Table 8 Comparison Programs

| Program | Test case requirements |
| --- | --- |
| DISKCMP | Compare two entire disks sectors from the same address on each disk, sector by sector. |
| PARTCMP | Compare two partitions. |
| ADJCMP | Compare two entire disks when there has been cylinder alignment, i.e., the destination drive does not have the same number of heads as the source and partitions on the destination drive may have been shifted to cylinder boundaries. |
| SECCMP | Compare two sectors. This program is used to investigate, in detail for selected sectors, the results obtained from the other three compare programs. This utility is described in Section 4.14. |

All the comparison software assumes the following preconditions:

- DISKWIPE has been run with different fill values on each disk.

- One disk, designated the source, has been copied by a disk imaging tool to the other disk, designated the destination. The destination may actually be created by either a copy or an image restore operation.

The programs DISKCMP, PARTCMP and ADJCMP compare the source to the destination and log the following:

- Relative and absolute size of source and destination

- Number of sectors compared

- Number of sectors that match (identical contents)

- Number of sectors that differ

- Total number of bytes that differ

If the destination is larger than the source then the following is also reported for destination sectors that do not correspond to any source sector:

- Number of sectors zero filled

- Number of sectors filled with the source disk fill byte

- Number of sectors filled with the destination disk fill byte

- Number of sectors filled with some other fill byte

- Number of sectors with some other content

The addresses of the first few sectors in each category are also listed for possible investigation by the SECCMP program. Contiguous sectors are reported as a range of sector addresses.

### Compare two disks by partitions: ADJCMP

Some disk imaging tools optionally create a destination with partitions aligned to disk cylinder boundaries. All the content of the source disk is reproduced on the destination, but not at the same location as on the source disk. The ADJCMP program is used to compare corresponding sectors of two disks where the corresponding sectors are located at different addresses on the source disk and the destination disk.

ADJCMP divides the source and destination disk into disk chunks. Where a disk chunk is defined as a group of contiguous sectors (as listed below). Corresponding disk chunks between the source and destination are identified, and sectors at the same location relative to the first sector of the corresponding chunks are compared. A disk chunk is one of the following:

- Partition defined in the partition table

- Partition boot track (the track just before the start of the partition)

- Unallocated sectors (a group of contiguous sectors not part of any partition)

ADJCMP automatically designates disk chunks found in the same order on the source and destination as corresponding. The user may optionally reassign the correspondence between pairs of disk chunks by an interactive dialog.

A comparison report is logged for each disk chunk and a summary report for the entire disk. An example summary report is as follows:

Figure 8 ADJCMP Summary Report Example

Summary Boot tracks 4 252 diffs 4 Partitions 6 2241540 diffs 9 Unallocated 4 931392 diffs 1 Total src sectors 3173184 Partition excess 76671 zero 74189 non-zero 2482 Disk excess 9422595 zero 9408998 non-zero 13597 Total dst sectors 12672450

An example summary report is presented in Figure 8. The summary report groups the results into four categories: boot tracks, partitions, partition excess areas (added to the destination partition so that the destination partition fills out the last cylinder of the partition) and disk excess sectors (sectors not part of a partition remaining on the destination after all the source has been copied to the destination. For boot tracks and partitions, the critical value is the number of sectors that do not match (diffs). For the partition fill area and excess sectors, the values reported indicate number of sectors filled with null bytes (zero) or with come other content.

In the example, there are 4 boot tracks with 252 sectors. There are 4 sectors of the 252 boot track sectors that differ. There are a total of 2241540 sectors allocated to 6 partitions with 9 sectors that do not match. There are 931392 sectors not allocated to a partition (or boot track). There is one unallocated sector on the source drive that does not match the corresponding sector on the destination. This accounts for the 3173184 sectors of the source drive and the corresponding sectors on the destination drive.

The excess sectors, i.e., destination sectors that do not correspond to any source sector, are analyzed in the latter part of the summary report. In this example, 76671 sectors are used to pad out partitions for cylinder alignment. Of these 76671 sectors, 74189 are filled with zeros and 2482 have other content. There are 9422595 sectors on the destination beyond the last partition. Of these sectors, 9408998 are zero filled and 13597 have other content.

#### ADJCMP command line

adjcmp case host src-drv src-fill dst-drv dst-fill [/opts]

Table 9ADJCMP Command Line Parameters

| Parameter | Description |
| --- | --- |
| case | A test case identifier |
| host | The name of the computer running the test |
| src-drv | The drive number in hex. Possible values begin at 80. The drive where the source drive is mounted. |
| src-fill | A two digit hex value, used to fill each sector by diskwipe. |
| dst-drv | The drive number in hex. Possible values begin at 80. The drive where the destination drive is mounted. |
| dst-fill | A two digit hex value, used to fill each sector by diskwipe. |
| /comment “…” | A comment for the test log file. If this option is not used, the program prompts for a log floppy to be inserted. If this option is used, then a log floppy is assumed to be already loaded. |
| /assign | Engage the user in a dialog to assign corresponding regions of the source and destination disks for comparison. |
| /new_log | Start a new log file rather than append to an existing log file. |
| /? | Print a summary of command options and exit |

#### ADJCMP error messages

- missing parameters

One or more of the six required command line parameters is missing.

- /comment option requires a comment

This option must be followed by the text of the comment on the command line. If the comment includes any space characters then the entire comment must begin and end with a double quote character (“). For example,

a:\adjcmp DI-01 lab 80 B4 81 CC /comment “Compare B4 to CC”

- could not access [src/dst] drive %x status code %d

The specified disk drive could not be accessed. The status code indicates the type of problem. 3000 indicates the program ran out of memory. 2000 indicates that the disk is too large. Other values indicate a problem with the drive. Most likely cause is the drive number does not refer to an attached drive or the indicated drive may not be properly installed.

- [src/dst] drive %s is not valid (0x80 <= drive <0x89)

The drive number is not valid. Most likely valid drive numbers are 80 and 81, sometimes 82 or 83. Other values are not likely to be valid in the expected operating environment of the software.

- [src/dst] read error 0x%02X on track starting at lba %ld

The disk may have bad sectors. There is likely a hardware problem that must be resolved. It is also possible that either BADDISK or BADX13 was executed and has not been cleared from the system by turning the computer off and rebooting.

- /select requires two parameters: src partition index\n and dst partition index

This option must be followed by two integer values.

- could not find [src/dst] partition index %d

The partition index specified does not exist on the selected disk. Try running without the /select option or use the PARTAB program to get the valid index values.

### Compare two disks ignoring disk layout: DISKCMP

The DISKCMP program compares sectors located at the same address from the source and destination disk. Disk partition tables or other content is irrelevant to the program; everything is compared. DISKCMP also tracks disk read errors, reports the location of the first few (about 10) and prints a count of the number of read errors from the source disk and from the destination disk. If there are any read errors, then the comparison log is not valid.

#### DISKCMP command line

diskcmp case host src-drv src-fill dst-drv dst-fill [/opts]

Table 10 DISKCMP Command Line Parameters

| Parameter | Description |
| --- | --- |
| case | A test case identifier |
| host | The name of the computer running the test |
| src-drv | The drive number in hex. Possible values begin at 80. The drive where the source drive is mounted. |
| src-fill | A two digit hex value, used to fill each sector by diskwipe. |
| dst-drv | The drive number in hex. Possible values begin at 80. The drive where the destination drive is mounted. |
| dst-fill | A two digit hex value, used to fill each sector by diskwipe. |
| /comment “…” | A comment for the test log file. If this option is not used, the program prompts for a log floppy to be inserted. If this option is used, then a log floppy is assumed to be already loaded. |
| /new_log | Start a new log file rather than append to an existing log file. |
| /? | Print a summary of command options and exit |

#### DISKCMP error messages

- /comment option requires a comment

This option must be followed by the text of the comment on the command line. If the comment includes any space characters then the entire comment must begin and end with a double quote character (“). For example,

a:\diskcmp DI-01 lab 80 B4 81 B9 /comment “Compare B4 with B9”

- could not access [src/dst] drive %x status code %d

The specified disk drive could not be accessed. The status code indicates the type of problem. 3000 indicates the program ran out of memory. 2000 indicates that the disk is too large. Other values indicate a problem with the drive. Most likely cause is the drive number does not refer to an attached drive or the indicated drive may not be properly installed.

- [src/dst] read error 0x%02X on track starting at lba %ld

The disk may have bad sectors. There is likely a hardware problem that must be resolved. It is also possible that either baddisk or badx13 was run and has not been cleared from the system by turning the computer off and rebooting.

### Compare two disk partitions: PARTCMP

The PARTCMP program compares sectors located at the same address relative to the beginning of the respective partitions. The source and destination must both have valid partition tables. The partitions to compare are specified by an index into the partition table. The possible index values can be determined in two ways. Either run the program in interactive mode (do not specify the /select option) or examine the output of the PARTAB program for each disk.

#### PARTCMP command line

partcmp case host src-drv src-fill dst-drv dst-fill [/opts]

Table 11 PARTCMP Command Line Parameters

| Parameter | Description |
| --- | --- |
| case | A test case identifier |
| host | The name of the computer running the test |
| src-drv | The drive number in hex. Possible values begin at 80. The drive where the source drive is mounted. |
| src-fill | A two digit hex value, used to fill each sector by diskwipe. |
| dst-drv | The drive number in hex. Possible values begin at 80. The drive where the destination drive is mounted. |
| dst-fill | A two digit hex value, used to fill each sector by diskwipe. |
| /comment “…” | A comment for the test log file. If this option is not used, the program prompts for a log floppy to be inserted. If this option is used, then a log floppy is assumed to be already loaded. |
| /select sx dx | Set the value of the source (sx) and destination (dx) partition index. If this option is not given then the program presents the user with a list of partition table entries (and index values) for selection. |
| /new_log | Start a new log file rather than append to an existing log file. |
| /boot | Include the partition boot track in the comparison. |
| /? | Print a summary of command options and exit |

#### PARTCMP error messages

- /comment option requires a comment

This option must be followed by the text of the comment on the command line. If the comment includes any space characters then the entire comment must begin and end with a double quote character (“). For example,

a:\partcmp DI-01 lab 80 B4 81 AA/comment “Compare partition: B4 & AA”

- could not access [src/dst] drive %x status code %d

The specified disk drive could not be accessed. The status code indicates the type of problem. 3000 indicates the program ran out of memory. 2000 indicates that the disk is too large. Other values indicate a problem with the drive. Most likely cause is the drive number does not refer to an attached drive or the indicated drive may not be properly installed.

- [src/dst] drive %s is not valid

The drive number is not valid. Most likely valid drive numbers are 80 and 81, sometimes 82 or 83. Other values are not likely to be valid in the expected operating environment of the software.

- [src/dst] read error 0x%02X on track starting at lba %ld

The disk may have bad sectors. There is likely a hardware problem that must be resolved. It is also possible that either BADDISK or BADX13 was executed and has not been cleared from the system by turning the computer off and rebooting.

- /select requires two parameters: src partition index\n and dst partition index

This option must be followed by two integer values.

- could not find [src/dst] partition index %d

The partition index specified does not exist on the selected disk. Try running without the /select option or use the PARTAB program to get the valid index values.

- Source and destination drives must be different

Source and destination partitions cannot be on the same drive.

## Verify no changes to a disk: DISKHASH

DISKHASH is used to detect changes to a disk. DISKHASH calls the SHA-1 routines in SHA.CPP, SHAUTIL.CPP and CONFIG.H. DISKHASH logs the hash value and the number of sectors hashed. DISKHASH should be run once as soon as a source disk has been set up and before any tests are run. Then DISKHASH is run after each test case is run. Comparing the hash values from the before log with the after log determines if executing a test case has changed the source disk. If the hash values agree then the source disk has not changed.

### DISKHASH command line

diskhash case host drive [/opts]

Table 12 DISKHASH Command Line Parameters

| Parameter | Description |
| --- | --- |
| case | A test case identifier |
| host | The name of the computer running the test |
| drive | The drive number in hex. Possible values begin at 80. The drive where the source drive is mounted. |
| /comment “…” | A comment for the test log file. If this option is not used, the program prompts for a log floppy to be inserted. If this option is used, then a log floppy is assumed to be already loaded. |
| /before | Name the log file HASHBLOG.TXT (Used if DISKHASH is run before a test case is executed.) |
| /after | Name the log file HASHALOG.TXT (Used if DISKHASH is run after a test case is executed.) |
| /new_log | Start a new log file rather than append to an existing log file. |
| /? | Print a summary of command options and exit |

### DISKHASH log file

The name of the log file is determined by the setting of a command line option. The log file name is either A:\HASHBLOG.TXT if /before option is selected or A:\HASHALOG.TXT if /after option is selected.

### DISKHASH error messages

- /comment option requires a comment

This option must be followed by the text of the comment on the command line. If the comment includes any space characters then the entire comment must begin and end with a double quote character (“). For example,

a:\diskhash DI-01 lab 80 B4 /comment “Compute hash of B4” /before

- must select /before or /after

The command line must include one of these options.

- could not access drive %x status code %d

The specified disk drive could not be accessed. The status code indicates the type of problem. 3000 indicates the program ran out of memory. 2000 indicates that the disk is too large. Other values indicate a problem with the drive. Most likely cause is the drive number does not refer to an attached drive or the indicated drive may not be properly installed.

## Verify no changes to a block of sectors: SECHASH

SECHASH is used to detect changes to a contiguous range of disk sectors. SECHASH calls the SHA-1 routines in SHA.CPP and SHAUTIL.CPP. SECHASH logs the hash value and the number of sectors hashed. SECHASH should be run twice. Once as soon as the area of the disk to be monitored has been set up and before any tests are run. Then SECHASH is run a second time after the test case is run. Comparing the hash values from the before log with the after log determines if executing a test case has changed the area of the disk being monitored. If the hash values agree then no change has occurred.

The usual use of SECHASH is to determine if executing an imaging tool changes the excess destination sectors of a partition copy operation.

### SECHASH command line

sechash case host drive [/opts]

Table 13 SECHASH Command Line Parameters

| Parameter | Description |
| --- | --- |
| case | A test case identifier |
| host | The name of the computer running the test |
| drive | The drive number in hex. Possible values begin at 80. The drive where the source drive is mounted. |
| /comment “…” | A comment for the test log file. If this option is not used, the program prompts for a log floppy to be inserted. If this option is used, then a log floppy is assumed to be already loaded. |
| /before | Name the log file HASHBSEC.TXT (Used if SECHASH is run before a test case is executed.) |
| /after | Name the log file HASHASEC.TXT (Used if SECHASH is run after a test case is executed.) |
| /first n | Start hashing with the sector at LBA address n. If this option is not specified, hashing starts at the first sector of the drive. |
| /last n | Stop hashing with the sector at LBA address n. If this option is not specified, hashing stops with the last sector of the drive. |
| /new_log | Start a new log file rather than append to an existing log file. |
| /? | Print a summary of command options and exit |

### SECHASH log file

The name of the log file is determined by the setting of a command line option. The log file name is either A:\HASHBSEC.TXT if /before option is selected or A:\HASHASEC.TXT if /after option is selected.

### SECHASH error messages

- /comment option requires a comment

This option must be followed by the text of the comment on the command line. If the comment includes any space characters then the entire comment must begin and end with a double quote character (“). For example,

a:\SECHASH DI-01 lab 80 B4 /comment “Compute hash of B4” /before

- must select /before or /after

The command line must include one of these options.

- could not access drive %x status code %d

The specified disk drive could not be accessed. The status code indicates the type of problem. 3000 indicates the program ran out of memory. 2000 indicates that the disk is too large. Other values indicate a problem with the drive. Most likely cause is the drive number does not refer to an attached drive or the indicated drive may not be properly installed.

- [/first|/last] option requires a sector number

These options must be followed by an integer. This message indicates that nothing followed the option.

- [/first|/last] sector number not valid

These options must be followed by an integer. This message indicates that whatever followed the option was not an integer.

## Log a test case: LOGCASE

Log administrative information about a test case run in the file case.txt.

### LOGCASE command line

logcase test-case host operator src dst media

Table 14 LOGCASE Command Line Parameters

| Parameter | Description |
| --- | --- |
| test-case | A test case identifier. |
| host | The name of the computer running the test |
| Operator | The operator setting up the disk. |
| src | The source disk for the test. |
| dst | The destination disk for the test. |
| media | The media disk for the test. |

## Log Disk Setup: LOGSETUP

Log administrative information about the setup of a disk for use as a source disk for a group of test cases in the file setup.txt.

### LOGSETUP command line

logsetup disk host operator OS

Table 15 LOGSETUP Command Line Parameters

| Parameter | Description |
| --- | --- |
| disk | The disk to be setup. |
| host | The name of the computer running the test |
| Operator | The operator setting up the disk. |
| OS | The Operating System loaded to the disk. |

## Print a partition table: PARTAB

The PARTAB program logs the partition table of the specified disk. The name of the log file is PTXXLOG.TXT, where XX is replaced by the drive number. This program can be used to determine partition indices for the PARTCMP program.

### PARTAB command line

partab case host drive[/opts]

Table 16 PARTAB Command Line Parameters

| Parameter | Description |
| --- | --- |
| case | A test case identifier |
| host | The name of the computer running the test |
| drive | The drive number in hex. Possible values begin at 80. |
| /comment “…” | A comment for the test log file. If this option is not used, the program prompts for a log floppy to be inserted. If this option is used, then a log floppy is assumed to be already loaded. |
| /all | Print empty table entries and extended partition entries. |
| /new_log | Start a new log file rather than append to an existing log file. |
| /? | Print a summary of command options and exit |

### PARTAB error messages

- /comment option requires a comment

This option must be followed by the text of the comment on the command line. If the comment includes any space characters then the entire comment must begin and end with a double quote character (“). For example,

a:\partab DI-01 lab 80 /comment “Partition table for disk labeled B4”

- could not access drive %x status code %d

The specified disk drive could not be accessed. The status code indicates the type of problem. 3000 indicates the program ran out of memory. 2000 indicates that the disk is too large. Other values indicate a problem with the drive. Most likely cause is the drive number does not refer to an attached drive or the indicated drive may not be properly installed.

- No partition table signature

The signature byte after the partition table did not have the correct value (0xAA55).

- Error reading partition table, code %d

There was a disk read error while accessing the partition table. This is usually caused by a corrupt partition table entry that points beyond the end of the disk.

## Examine or Change a Disk: DISKCHG

DISKCHG is intended as a tool to aid testing the other support programs. DISKCHG has the following functions:

- Dump part of a sector in hexadecimal

- Change a single byte of a sector

- Zero the bytes of a sector

- Fill a sector in DISKWIPE style

### DISKCHG command line

diskchg test-case host drive [/opts]

Table 17 DISKCHG Command Line Parameters

| Parameter | Description |
| --- | --- |
| test-case | A test case identifier |
| host | The name of the computer running the test |
| drive | The drive number in hex. Possible values begin at 80. |
| /exam | Prompt the user for sectors to examine. |
| /read addr offset length | Print <length> bytes starting at <offset> from sector at <addr> |
| /write addr offset new_value | Replace byte at <offset> in sector at <addr> with <new_value> (in hex) |
| /fill addr fill_addr heads new_value | Fill sector at <addr> in DISKWIPE style for address <fill_addr> using a disk geometry of <heads> heads with fill byte of <new_value> (in hex) if <heads> is zero, then number of heads on disk is used |
| /zero addr | Set all bytes of sector at <addr> to zero <addr> can be specified as either an LBA address (an integer) or as cylinder/head/sector (three slash separated integers) |
| /comment | A comment for the test log file. If this option is not used, the program prompts for a log floppy to be inserted. If this option is used, then a log floppy is assumed to be already loaded. |
| /new_log | Start a new log file rather than append to an existing log file. |
| /? | Print a summary of command options and exit |

## Compare two disk sectors: SECCMP

The SECCMP program is used to compare two sectors. SECCMP is not a required part of any test. It is used to investigate unexpected test results.

### SECCMP command line

seccmp case host src-drv src-fill dst-drv dst-fill [/opts]

Table 18 SECCMP Command Line Parameters

| Parameter | Description |
| --- | --- |
| case | A test case identifier |
| host | The name of the computer running the test |
| src-drv | The drive number in hex. Possible values begin at 80. The drive where the source drive is mounted. |
| src-fill | A two digit hex value, used to fill each sector by diskwipe. |
| dst-drv | The drive number in hex. Possible values begin at 80. The drive where the destination drive is mounted. |
| dst-fill | A two digit hex value, used to fill each sector by diskwipe. |
| /comment “…” | A comment for the test log file. If this option is not used, the program prompts for a log floppy to be inserted. If this option is used, then a log floppy is assumed to be already loaded. |
| /sector src dst | The LBA addresses of two sectors to compare. If this option is omitted, the program asks for the addresses. |
| /log name | Override the default log file name (CMPSLOG.TXT) |
| /new_log | Start a new log file rather than append to an existing log file. |
| /? | Print a summary of command options and exit |

### SECCMP error messages

- /comment option requires a comment

This option must be followed by the text of the comment on the command line. If the comment includes any space characters then the entire comment must begin and end with a double quote character (“). For example,

a:\seccmp DI-01 lab 80 B4 81 AE /comment “Compare sector on B4 to AE”

- [src/dst] drive %s is not valid

The drive number is not valid. Most likely valid drive numbers are 80 and 81, sometimes 82 or 83. Other values are not likely to be valid in the expected operating environment of the software.

- [src/dst] read error 0x%02X at lba %ld

The disk may have bad sectors. There is likely a hardware problem that must be resolved. It is also possible that either BADDISK or BADX13 was executed and has not been cleared from the system by turning the computer off and rebooting.

- /sector option requires src and dst sector LBA addresses

This option must be followed by two integer values indicating the absolute LBA address of the source and destination sectors.

- Source and destination drives must be different

Source and destination partitions cannot be on the same drive.

## Copy a block of sectors: SECCOPY

Copy a block of sectors from one disk to another. This program has two main uses. First the program can be used to setup identical partitions for testing PARTCMP. Second, this program can be used to restore to the original contents a small portion of a source disk that has been changed.

### SECCOPY command line

seccopy test-case host src_drive src_addr dst_drive dst_addr length [/opts]

Table 19 SECCOPY Command Line Parameters

| Parameter | Description |
| --- | --- |
| test-case | A test case identifier |
| host | The name of the computer running the test |
| src_drive | The drive number in hex. Possible values begin at 80. The drive where the source drive is mounted. |
| src_addr | Address on the source drive of the first sector to copy. The sector address can be specified either as LBA or C/H/S/ |
| dst_drive | The drive number in hex. Possible values begin at 80. The drive where the destination drive is mounted. |
| dst_addr | Address on the destination drive to receive the first sector copied. The sector address can be specified either as LBA or C/H/S/ |
| length | The number of sectors to copy. |
| /comment “…” | A comment for the test log file. If this option is not used, the program prompts for a log floppy to be inserted. If this option is used, then a log floppy is assumed to be already loaded. |
| /new_log | Start a new log file rather than append to an existing log file. |
| /? | Print a summary of command options and exit |

# Example Test Case

This section illustrates using the software on an example test case. This example demonstrates how to use FS-TST to test a disk imaging tool. This example is not a tutorial on investigative procedure. The activities of testing a disk imaging tool have subtle differences from proper usage in an investigation of a disk imaging tool.

In this example, the tool must demonstrate compliance with each of the four test assertions listed under Test items in Table 20. The first assertion requires the tool to create a copy of a source disk on another disk of the same size. The copy, however, is not done directly. First, an image file is created and the destination is created from the image file. An image file is a file that contains all the information required to recreate an exact duplicate of the original. In the simplest case, an image file is a bit for bit copy of the original. However, some imaging tools include additional information in an image file to allow for verification of file integrity or even error correction. A separate disk drive, called the media disk, is used to contain the image file. The second assertion requires the tool to give notice of any I/O errors during the duplication process. The third assertion specifies treatment of the destination if an I/O error has occurred. The last assertion is critical for maintaining the integrity of any evidence on the source disk.

The test case is executed in three phases: setup disks, execute tool, and measure results. The setup phase creates the source, destination and media disks. After the source disk is created, a SHA-1 is computed for later reference. The execute phase runs any TSRs to simulate disk errors and then executes the tool. The measure results phase compares the source to the destination and computes a SHA-1 of the source disk. The SHA-1 is compared to the reference hash to verify that the source has not been changed.

Table 20 Example Test Case Specification

| Test case ID | EX-01 |
| --- | --- |
| Test items | - If a duplicate destination disk is created from an image file of a source disk with the same geometry, then the disks must compare equal. - If the tool encounters any read errors while reading from the source, then the tool must detect the error, and notify the user. - If the tool is able to create a destination from an image file that contains read errors, the destination sectors corresponding to the unreadable data must be treated as fill sectors (the tool may allow a specified action or may fill the sectors with zeros). - The contents of the source disk must not be changed by the tool. |
| Hardware Environment |  |
| PCs | One PC named Lab |
| Disks | Three disks: source disk labeled AA, destination disk labeled BB and media disk labeled CC |
| Partitions | One Fat 32 partition. |
| Output Specifications (expected results) | - The source and destination disks compare equal except for a block of sectors around the bad sector. - The tool under test logs an I/O error message - A SHA-1 hash of the source disk, computed before running the tool under test, is the same as a SHA-1 hash computed after running the tool under test. |

## Pre-test preparations

These programs are intended to support testing of a disk imaging tool. For this example, suppose that we are testing an imaginary imaging tool called NSIT (No Such Imaging Tool). There is not such a tool but it serves to illustrate using the support tools. We need the following:

- A test platform (pc to run the test) with three disk drives. In our test scenario the PC is a forensic Lab machine and the disk on drive 0x80 has been seized from a suspect. We want to make an image file on disk drive 0x82 and make a duplicate on disk drive 0x81 of the original disk that is installed as drive 0x80. The name of the PC is lab.forensics-lab.agency.gov (lab for short).

- Two of the disk drives (0x80 and 0x81) have the same geometry.

- A forensic DOS boot disk. i.e., the boot floppy that an investigator would use.

- The support software and NSIT loaded to a CD, called the tool CD, that is mounted as drive Z: on the PC when booted from the DOS boot disk.

- A floppy disk for keeping log files. This is referred to as the log floppy.

## Setup

The test case setup steps from Figure 1 General Structure of a Test Case are repeated in Table 21 along with the support software (or other) program required for the step. Note that for each execution of the diskwipe program a different command line option is indicated because of the different type of disk initialized in each step.

Table 21 Commands Used During Test Setup

| Step from Figure 1 | Program to run |
| --- | --- |
| - Record details of source disk setup. | logsetup |
| - Initialize the source disk to a known value. | diskwipe … /src |
| - Hash the source disk and save the hash value. | diskhash … /before |
| - Record details of test case setup. | logcase |
| - Initialize a destination disk. | diskwipe … /dst |
| - If the test requires a partition on the destination, create and format a partition on the destination disk. | fdisk, format or similar tool |
| - If the test uses an image file, partition and format a media disk. | diskwipe … /media |

During the setup phase the tester acts in multiple roles. Three disks have to be setup: a source, a destination and a media disk to contain the image file. When we are setting up the source disk we are acting in the role of the suspect. When setting up the destination and media disk we are in the role of the forensic investigator. The first part of the setup uses the diskwipe program to write a known pattern unique to each disk. This is not what the owner of the seized disk or a forensic investigator would do but it is important for testing the imaging tool to be able to determine easily the origin of any data on the destination disk. We will wipe the source disk with 0xAA, the destination disk with 0xBB and the media disk with 0xCC. After the source is setup the diskhash program computes a SHA-1 hash that can be used later to verify that the source disk has not been changed (compute the hash again later and if it is different then the source has been changed). Table 22 presents the commands issued in the setup phase of the test case. The column labeled Step indicates the step number from the general test case description presented in Figure 1. The Command column shows the actual command that would be typed. A description of what the command is accomplishing is in the Comments column. Steps 1-3 setup a source disk. Step 1 records that Joe, the operator, had disk labeled AA installed on drive 80h. In step 2, disk AA is erased by writing a unique pattern to each sector of the drive. After diskwipe is run, a FAT32 partition is created and Windows NT is loaded to the disk (the actions to do this are not shown in the table). In step 3, a SHA-1 is computed for the disk. The SHA-1 hash can be used to verify that disk AA has not been changed by some later action, e.g. by running an imaging tool.

Table 22 Example Test Case Setup

| Setup Actions |  |  |
| --- | --- | --- |
| Step | Command | Comments |
| 1 | z:\logsetup 80:aa lab joe “windows nt” | Record information about a source disk setup. The test operator, Joe, is setting up disk AA as the source disk (with Windows NT) for the test. The disk labeled AA is installed on BIOS drive 80h. Boot from DOS boot floppy, insert tool CD, replace floppy with log floppy, run logsetup to record source disk setup details. |
| 2 | z:\diskwipe EX-01 lab 80 aa /src | Write a unique pattern to each sector of the source disk. Run diskwipe. After running diskwipe, create a FAT 32 partition and load Windows NT to the FAT 32 partition. (only the diskwipe command is shown). |
| 3 | z:\diskhash EX-01 lab 80 /before | Compute a reference SHA-1 hash of the source disk. Boot from DOS boot floppy, insert tool CD, replace floppy with log floppy, run diskhash. The diskhash log file, HASHBLOG.TXT contains the reference hash for the source drive. |
| 4 | z:\logcase EX-01 lab joe aa bb cc | Record information about the test case. The destination disk is BB and disk CC is used to contain an image. |
| 5 | z:\diskwipe EX-01 lab 81 bb /dst | Write a unique pattern to each sector of the destination disk. This step erases the destination disk by writing a pattern to the destination that is different from the one written to the source disk. |
| 6 | No partition required on the destination. | No partition is required since the test case calls for an entire disk copy. |
| 7 | z:\diskwipe EX-01 lab 82 cc /media | Write a unique pattern to the media disk. After wiping the media disk, a partition must be created to hold the image (not shown). |

## Execute

The test case execute steps from Figure 1 General Structure of a Test Case are repeated in Table 23 along with the support software (or other) program required for the step.

Table 23 Commands Used During Tool Execution

| Step from Figure 1 | Program to run |
| --- | --- |
| - If the test requires a disk I/O error, setup disk error simulation. | baddisk if legacy BIOS, badx13 if extended BIOS |
| - If the test requires an image file, use the tool to create an image file of the source on the media disk. | the tool being tested |
| - If the test requires a corrupted image file, corrupt the image file. | corrupt |
| - Use the disk imaging tool to create the destination disk either from a copy of the source disk or by restoring an image file of the source to the destination. | the tool being tested |

During the execute phase we run the tool (NSIT). Since the test specification calls for a test of NSIT I/O error handling we use baddisk to simulate a bad sector at C/H/S address 00127/00063/00063. (a random sector picked for this test case). Since this is a read error, we must block both the read command and the read long command. (NSIT is very persistent in error recovery and tries the read long command if the read command fails.)

Table 24 Example Test Case Execution

| Execute Actions |  |  |
| --- | --- | --- |
| Step | Command | Comments |
| 8 | z:\baddisk 0 63 127 63 63 2 3 >bd-rd-17.txt z:\baddisk 0 63 127 63 63 2 10 >bd-rl-17.txt | Run TSRs to simulate a bad disk sector. Boot from DOS boot floppy, insert tool CD, replace floppy with log floppy, run baddisk twice |
| 9 | z:\nsit /image case-17.img | Run the imaging tool to create an image that is stored on the media disk as case-17.img |
| 10 | Nothing required here. |  |
| 11 | z:\nsit /restore case-17.img | Boot from DOS boot floppy to clear the baddisk TSR, insert tool CD, replace floppy with log floppy, run disk imaging tool to restore a copy of the source (from the image file) to the destination. |

## Measure

The test case measurement steps from Figure 1 General Structure of a Test Case are repeated inTable 25 along with the support software (or other) program required for the step.

Table 25 Commands Used to Measure Test Results

| Step from Figure 1 | Program to run |
| --- | --- |
| - Compare the source to the destination. | One of the three compare programs, disks: diskcmp partitions: partcmp aligned disk: adjcmp |
| - Compute a hash of the source disk, compare with the saved hash value. | diskhash … /after |

The measure actions are where we determine if the disk imaging tool yields the expected results.

Table 26 Example Test Case Measurement

| Execute Actions |  |  |
| --- | --- | --- |
| Step | Command | Comments |
| 12 | z:\diskcmp EX-01 80 aa 81 bb | Compare the source to the destination. The log file, CMPLOG.TXT should indicate that most sectors match except for a small number of sectors including the simulated bad sector. In the ideal result, the selected sector is the only sector that does not match. |
| 13 | z:\diskhash EX-01 80 /after | Verify that the source disk has not changed. Boot from DOS boot floppy, insert tool CD, replace floppy with log floppy, run disk hash. The hash in the two hash log files should match. |

An additional (optional) step would be to use the seccmp program to examine the content of the simulated bad sector to determine what NSIT did with the destination sector corresponding to the simulated bad sector from the source. Be sure the baddisk TSR is cleared from memory (i.e., cold boot) before running seccmp. The C/H/S address 00127/063/63 needs to be converted to a LBA address: 516095 for seccmp.

z:\seccmp EX-01 lab 80 AA 81 BB /sector 516095 516095

The relevant portion of the seccmp log file would look something like:

Compare sectors at: Src 516095 (0+516095) Dst 516095 (0+516095)

Src filled by AA from 00127/063/63 000000516095

Dst zero filled

511 bytes different

† Certain trade names and company products are mentioned in the text or identified. In no case does such identification imply recommendation or endorsement by the National Institute of Standards and Technology, nor does it imply that the products are necessarily the best available for the purpose.

* It should be noted that an I/O error encountered by a support program during test case setup or measurement is not the same as deliberately introducing an I/O error for the program being tested. If an I/O error occurs during setup or measurement then that process is incomplete or incorrect and therefore not usable. But an I/O error may be deliberately introduced for the tool being tested without any impact on the support programs since the I/O error only occurs during the tool execution.