WORKSHOP CASE STUDY

The following is a description of a single story frame to be used as a case study during the second AISC CIMsteel technical workshop.  We would appreciate it if each attendee would illustrate their interpretation of the written description of this case study using any desired software application.

For example, if you are representing a structural analysis/design software development company, we would appreciate it if you would develop a 3D model using your application and then perform a structural analysis (and, if you choose, design) on the structure using the specified applied loading.

If you are representing a 3D modeling software development company, we would appreciate it if you would develop a comprehensive 3D model using your application.

If you are representing a detailing software development company, we would appreciate it if you would develop a comprehensive 3D model and accompanying fabrication drawings using your application.

Finally, if you are representing an MIS software development company, we would appreciate it if you would develop a comprehensive material take-off and estimate for completing the fabrication and construction/erection phases of the structure.  Furthermore, it would be helpful if you would provide a detailed overview of how the application will account for each piece throughout each of the aforementioned phases.

In light of this exercise, we would appreciate it if you would please bring a notebook computer with your specific software application installed so that you may demonstrate your interpretation of the case study if called upon (due to time constraints, not all participants will be able to demonstrate their interpretation of the case study).

In order to prepare for the workshop, please email screen captures (optimized and compressed as much as possible using PKZIP) of all pertinent information in either JPEG or GIF format to Steve Hamburg by 12:00 PM Central Time, Monday, June 21, 1999.  If you choose to use an alternate file format, please insure that you provide Steve with means of accessing the files.

# DEFINITIONS OF INTERNAL DATA STRUCTURES

One critical element of the upcoming workshop involves the development of mapping tables.  The purpose of this exercise is to train workshop attendees to assess the scope and types of data each specific software application will be importing and exporting to other software applications through Part 21 files.  It is imperative that each attending software development company bring complete definitions of the internal data structures (i.e. variable names along with specific data each variable contains) for an existing application to be shared with other workshop participants.

If this is a sensitive issue, please consider providing definitions of data structures for an outdated or even discontinued software application.  The effectiveness and applicability of this workshop hinges upon software development company participants sharing their internal data structures with other workshop attendees.  Finally, please have the definitions of the internal data structures available in electronic form during the upcoming workshop.

We sincerely appreciate your participating in this exercise.  Thank you very much for your cooperation.

Steven E. Hamburg, P.E.				Mark V. Holland

Director of Electronic Communications			Chief Engineer

AISC							Paxton & Vierling Steel

Dr. Alastair Watson					Dr. Andrew Crowley

Senior Lecturer in CAE					Principal Engineer

The University of Leeds					Steel Construction Institute

## CASE STUDY DESCRIPTION

Project Information:

Engineers of Record – Mark Holland (Lead Engineer) and Steven Hamburg (Assistant Engineer)

Engineering Consulting Company – Hamburg Consulting, Inc.

Fabricator – Paxton & Vierling Steel

Architect – Carlos Aguera Aesthetics, LLC

Owner – Watson, Crowley, Hamburg & Holland Enterprises, Inc.

Purpose of Structure – To support two coffee extractors for a food & beverage facility

Location – Miami, FL

Wind condition must be designed for 100-year hurricane.

Geometry of the Frame:

- The frame consists of a single story, one bay in width by two bays in length.

- The distance from the bottom of the base plate to the top of the beam flanges (T.O.S) is 25 feet.

- The length of the structure runs East / West.

- The single width of the structure runs North / South.

- The width of the structure is 25 feet (center-to-center of girders).

- The length of the structure consists of two bays, each 30 feet in length (center-to-center of girders).

- There are two in-fill beams in each bay running North / South. These in-fill beams are located at 10 feet on center.

Frame Lateral Restraint and Connections:

- Each 25 foot spanning bay (girder running North / South) makes a rigid connection to the supporting column. 
The beam end connection is designed to fully develop the member’s capacity.
This is to be a bolted connection.

- Each 30-foot bay (both sides) are to be “X” braced frames using double angles. The work points for the bracing is the gravity axis of all connected elements except the angle bracing. The work line will be the angles’ gage line.
The brace end connections will be designed to develop ½ of the member’s axial capacity or as required by the applied loads.

- Horizontal cross bracing is to be used in both bays using structural tees with stem pointing downward.  The horizontal bracing in the western-most bay will start at the southwest corner and run to the northeastern corner.  The horizontal bracing in the eastern-most bay will start at the northwest corner and run to the southeastern corner.

- The horizontal bracing members are to be bolted to the in-fill beams as they run under the in-fill beams using filler plates as necessary.  The horizontal bracing members are connected to the in-fill beams framing at the column locations with ½” thick gusset plates and (4) four A325 ¾” bolts.

- The in-fill beams’ end connections are to be designed to develop ½ of the member’s allowable bending capacity. The connections shall consist of using shear tabs.

- The column bases will not be designed to transfer any moment.

Loading Conditions

- Apply a 1.25 KLF wind load to each girder (one load case for each direction).

- Apply a 100 PSF live load over entire 60 ft x 25 ft plan area of the structure.

- Analyze structure to include selfweight of all structural members in DL loading condition.

- Apply a 50 PSF dead load to each in-fill beam.

- Analyze structure for following load cases and combinations

- DL only

- DL + LL

- DL + LL + WL (North Direction)

## CASE STUDY DESCRIPTION (Continued)

Frame Member Sizes and Grade:

- All columns are W27x194 members. The web is oriented running North / South.

- The Girders running North / South (the 25 foot span) are W24x68 members.

- The Girders running East / West (the 30 foot span) are W21x44 members.

- The in-fill beams are W16x36 members.

- All angle braces are double angles, L8x8x¾.

- The horizontal bracing members are WT6x11 structural tees.

- All main members are made from A572 Grade 50.

All connection material is made from A36 material.
The angle braces are made from A36 material.

Added Conditions:

- All bolts are to be A325 ¾” or 1”.

All steel will be painted using a Zinc-rich paint.