III.  Uniform Laws

Page

8T46TA.	Uniform Weights and Measures Law	13

8T46TB.	Uniform Weighmaster Law	29

8T46TC.	Uniform Engine Fuels and Automotive Lubricants Inspection Law	41

THIS PAGE INTENTIONALLY LEFT BLANK

A.  Uniform Weights and Measures Law

as adopted by

The National Conference on Weights and Measures*

###### 1.	Background

Recognition of the need for uniformity in weights and measures laws and regulations among the states was first noted at the second Annual Meeting of the National Conference on Weights and Measures (NCWM) in April 1906.  In the following year, basic outlines of a “Model State Weights and Measures Law” were developed.  The first “Model Law,” as such, was formally adopted by the Conference in 1911.

Through the years, almost without exception, each state has relied upon the NCWM Weights and Measures Law when the state first enacted comprehensive weights and measures legislation.  This has led to a greater degree of uniformity in the basic weights and measures requirements throughout the country.

The original Law was regularly amended to provide for new developments in commercial practices and technology.  This resulted in a lengthy and cumbersome document and the need for a simplification of the basic weights and measures provisions.  The 1971 NCWM adopted a thoroughly revised, simplified, modernized version of the “Model State Weights and Measures Law.”  This Law now can serve as a framework for all the many concerns in weights and measures administration and enforcement.

The title of the Law was changed by the 1983 NCWM.  Amendments or revisions to the Law since 1971 are noted at the end of each section.

Sections 4 through 10 of the Uniform Weights and Measures Law adopt NIST Handbook 44 and the Uniform Regulations in NIST Handbook 130 by citation.  In addition, these sections adopt supplements to and revisions of Handbook 44 and the Uniform Regulations “except insofar as modified or rejected by regulation.”  Some state laws may not permit enacting a statute that provides for automatic adoption of future supplements to or revisions of a Uniform Regulation covered by that statute.  If this should be the case in a given state, two alternatives are available:

- Sections 4 through 10 may be enacted without the phrase “. . . and supplements thereto or revisions thereof . . .”; or

- Sections 4 through 10 may be enacted by replacing “. . . except insofar as modified or rejected by regulation . . .” with the phrase “. . . as adopted, or amended and adopted, by rule of the director.”

Either alternative requires action on the part of the Director to adopt a current version of Handbook 44 and each Uniform Regulation each time a supplement or revision is made by the NCWM.

###### 2.	Status of Promulgation

See the table beginning on page 10, Section II. Uniformity of Laws and Regulations of Handbook 130 for the status of adoption of the Uniform Weights and Measures Law.

*The National Conference on Weights and Measures (NCWM) is supported by National Institute of Standards and Technology (NIST) in partial implementation of its statutory responsibility for “cooperation with the states in securing uniformity in weights and measures laws and methods of inspection.”

THIS PAGE INTENTIONALLY LEFT BLANK

Uniform Weights and Measures Law

Table of Contents

Section	Page

8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T8T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46T46TSection 1.  Definitions	17

1.1.	Weight(s) and (or) Measure(s).	17

1.2.	Weight.	17

1.3.	Correct.	17

1.4.	Director.	17

1.5.	Person.	17

1.6.	Sale from Bulk.	17

1.7.	Package.	17

1.8.	Net “Mass” or Net “Weight.”	17

1.9.	Random Weight Package.	17

1.10.	Standard Package.	18

1.11.	Commercial Weighing and Measuring Equipment.	18

1.12.	Standard, Field.	18

1.13.	Accreditation.	18

1.14.	Calibration.	18

1.15.	Metrological Traceability.	18

1.16.	Measurement Uncertainty.	18

1.17.	Verification.	18

1.18. 	Recognition.	18

1.19.	Standard, Reference Measurement.	19

1.20.	Standard, Working Measurement.	19

1.21.	Metrological Traceability Chain.	19

1.22. Metrological Traceability to a Measurement Unit.	19

Section 2.  Systems of Weights and Measures	19

Section 3.  Physical Standards	19

Section 4.  Technical Requirements for Weighing and Measuring Devices	19

Section 5.  Requirements for Packaging and Labeling	20

Section 6.  Requirements for the Method of Sale of Commodities	20

Section 7.  Requirements for Unit Pricing	20

Section 8.  Requirements for the Registration of Servicepersons and Service Agencies for Commercial Weighing and Measuring Devices	20

Section 9.  Requirements for Open Dating	21

Section 10.  Requirements for Type Evaluation	21

Section 11.  State Weights and Measures Division	21

Section 12.  Powers and Duties of the Director	21

Section 13.  Special Police Powers	23

Section 14.  Powers and Duties of Local Officials	23

Section 15.  Misrepresentation of Quantity	24

Section 16.  Misrepresentation of Pricing	24

Section 17.  Method of Sale	24

Section 18.  Sale from Bulk	24

Section 19.  Information Required on Packages	25

Section 20.  Declarations of Unit Price on Random Weight Packages	25

Section 21.  Advertising Packages for Sale	25

Section 22.  Prohibited Acts	25

Section 23.  Civil Penalties	26

23.1.	Assessment of Penalties.	26

23.2.	Administrative Hearing.	26

23.3.	Collection of Penalties.	26

Section 24.  Criminal Penalties	26

24.1.  Misdemeanors.	26

24.2.	Felonies.	26

Section 25.  Restraining Order and Injunction	26

Section 26.  Presumptive Evidence	27

Section 27.  Separability Provision	27

Section 28.  Repeal of Conflicting Laws	27

Section 29.  Regulations to be Unaffected by Repeal of Prior Enabling Statute	27

Section 30.  Effective Date	27

Uniform Weights and Measures Law

###### Section 1.  Definitions

When used in this Act:

  - Weight(s) and (or) Measure(s). – The term “weight(s) and (or) measure(s)” means all weights and measures of every kind, instruments and devices for weighing and measuring, and any appliance and accessories associated with any or all such instruments and devices.

1.2.	Weight. – The term “weight” as used in connection with any commodity or service means net weight.  When a commodity is sold by drained weight, the term means net drained weight.

(Amended 1974 and 1990)

1.3.	Correct. – The term “correct” as used in connection with weights and measures means conformance to all applicable requirements of this Act.

1.4.	Director. – The term “director” means the __________ of the Department of __________.

1.5.	Person. – The term “person” means both plural and the singular, as the case demands, and includes individuals, partnerships, corporations, companies, societies, and associations.

1.6.	Sale from Bulk.  – The term “sale from bulk” means the sale of commodities when the quantity is determined at the time of sale.

1.7.	Package. – Except as modified by Section 1. Application of the Uniform Packaging and Labeling Regulation, the term “package,” whether standard package or random package, means any commodity:

- enclosed in a container or wrapped in any manner in advance of wholesale or retail sale; or

- whose weight or measure has been determined in advance of wholesale or retail sale.

An individual item or lot of any commodity on which there is marked a selling price based on an established price per unit of weight or of measure shall be considered a package (or packages).

(Amended 1991)

1.8.	Net “Mass” or Net “Weight.” – The term “net mass” or “net weight” means the weight [NOTE 1, page 17] of a commodity excluding any materials, substances, or items not considered to be part of the commodity.  Materials, substances, or items not considered to be part of the commodity include, but are not limited to, containers, conveyances, bags, wrappers, packaging materials, labels, individual piece coverings, decorative accompaniments, and coupons, except that, depending on the type of service rendered, packaging materials may be considered to be part of the service.  For example, the service of shipping includes the weight of packing materials.

(Added 1988) (Amended 1989, 1991, and 1993)

1.9.	Random Weight Package. – A package that is one of a lot, shipment, or delivery of packages of the same commodity with no fixed pattern of weights.

(Added 1990)

NOTE 1:  When used in this Law, the term “weight” means “mass.”  (See paragraphs L. “Mass” and “Weight” and V. Use of the Terms “Mass” and “Weight” in Section I. Introduction of NIST Handbook 130 for an explanation of these terms.)

(Note added 1993)

  - Standard Package. – A package that is one of a lot, shipment, or delivery of packages of the same commodity with identical net contents declarations.

Examples:

l L bottles or 12 fl oz cans of carbonated soda

500 g or 5 lb bags of sugar

100 m or 300 ft packages of rope

(Added 1991) (Amended 1993)

  - Commercial Weighing and Measuring Equipment. – The term “commercial weighing and measuring equipment” means weights and measures and weighing and measuring devices commercially used or employed in establishing the size, quantity, extent, area, or measurement of quantities, things, produce, or articles for distribution or consumption, purchased, offered, or submitted for sale, hire, or award, or in computing any basic charge or payment for services rendered on the basis of weight or measure.

(Added 1995)

  - Standard, Field. – A physical standard that meets specifications and tolerances in NIST Handbook 105-series standards (or other suitable and designated standards) and is traceable to the reference or working standards through comparisons, using acceptable laboratory procedures, and used in conjunction with commercial weighing and measuring equipment (1.13. Accreditation).

(Added 2005)

  - Accreditation. – A formal recognition by a recognized Accreditation Body that a laboratory is competent to carry out specific tests or calibrations or types of tests or calibrations.  NOTE:  Accreditation does not ensure compliance of standards to appropriate specifications.

(Added 2005)

  - Calibration. – An operation that, under specified conditions, in a first step, establishes a relation between the quantity values with measurement uncertainties provided by measurement standards and corresponding indications with associated measurement uncertainties and, in a second step, uses this information to establish a relation for obtaining a measurement result from an indication.

(Added 2005) (Amended 2013)

  - Metrological Traceability. – The property of a measurement result whereby the result can be related to a reference through a documented unbroken chain of calibrations, each contributing to the measurement uncertainty.

(Added 2005) (Amended 2013)

1.16.	Measurement Uncertainty. – A non-negative parameter characterizing the dispersion of the quantity values being attributed to a measurand, based on the information used.

(Added 2005) (Amended 2013)

1.17.	Verification. – The formal evaluation of a standard or device against the specifications and tolerances for determining conformance.

(Added 2005)

1.18. 	Recognition. – A formal recognition by NIST Office of Weights and Measures that a laboratory has demonstrated the ability to provide traceable measurement results and is competent to carry out specific tests or calibrations or types of tests or calibrations.

(Added 2005)

1.19.	Standard, Reference Measurement. – A measurement standard designated for the calibration of other measurement standards for quantities of a given kind in a given organization or at a given location.  The term “reference measurement standards” usually means the physical standards of the state that serve as the legal reference from which all other standards for weights and measures within that state are derived.

(Added 2005) (Amended 2013)

1.20.	Standard, Working Measurement. – A measurement standard that is used routinely to calibrate or verify measuring instruments or measuring systems.  The term “working measurement standards” means the physical standards that are traceable to the reference standards through calibrations or verifications, using acceptable laboratory procedures, and used in the enforcement of weights and measures laws and regulations.

(Added 2005) (Amended 2013)

1.21.	Metrological Traceability Chain. – Sequence of measurement standards and calibrations that is used to relate a measurement result to a reference.

(Added 2013)

1.22. Metrological Traceability to a Measurement Unit. – Metrological traceability where the reference is the definition of a measurement unit through its practical realization.

(Added 2013)

###### Section 2.  Systems of Weights and Measures

The International System of Units (SI) and the system of weights and measures in customary use in the United States are jointly recognized, and either one or both of these systems shall be used for all commercial purposes in the state.

The definitions of basic units of weight and measure, the tables of weight and measure, and weights and measures equivalents as published by NIST are recognized and shall govern weighing and measuring equipment and transactions in the state.

(Amended 1993)

NOTE 2:  SI or SI Unit. – means the International System of Units as established in 1960 by the General Conference on Weights and Measures and interpreted or modified for the United States by the Secretary of Commerce.  See “Interpretation of the International System of Units for the United States” in “Federal Register” (Volume 73, No. 96, pages 28432 to 28433) for May 16, 2008, and 15 United States Code, Section 205a - 205l “Metric Conversion.”  See also NIST Special Publication 330, “The International System of Units (SI),” 2008 edition and NIST Special Publication 811, “Guide for the Use of the International System of Units (SI),” 2008 edition that are available at www.nist.gov/pml/wmd/metric/index.cfm or by contacting TheSI@nist.gov.

(Added 1993)

###### Section 3.  Physical Standards

Weights and measures that are traceable to the U.S. prototype standards supplied by the Federal Government, or approved as being satisfactory by NIST, shall be the state reference and working standards of weights and measures, and shall be maintained in such calibration as prescribed by the NIST as demonstrated through laboratory accreditation or recognition.  All field standards may be prescribed by the Director and shall be verified upon their initial receipt and as often thereafter as deemed necessary by the Director.

(Amended 2005)

Section 4.  Technical Requirements for Weighing and Measuring Devices [NOTE 3, page 20]

The specifications, tolerances, and other technical requirements for commercial, law enforcement, data gathering, and other weighing and measuring devices as adopted by the NCWM, published in the National Institute of Standards and Technology Handbook 44, “Specifications, Tolerances, and Other Technical Requirements for Weighing and Measuring Devices,” and supplements thereto or revisions thereof, shall apply to weighing and measuring devices in the state, except insofar as modified or rejected by regulation.

(Amended 1975)

NOTE 3:  Sections 4 through 10 of the Uniform Weights and Measures Law adopt NIST Handbook 44 and Uniform Regulations in NIST Handbook 130 by citation.  In addition, these sections adopt supplements to and revisions of NIST Handbook 44 and the Uniform Regulations “except insofar as modified or rejected by regulation.”  Some state laws may not permit enacting a statute that provides for automatic adoption of future supplements to or revisions of a regulation covered by that statute.  If this should be the case in a given state, two alternatives are available:

- Sections 4 through 10 may be enacted without the phrase “. . . and supplements thereto or revisions thereof . . .”; or

- Sections 4 through 10 may be enacted by replacing “. . . except insofar as modified or rejected by regulation . . .” with the phrase “. . . as adopted, or amended and adopted, by rule of the director.”

Either alternative requires action on the part of the Director to adopt a current version of Handbook 44 and Uniform Laws or Regulations each time a supplement is added or revision is made by the NCWM.

Section 5.  Requirements for Packaging and Labeling [NOTE 3, page 20]

The Uniform Packaging and Labeling Regulation as adopted by the NCWM and published in the National Institute of Standards and Technology Handbook 130, “Uniform Laws and Regulations,” and supplements thereto or revisions thereof, shall apply to packaging and labeling in the state, except insofar as modified or rejected by regulation.

(Added 1983)

Section 6.  Requirements for the Method of Sale of Commodities [NOTE 3, page 20]

The Uniform Regulation for the Method of Sale of Commodities as adopted by the NCWM and published in National Institute of Standards and Technology Handbook 130, “Uniform Laws and Regulations,” and supplements thereto or revisions thereof, shall apply to the method of sale of commodities in the state, except insofar as modified or rejected by regulation.

(Added 1983)

Section 7.  Requirements for Unit Pricing [NOTE 3, page 20]

The Uniform Unit Pricing Regulation as adopted by the NCWM and published in the National Institute of Standards and Technology Handbook 130, “Uniform Laws and Regulations,” and supplements thereto or revisions thereof, shall apply to unit pricing in the state, except insofar as modified or rejected by regulation.

(Added 1983)

Section 8.  Requirements for the Registration of Servicepersons and Service Agencies for Commercial Weighing and Measuring Devices [NOTE 3, page 20]

The Uniform Regulation for the Voluntary Registration of Servicepersons and Service Agencies for Commercial Weighing and Measuring Devices as adopted by the National NCWM and published in the National Institute of Standards and Technology Handbook 130, “Uniform Laws and Regulations,” and supplements thereto or revisions thereof, shall apply to the registration of servicepersons and service agencies in the state, except insofar as modified or rejected by regulation.

(Added 1983)

Section 9.  Requirements for Open Dating [NOTE 3, page 20]

The Uniform Open Dating Regulation as adopted by the NCWM and published in the National Institute of Standards and Technology Handbook 130, “Uniform Laws and Regulations,” and supplements thereto or revisions thereof, shall apply to open dating in the state, except insofar as modified or rejected by regulation.

(Added 1983)

Section 10.  Requirements for Type Evaluation [NOTE 3, page 20]

The Uniform Regulation for National Type Evaluation as adopted by the NCWM and published in National Institute of Standards and Technology Handbook 130, “Uniform Laws and Regulations,” and supplements thereto or revisions thereof, shall apply to type evaluation in the state, except insofar as modified or rejected by regulation.

(Added 1985)

###### Section 11.  State Weights and Measures Division

There shall be a State Division of Weights and Measures located for administrative purposes within the Department of _________ (agency, etc.).  The Division is charged with, but not limited to, performing the following functions on behalf of the citizens of the state:

- Assuring that weights and measures in commercial services within the state are suitable for their intended use, properly installed, and accurate, and are so maintained by their owner or user.

- Preventing unfair or deceptive dealing by weight or measure in any commodity or service advertised, packaged, sold, or purchased within the state.

- Making available to all users of physical standards or weighing and measuring equipment the precision calibration and related metrological certification capabilities of the weights and measures facilities of the Division.

- Promoting uniformity, to the extent practicable and desirable, between weights and measures requirements of this state and those of other states and federal agencies.

- Encouraging desirable economic growth while protecting the consumer through the adoption by rule of weights and measures requirements as necessary to assure equity among buyers and sellers.

(Added 1976)

Section 12.  Powers and Duties of the Director

The Director shall:

- maintain traceability of the state standards as demonstrated through laboratory accreditation or recognition;

(Amended 2005)

- enforce the provisions of this Act;

- issue reasonable regulations for the enforcement of this Act, which regulations shall have the force and effect of law;

- establish labeling requirements, establish requirements for the presentation of cost per unit information, establish standards of weight, measure, or count, and reasonable standards of fill for any packaged commodity; and establish requirements for open dating information;

(Added 1973)

- grant any exemptions from the provisions of this Act or any regulations promulgated pursuant thereto when appropriate to the maintenance of good commercial practices within the state;

- conduct investigations to ensure compliance with this Act;

- delegate to appropriate personnel any of these responsibilities for the proper administration of this office;

- verify the field standards for weights and measures used by any jurisdiction within the state, before being put into service, tested annually or as often thereafter as deemed necessary by the Director based on statistically evaluated data, and approve the same when found to be correct;

(Amended 2005)

- have the authority to inspect and test commercial weights and measures kept, offered, or exposed for sale;

(Amended 1995)

- inspect and test, to ascertain if they are correct, weights and measures commercially used:

- in determining the weight, measure, or count of commodities or things sold, or offered or exposed for sale, on the basis of weight, measure, or count; or

- in computing the basic charge or payment for services rendered on the basis of weight, measure, or count.

- test all weights and measures used in checking the receipt or disbursement of supplies in every institution, the maintenance of which funds are appropriated by the legislature of the state;

- approve for use, and may mark, such commercial weights and measures as are found to be correct, and shall reject and order to be corrected, replaced, or removed such commercial weights and measures as are found to be incorrect.  Weights and measures that have been rejected may be seized if not corrected within the time specified or if used or disposed of in a manner not specifically authorized.  The Director shall remove from service and may seize the weights and measures found to be incorrect that are not capable of being made correct;

(Amended 1995)

- weigh, measure, or inspect packaged commodities kept, offered, or exposed for sale, sold, or in the process of delivery, to determine whether they contain the amounts represented and whether they are kept, offered, or exposed for sale in accordance with this Act or regulations promulgated pursuant thereto.  In carrying out the provisions of this section, the Director shall employ recognized sampling procedures, such as are adopted by the NCWM and are published in the National Institute of Standards and Technology Handbook 133, “Checking the Net Contents of Packaged Goods;”

(Amended 1984, 1988, and 2000)

- prescribe, by regulation, the appropriate term or unit of weight or measure to be used, whenever the Director determines that an existing practice of declaring the quantity of a commodity or setting charges for a service by weight, measure, numerical count, time, or combination thereof, does not facilitate value comparisons by consumers, or offers an opportunity for consumer confusion;

(Amended 1991)

- allow reasonable variations from the stated quantity of contents, which shall include those caused by loss or gain of moisture during the course of good distribution practice or by unavoidable deviations in good manufacturing practice only after the commodity has entered intrastate commerce;

- provide for the training of weights and measures personnel, and may establish minimum training and performance requirements, which shall then be met by all weights and measures personnel, whether county, municipal, or state.  The Director may adopt the training standards of the National Conference on Weights and Measures’ National Training Program and the laboratory metrology standards specified by the NIST accreditation and/or recognition requirements; and

(Added 1991) (Amended 2005)

- verify advertised prices, price representations, and point-of-sale systems, as deemed necessary, to determine:

- the accuracy of prices and computations and the correct use of the equipment; and

- if such system utilizes scanning or coding means in lieu of manual entry, the accuracy of prices printed or recalled from a database.  In carrying out the provisions of this section, the Director shall:

  - employ recognized procedures, such as are designated in National Institute of Standards and Technology Handbook 130, Uniform Laws and Regulations in the Areas of Legal Metrology and Engine Fuel Quality, “Examination Procedures for Price Verification;”

  - issue necessary rules and regulations regarding the accuracy of advertised prices and automated systems for retail price charging (referred to as “point-of-sale systems”) for the enforcement of this section, which rules shall have the force and effect of law; and

  - conduct investigations to ensure compliance.

(Added 1995)

Section 13.  Special Police Powers

When necessary for the enforcement of this Act or regulations promulgated pursuant thereto, the Director is:

- Authorized to enter any commercial premises during normal business hours, except that in the event such premises are not open to the public, he/she shall first present his/her credentials and obtain consent before making entry thereto, unless a search warrant has previously been obtained.

- Empowered to issue stop use, hold, and removal orders with respect to any weights and measures commercially used, stop sale, hold, and removal orders with respect to any packaged commodities or bulk commodities kept, offered, or exposed for sale.

- Empowered to seize, for use as evidence, without formal warrant, any incorrect or unapproved weight, measure, package, or commodity found to be used, retained, offered, or exposed for sale or sold in violation of the provisions of this Act or regulations promulgated pursuant thereto.

- Empowered to stop any commercial vehicle and, after presentation of his credentials, inspect the contents, require the person in charge of that vehicle to produce any documents in his possession concerning the contents, and require him to proceed with the vehicle to some specified place for inspection.

- With respect to the enforcement of this Act, the Director is hereby vested with special police powers, and is authorized to arrest, without formal warrant, any violator of this Act.

Section 14.  Powers and Duties of Local Officials

Any weights and measures official appointed for a county or city shall have the duties and powers enumerated in this Act, excepting those duties reserved to the state by law or regulation.  These powers and duties shall extend to their respective jurisdictions, except that the jurisdiction of a county official shall not extend to any city for which a weights and measures official has been appointed.  No requirement set forth by local agencies may be less stringent than or conflict with the requirements of the state.

(Amended 1984)

Section 15.  Misrepresentation of Quantity

No person shall:

- sell, offer, or expose for sale a quantity less than the quantity represented; nor

- take more than the represented quantity when, as buyer, he/she furnishes the weight or measure by means of which the quantity is determined; nor

- represent the quantity in any manner calculated or tending to mislead or in any way deceive another person.

(Amended 1975 and 1990)

Section 16.  Misrepresentation of Pricing

No person shall misrepresent the price of any commodity or service sold, offered, exposed, or advertised for sale by weight, measure, or count, nor represent the price in any manner calculated or tending to mislead or in any way deceive a person.

Section 17.  Method of Sale

Except as otherwise provided by the Director or by firmly established trade custom and practice,

- commodities in liquid form shall be sold by liquid measure or by weight; and

- commodities not in liquid form shall be sold by weight, by measure, or by count.

The method of sale shall provide accurate and adequate quantity information that permits the buyer to make price and quantity comparisons.

(Amended 1989)

Section 18.  Sale from Bulk

All bulk sales in which the buyer and seller are not both present to witness the measurement, all bulk deliveries of heating fuel, and all other bulk sales specified by rule or regulation of the director shall be accompanied by a delivery ticket containing the following information:

- the name and address of the buyer and seller;

- the date delivered;

- the quantity delivered and the quantity upon which the price is based, if this differs from the delivered quantity for example, when temperature compensated sales are made;

(Amended 1991)

- the unit price, unless otherwise agreed upon by both buyer and seller;

(Added 1991)

- the identity in the most descriptive terms commercially practicable, including any quality representation made in connection with the sale; and

- the count of individually wrapped packages, if more than one, in the instance of commodities bought from bulk but delivered in packages.

(Amended 1983 and 1991)

Section 19.  Information Required on Packages

Except as otherwise provided in this Act or by regulations promulgated pursuant thereto, any package, whether a random package or a standard package, kept for the purpose of sale, or offered or exposed for sale, shall bear on the outside of the package a definite, plain, and conspicuous declaration of:

- the identity of the commodity in the package, unless the commodity is a food, other than meat or poultry, that was repackaged in a retail establishment and the food is displayed to the purchaser under either of the following circumstances:

      - its interstate labeling is clearly in view or with a counter card, sign or other appropriate device bearing prominently and conspicuously the common or usual name of the food; or

      - the common or usual name of the food is clearly revealed by its appearance.

(Amended 2001)

- the quantity of contents in terms of weight, measure, or count; and,

- the name and place of business of the manufacturer, packer, or distributor, in the case of any package kept, offered, or exposed for sale, or sold in any place other than on the premises where packed.

(Amended 1991)

Section 20.  Declarations of Unit Price on Random Weight Packages

In addition to the declarations required by Section 19. Information Required on Packages of this Act, any package being one of a lot containing random weights of the same commodity, at the time it is offered or exposed for sale at retail, shall bear on the outside of the package a plain and conspicuous declaration of the price per kilogram or pound and the total selling price of the package.

(Amended 1986)

Section 21.  Advertising Packages for Sale

Whenever a packaged commodity is advertised in any manner with the retail price stated, there shall be closely and conspicuously associated with the retail price a declaration of quantity as is required by law or regulation to appear on the package.

(Amended 1993)

###### Section 22.  Prohibited Acts

No person shall:

- use or have in possession for use in commerce any incorrect weight or measure;

- sell or offer for sale for use in commerce any incorrect weight or measure;

- remove any tag, seal, or mark from any weight or measure without specific written authorization from the proper authority;

- hinder or obstruct any weights and measures official in the performance of his or her duties; or

- violate any provisions of this Act or regulations promulgated under it.

###### Section 23.  Civil Penalties

23.1.	Assessment of Penalties. – Any person who by himself or herself, by his or her servant or agent, or as the servant or agent of another person, commits any of the acts enumerated in Section 22. Prohibited Acts may be assessed by the __________ a civil penalty of:

- not less than $__________ nor more than $__________ for a first violation;

- not less than $__________ nor more than $__________ for a second violation within __________ from the date of the first violation; and

- not less than $__________ nor more than $__________ for a third violation within __________ from the date of the first violation.

23.2.	Administrative Hearing. – Any person subject to a civil penalty shall have a right to request an administrative hearing within __________ days of receipt of the notice of the penalty.  The Director or his/her designee shall be authorized to conduct the hearing after giving appropriate notice to the respondent.  The decision of the Director shall be subject to appropriate judicial review.

23.3.	Collection of Penalties. – If the respondent has exhausted his or her administrative appeals and the civil penalty has been upheld, he or she shall pay the civil penalty within __________ days after the effective date of the final decision.  If the respondent fails to pay the penalty, a civil action may be brought by the Director in any court of competent jurisdiction to recover the penalty.  Any civil penalty collected under this Act shall be transmitted to __________.

(Added 1989) (Amended 1995)

Section 24.  Criminal Penalties

24.1.  Misdemeanors. – Any person who commits any of the acts enumerated in Section 22. Prohibited Acts shall be guilty of a Class __________ misdemeanor and upon a first conviction thereof shall be punished by a fine of not less than $__________ nor more than $__________ or by imprisonment for not more than __________ months, or both.  Upon a subsequent conviction thereof, he or she shall be punished by a fine of not less than $__________ nor more than $__________ or by imprisonment for up to __________, or both.

24.2.	Felonies. – Any person who:

- intentionally violates any provisions of this Act or regulations under it;

- is convicted under the misdemeanor provisions of Section 24(a) more than three times in a two-year period; or

- uses or has in his or her possession a device which has been altered to facilitate fraud shall be guilty of a Class __________ felony and upon a first offense shall be punished by a fine of not less than $__________, or by imprisonment for not more than __________, or both.

(Added 1989)

Section 25.  Restraining Order and Injunction

The Director is authorized to apply to any court of competent jurisdiction for a restraining order, or a temporary or permanent injunction, restraining any person from violating any provision of this Act.

(Retitled 1989)

Section 26.  Presumptive Evidence

Whenever there shall exist a weight or measure or weighing or measuring device in or about any place in which or from which buying or selling is commonly carried on, there shall be a rebuttable presumption that such weight or measure or weighing or measuring device is regularly used for the business purposes of that place.

###### Section 27.  Separability Provision

If any provision of this Act is declared unconstitutional, or the applicability thereof to any person or circumstance is held invalid, the constitutionality of the remainder of the Act and the applicability thereof to other persons and circumstances shall not be affected thereby.

###### Section 28.  Repeal of Conflicting Laws

All laws and parts of laws contrary to or inconsistent with the provisions of this Act are repealed except as to offenses committed, liabilities incurred, and claims made there under prior to the effective date of this Act.

###### Section 29.  Regulations to be Unaffected by Repeal of Prior Enabling Statute

The adoption of this Act or any of its provisions shall not affect any regulations promulgated pursuant to the authority of any earlier enabling statute unless inconsistent with this Act or modified or revoked by the Director.

###### Section 30.  Effective Date

This Act shall become effective on __________.

THIS PAGE INTENTIONALLY LEFT BLANK