Q U A L I T Y

M A N U A L

T E M P L A T E

Developed for State Legal

Metrology Laboratories

G. Diane Lee, Author

NIST Office of Weights and Measures

Gilbert M. Ugiansky, Ph.D., Chief

Gaithersburg, MD 20899

U.S. DEPARTMENT OF COMMERCE

Ronald H. Brown, Secretary

TECHNOLOGY ADMINISTRATION

Mary L. Good, Under Secretary for Technology

NATIONAL INSTITUTE OF STANDARDS

AND TECHNOLOGY

Arati Prabhakar, Director

5802

N I S T I R

March 1996

Preface

The National Institute of Standards and Technology (NIST), Office of Weights and Measures (OWM) provides support to state metrology laboratories through training and accreditation.  The accreditation program is a means of recognizing that the laboratory has the capability to perform reliable measurements and that the metrologist has fulfilled the requirements and submitted the data requested by OWM.  In 1991 the accreditation program was revised  to conform to ISO/IEC Guide 25, “General Requirements for the Competence of Calibration and Testing Laboratories.”  To assist the state laboratories in conforming to the standard, training on ISO/IEC Guide 25 and a training version of the Quality Manual Template (Template) were provided in 1993 at the six Regional Measurement Assurance Program (RMAP) metrology meetings.  The Template was developed further and additional training was provided  at the 1995 RMAP meetings and at the 1995 Annual Meeting of the National Conference on Weights and Measures (NCWM).

Laboratories conforming to ISO 9000 standards must have a documented quality system.  ISO/IEC Guide 25 and ANSI/NCSL Z540-1, “Calibration Laboratories and Measuring and Test Equipment- General Requirements,” provide specific guidance for establishing a quality system for calibration laboratories.  A major requirement of these standards is documentation of the quality system.

This Quality Manual Template was designed for use by state legal metrology laboratories as a guide to documenting a quality system.  The Template meets the requirements of ISO/IEC Guide 25, ANSI/NCSL Z540-1, and NIST Handbook 143, “State Weights and Measures Laboratories  Program Handbook” and has been reviewed by the National Voluntary Laboratory Accreditation Program (NVLAP) for conformance to NIST Handbook 150, “NVLAP Procedures and General Requirements.”

State legal metrology laboratories are encouraged to follow the Template and adapt it to their specific operations.   This Template may also be adapted for other calibration laboratory systems.

Acknowledgments

This Quality Manual Template (Template) was developed through review of various state metrology quality manuals.  In addition, we received numerous helpful comments and suggestions from state metrologists when the training version of the Template was presented at the RMAP meetings in 1993, and again when the Template was presented at the 1995 RMAP meetings.  The NVLAP review of  the Template also provided useful input.   Special thanks go to all who have contributed  to the development of this manual and especially to:

Georgia Harris, of the NIST OWM State Laboratory Program, for developing the training version of the Template, which was a key document in the development of the Template; for providing guidance with ISO standards, planning and review of the Template.

Staff of the NIST National Voluntary Laboratory Accreditation Program (NVLAP) for their review and comments, which provided input to the development of this Template.

Joseph Rothleder, of the California Department of Food and Agriculture Metrology Laboratory, for providing a draft of the California metrology laboratory quality manual, which was used as a guide for structuring the Template, and for his helpful comments and suggestions.

Laboratory Staff of the Minnesota Department of Public Service Metrology Laboratory, for use of the  Minnesota quality manual.  Excerpts of the Minnesota manual were used during training and served as input in the development of the Template.

Introduction

The Quality Manual Template (Template) has two sections: (1) the Introduction and (2) the Template.

The introduction provides information to assist the user with understanding the benefits of documenting the quality system.  It covers critical elements of the quality system to prevent possible deficiencies and provides guidelines on how to use the Template.  The introduction contains the following four sections:

C	Quality Manual Objectives

C	Documentation of the Quality System

C	Critical Elements of the Quality Manual Template

C	How to Use the Quality Manual Template.

The Template is an example of a typical state metrology laboratory quality manual.  It contains the essential sections of a quality manual, appendices, and sample forms that can be used by the laboratory.

1.	Quality Manual Objectives

There are a number of reasons for maintaining a laboratory quality manual. This section addresses several of the major ones.

Document the quality system.  A quality manual is documentation of the laboratory’s quality system in accordance with specific guidelines, i.e., ISO/IEC Guide 25, and ANSI/NCSL Z540-1.  Documenting the quality system provides means for ensuring uniformity and consistency in the system.

Obtain management commitment.  Management commitment is essential in maintaining a quality system.  A quality manual must define and document management’s policy, objectives, and commitment to quality.  Management with executive responsibility must adopt and sign the quality manual, providing evidence of management commitment to the quality system.  The cover page of this Template includes an area for management’s signature of adoption.

State metrology laboratories must also complete NIST Handbook 143, Appendix C, Part 1, Internal Assessment and Management Review Outline, at least annually.  Management must sign this form as documented evidence of management review of the quality system.

Provide potential for consistent quality.   Periodic review and update of the quality manual allows for continuous improvement and consistent quality.  The quality manual update will in part be a result of the internal audits performed.

State metrology laboratories should follow the requirements of NIST Handbook 143 (NIST HB 143) for conducting internal audits of the quality system.  Appendix C, Part 2, Laboratory Assessment Checklist, of NIST HB 143 should be used as a general checklist for the audit, and the laboratory should include audit information on specific technical areas of the laboratory.

Meet accreditation and registration requirements.  The quality manual should meet the  requirements for which the laboratory is seeking conformance.  This Template  conforms to the ISO/IEC Guide 25 and ANSI/NCSL Z540-1 standards for calibration and testing laboratories.

2.	Documentation of the Quality System

The quality system can be described in terms of a documentation hierarchy consisting of four levels: (1) quality manual and policy; (2) general procedures (administrative procedures); (3) specific procedures (measurement procedures); and (4) forms, records, and work instructions.  All levels are included or referenced in the quality manual.  The following are helpful hints in documenting the quality system.

Determine the scope and limitations of the quality system.  The scope of the quality system is determined by the measurement services that the laboratory offers.  Most state metrology laboratories offer measurement services in mass, volume, and length.

The limits associated with each measurement service, such as the uncertainties and applications for each measurement service, must be addressed in the quality  manual.  Appendix C, “Measurement Parameters,” provides a table listing the measurement parameters, range, application, and uncertainties.

Gather all information related to quality policy and procedures.  Quality policy information includes management’s commitment to quality, state laws, and general laboratory policy.  This information should be included in the appropriate sections of the quality manual.

Administrative and measurement-related procedures should be referenced in the quality manual and placed in a separate manual to decrease the time associated with the control and maintenance of the quality manual.

Identify missing elements in the quality manual.  Reviewing the ISO standards and the Template  will guide the laboratory in identifying any missing elements in the quality manual.

Follow generally accepted manuals or outlines. Following a manual that has been approved minimizes the risks of excluding standard and/or accreditation requirements.

Review and adopt the manual.  State metrology laboratories may submit initial drafts of their quality manuals to OWM for review and comments.  A controlled copy (see Section 3) should be sent to OWM after final revisions are made and the manual is adopted by management.

3.	Critical Elements of the Quality Manual Template

This Template was circulated to the state metrology laboratories and other interested parties for evaluation against current systems and to determine likely deficiencies.  The most widely received comments pertained to the following sections: Laboratory Facilities and Environment, Software Quality Assurance, Document Control, Calibration of Balances and Environmental Equipment, Records, and Handling and Tracking of Test Items.  The following information should enable the laboratory to correct possible deficiencies.

Laboratory Facilities and Environment.  The facilities and environmental conditions of a state metrology laboratory play an important role in the quality of measurement that is produced. As such the laboratory must be maintained in a manner which would not negatively affect the integrity of measurements and provisions must be made for the appropriate monitoring of environmental conditions.  The Template addresses how the laboratory will be maintained and cites the use of applicable sections of NCSL Recommended Practice #7.  This section references an appendix that provides an area to include a diagram of the facilities along with the dimensional measurements.  The Template also addresses the minimum level of records for effective monitoring of environmental conditions in the laboratory.

Software Quality Assurance.  Most state metrology laboratories rarely perform lengthy calculations or generate reports in longhand.  The computer is relied upon for many, if not all, computational and administrative activities.  Therefore, it is very important to ensure that the information derived from software programs is accurate, stored data is protected, and security is provided to limit access and prevent loss of information.  Software Quality Assurance is covered in Section 10, Calibration Test Methods and Procedures, of the Template.  Procedures and a form for verification of the software are referenced in this section and the form is included in the appendices.

Document Control.  All state metrology laboratories maintain forms, records, and documents to assist with recording measurements, administrative activities, quality issues, and reporting results.  The laboratory must have a documented system for document control to ensure that current documents and forms are in use and/or distributed, and records are properly maintained.

This Template references procedures and appendices for document control and contains policy for the control of the quality manual.

C	Forms/Documents.  Forms must have a form number, revision number,  and date. Documents such as the quality manual should contain revision numbers and dates on each page, a document distribution list should be maintained, and the quality manual must contain a signature of adoption.

Appendix N, “Document Control,” is a table that includes: a list of documents and forms, document and form numbers, persons responsible for review, and revision date.  Section 3, “Quality Policy,” addresses document control for the quality manual.

C	Records.  The location, identification, and length of retention of all records must be documented.  Section 12, “Records,” includes a table that contains a list of records, record locations, and retention times.

Calibration of Balances and Environmental Equipment.  State metrology laboratories use balances for comparison of mass standards traceable to NIST to determine the value and uncertainty of unknown weights.  In this situation, the balance is used for comparison only; therefore, calibration would not be necessary.  The balance may be serviced periodically, and this should be documented. If  a laboratory is directly reading the balance results (comparing unknown weights to the balance weights), the internal weights of the balance must be calibrated and verified on a periodic basis.

Air buoyancy corrections are applied to mass calibration measurements, and temperature is included in calculations for volume and length measurements.  The integrity of these measurements relies partly on data obtained from laboratory environmental equipment.  If this equipment is not accurate, the measurements are adversely affected.  Therefore, it is important to ensure that environmental equipment is operating effectively, through periodic calibration.

Calibration of equipment directly affects the integrity of a measurement and hence must be performed by the national laboratory (NIST in the United States) or by a calibration laboratory whose traceability to the national laboratory has been validated through an accreditation process.

The Template addresses calibration issues in Sections 8 and 9.  Section 8, “Standards, Equipment and Associated Apparatus,” includes information on labeling equipment with the calibration status.

Section 9, “Measurement Traceability and Calibration,” references procedures for setting and changing calibration intervals of equipment.

Records.  This Template categorizes records as administrative, measurement-related, and process measurement assurance.  Test results, which are measurement-related records, are usually forwarded to the customer as a  Report of Test or Calibration Report in a format designed by the laboratory.  Section 13, “Calibration Certificates and Test Reports,” covers the specific information that must be included in all calibration test reports. Other records that must be maintained include records of calibration and maintenance of standards, and equipment and software verification. The Template contains forms which can be used to document this information and maintain them in the laboratory files.

Handling and Tracking of Test Items.  The laboratory must ensure that artifacts submitted for test are handled properly and have proper storage and security.  Establishing and documenting this system can help prevent loss, damage, and theft, and provide a means for tracking the test item during and after test.  Section 11, “ Handling and Storage of Calibration and Test Items,” documents the use of work logs and work orders.  This section also references procedures for review of incoming work, receipt and retention of the calibration items, and procedures for returning completed test items.  Appendix O, “Forms,” is referenced and contains a work log form which can be used in the laboratory.

4.	How To Use The Quality Manual Template

The Quality Manual Template consists of 18 Sections, followed by Appendices A through O.  Sections 1 through 16 directly relate to ISO/IEC Guide 25 and are cross-referenced in the table of contents.  Sections 17 and 18 address Site Security and Safety which are areas of specific importance to state metrology laboratories.  The Template also contains a Table of Appendices, which is cross-referenced to sections in the manual, and a table which cross-references the Template, ISO/IEC Guide 25, and NIST Handbook 150.  This manual is available on disk or can be downloaded from the OWM (WAMIS) Bulletin Board (301/869-1665).

State metrology laboratories and other interested parties should use this Template in conjunction with NIST Handbook 143, “State Laboratory Program Handbook,” (state metrology laboratories only); ISO/IEC Guide 25; and ANSI/NCSL Z540-1.  Using the table of content’s cross-reference to ISO/IEC Guide 25, the laboratory should review each section to determine if the documented systems in the Template represent the system of the laboratory.  Sections of the manual should be edited to represent the specific quality system of the laboratory.  The laboratory must be mindful that many areas of the Template follow ISO/IEC Guide 25, ANSI/NCSL Z540-1 and/or NIST Handbook 143.  In a situation where the laboratory’s policy or procedures differ from these standards (NIST Handbook 143 applies to state metrology laboratories only), the laboratory's policy or procedures must be evaluated to ensure conformance to the standard.

Quality Manual Template

State Calibration Laboratory

12345 Some St.

Special City, ST 54321

Adoption:

| QUALITY MANUAL (Based on ISO/IEC Guide 25) |  |
| --- | --- |
| Issued under the authority of the director of the State Bureau of Standards pursuant to Statute CXY. |  |
| Adopted by: |  |
| Title/Position: |  |
| Signature: |  |
| Date: |  |

Document Control:

| Issue Date |  |  |
| --- | --- | --- |
| Issued to |  |  |
| Controlled | Y | N |
| Copy No. |  |  |

Controlled issues of the Quality Manual may not be copied.

Controlled copies of this quality manual are controlled in their entirety as updates or revisions are made.

Revisions to the appendices are controlled  and each distributed separately as updates and revisions are made.

Revisions to the appendices are controlled and maintained in the laboratory.  Updates or revisions to appendices must be requested.

MANUAL REVISIONS

| DATE | PAGE NO. | PARAGRAPH NO. | COMMENTS | APPROVAL: |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | NAME | SIGNATURE |

APPENDICES REVISIONS

| DATE | PAGE NO. | PARAGRAPH NO. | COMMENTS | APPROVAL: |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | NAME | SIGNATURE |

| [State] QUALITY MANUAL Table of Contents |  |  |  |  |
| --- | --- | --- | --- | --- |
| Section | ISO/IEC Guide 25 Clause | Title | Date Revision | Page |
| 1 | 5.2h | Scope and Parameters | 1996.1 | 13 |
| 2 | 2; 3 | References and Definitions | 1996.1 | 14-15 |
| 3 | 5.1; 5.2a, r | Quality Policy | 1996.1 | 16-17 |
| 4 | 4; 5.2b, c, f | Organization and Management | 1996.1 | 18-19 |
| 5 | 5; 5.2n, s | Quality System, Audit and Review (Audit & Assessment) | 1996.1 | 20-21 |
| 6 | 6; 5.2e | Personnel | 1996.1 | 22-23 |
| 7 | 7 | Laboratory Facilities and Environment | 1996.1 | 24-25 |
| 8 | 8; 5.2l, m | Standards, Equipment and Associated Apparatus | 1996.1 | 26-27 |
| 9 | 9; 5.2g, n | Measurement Traceability and Calibration | 1996.1 | 28-29 |
| 10 | 10; 5.2j, o, p | Calibration Test Methods and Procedures | 1996.1 | 30-31 |
| 11 | 11; 5.2k,i | Handling and Storage of Calibration & Test Items | 1996.1 | 32 |
| 12 | 12; 5.2d, f | Records | 1996.1 | 33-35 |
| 13 | 13; 5.2f | Calibration Certificates and Test Reports | 1996.1 | 36-37 |
| 14 | 14; 13.3 | Subcontracting | 1996.1 | 38 |
| 15 | 15 | Outside Support Services and Supplies | 1996.1 | 39 |
| 16 | 16; 5.2o, q; 5.5 | Complaints and Corrective Action | 1996.1 | 40 |
| 17 | 7.5; 11.3 | Site Security | 1996.1 | 41 |
| 18 | ----- | Safety | 1996.1 | 42 |

| Table of Appendices |  |  |  |
| --- | --- | --- | --- |
| Item | Section Reference | Title | Page |
| A | 2 | Definitions | 44-46 |
| B | 4, 5 | Organization Charts | 47-48 |
| C | 1, 3 | Measurement Parameters | 49-50 |
| D | 5, 7 | Diagram of Facilities | 51 |
| E | 5, 7 | Environmental Conditions | 52 |
| F | 5, 8 | Equipment and Materials | 53-54 |
| G | 5, 8 | Standards and Reference Materials | 55-56 |
| H | 3, 5, 7, 8, 9, 10, 11, 15, 16 | Procedures List (Measurement Procedures; Administrative Procedures) | 57-60 |
| I | 5, 9 | Assessment of Uncertainties | 61 |
| J | 5, 9 | Control Charts List | 62 |
| K | 5, 9 | Proficiency Tests | 63 |
| L | 5, 6, 9 | Personnel Training and Competency | 64 |
| M | 4, 5, 6 | Job Descriptions and/or Duty Statements | 65-67 |
| N | 3, 5, 18 | Document Control | 68-69 |
| O | 5, 7, 8, 10, 11, 12, 16 | Forms (Blank) - Training Record - Calibration and Maintenance Log (Standards and Equipment) - Software Program Verification - Weighing Equipment Assessment - Proficiency Test Results - Audit Log - Work Log - Complaints/Corrective Action - Environmental Conditions/Deviations Log - Subcontractors and Outside Suppliers Verification Log | 71 72 73 74 75 76 77 78 79 80 |

| Quality Manual Cross-Referenced to ISO/IEC Guide 25 and NIST HB 150 |  |  |  |
| --- | --- | --- | --- |
| QM Section | Title | ISO/IEC Guide 25 Clause | NIST HB 150 285.33 (c)(2) |
| 1 | Scope and Parameters | 5.2h | (viii) |
| 2 | References and Definitions | 2, 3 | 285.4, 285.5 |
| 3 | Quality Policy | 5.1; 5.2a, r | (I), (xviii); 285.33 (c)(1) |
| 4 | Organization and Management | 4; 5.2b, c, f | (ii), (iii), (vi), 285.33 (b) |
| 5 | Quality System, Audit and Review (Audit & Assessment) | 5; 5.2n, s | (xiv), (xix); 285.33(c) |
| 6 | Personnel | 6; 5.2e | (v), 285.33 (d) |
| 7 | Laboratory Facilities and Environment | 7 | 285.33 (e) |
| 8 | Standards, Equipment and Associated Apparatus | 8; 5.2l, m | (xii), (xiii); 285.33 (f) |
| 9 | Measurement Traceability and Calibration | 9; 5.2g, n | (vii), (xiv), (xxi), (xxii); 285.33(g) |
| 10 | Calibration Test Methods and Procedures | 10; 5.2j, o, p | (x), (xvi), (xv); 285.33 (h) |
| 11 | Handling and Storage of Calibration & Test Items | 11; 5.2k, I | (ix), (xi); 285.33(I) |
| 12 | Records | 12; 5.2d, f | (iv), (vi) 285.33 (j) |
| 13 | Calibration Certificates and Test Reports | 13; 5.2f | (vi), (xx); 285.8; 285.33(k) |
| 14 | Subcontracting | 14; 13.3 | 285.33 (l); 285.33(k)(3) |
| 15 | Outside Support Services and Supplies | 15 | 285.33(m) |
| 16 | Complaints and Corrective Action | 16; 5.2o, q; 5.5 | (xvii), (xv); 285.33(c)(5); 285.33(n) |
| 17 | Site Security | 7.5; 11.3 | 285.33(e)(5); 285.33(I)(3) |
| 18 | Safety | ----- | ----- |

| Scope and Parameters | SECTION 1 Page 1 of 1 Rev. 1 Date: February 1996 |
| --- | --- |

1.0	Scope and Parameters

1.1	Scope

1.1.1	This Quality Manual describes the quality assurance program used in the             State metrology laboratory and sets out the established requirements to competently and effectively achieve the program objectives of the State metrology laboratory.  The measurement program objectives of the laboratory are:

1.1.1.1	Maintain the State standards of mass, length, volume, temperature, frequency, and density; (NOTE: add any other parameters here) and

1.1.1.2	Provide reliable metrological measurement services suited to the needs of weights and measures staff and industry.

1.1.2	All primary standards have been calibrated by the National Institute of Standards and Technology (NIST), or by a NIST-accredited laboratory and remain traceable to NIST through continuous internal surveillance and external comparison.  All measurements and calibration values produced by the metrology laboratory are traceable to NIST.

1.2	Parameters

1.2.1	Measurements, calibrations, and tests performed by the metrology laboratory resulting in certificates, reports, or other summarizing statements are limited to the test parameters listed in Appendix C.

1.2.2	The laboratory quality manual, based on ISO/IEC Guide 25, used in conjunction with applicable portions of the reference documents maintained in the laboratory, constitute the laboratory's quality control system applicable to the test parameters listed in Appendix C.

| References and Definitions | SECTION 2 Page 1 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

2.0	References and Definitions

All critical references described in this quality manual are maintained on file in the laboratory and are accessible to all laboratory staff and management.

2.1	Critical References

2.1.1	ANSI/NCSL Z540-1, 1994, Calibration Laboratories and Measuring and Test Equipment - General Requirements.

2.1.2	ISO/IEC Guide 25, 1990, third edition,  General Requirements for the Competence of Calibration and Testing Laboratories.

2.1.3	National Institute of Standards and Technology (NIST) Handbook (HB) 143, 1985 (or later version if available), State Weights and Measures Laboratories Program Handbook.  John K. Taylor and Henry V. Oppermann.

2.1.4	NIST HB 145, 1986 (or later version if available), Handbook for the Quality Assurance of Metrological Measurements.  John K. Taylor and Henry V. Oppermann.

2.1.5	NIST HB 150, 1994,  National Voluntary Laboratory Accreditation Program (NVLAP) Procedures and General Requirements.  James L. Cigler and Vanda R. White

2.1.6	NIST HB 150-2 (Draft), 1995, National Voluntary Laboratory Accreditation Program (NVLAP) Calibration Laboratories Technical Guide.  Jon M. Crickenberger

2.1.7	NIST Technical Note 1297, 1994, Guidelines for Evaluating and Expressing the Uncertainty of NIST Measurement Results.  Barry N. Taylor and Chris E. Kuyatt.

2.1.8	National Conference of Standards Laboratories (NCSL), 1993, Recommended Practice (RP) No. 7, Laboratory Design.

2.1.9	NISTIR 5802, 1996, Office of Weights and Measures Quality Manual Template.  G. Diane Lee

2.1.10	Applicable State laws and department policies and guidelines (NOTE: include here State statutes which denote the legal status of the metrology laboratory).

2.2	Additional References

2.2.1	NIST Special Publication 791, 1994, State Weights and Measures Laboratories: State Standards Program Description and Directory.  Georgia L. Harris

2.2.2	NIST HB 130, 1996, Uniform Laws and Regulations.  Joan A. Koenig, Kenneth S. Butcher and Terry L. Grimes

| References and Definitions | SECTION 2 Page 2 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

NOTE: This quality manual was written using the references listed in this section.  The core of this manual was written following the guidelines in ISO/IEC Guide 25 and modified to promote a specific quality assurance program for the laboratory.

2.3	Definitions are contained in Appendix A.

| Quality Policy | SECTION 3 Page 1 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

3.0	Quality Policy

3.1	Policy

The laboratory conducts all measurements under conditions and by using techniques that are conducive to a high degree of reliability and follows generally recognized good laboratory practices as noted in Appendix H.  It is our policy to provide the highest quality measurement services attainable, to clients and field staff, through continuous improvement of the quality system.  Quality in our services is a  constant effort and focus.

3.2	Tests

The laboratory conducts tests for the parameters listed in Appendix C in accordance with the procedures, practices, and conditions (hereafter referred to as "procedures") required, recommended, and/or approved by the National Institute of Standards and Technology (NIST) and/or its affiliated departments, agencies, and organizations (see referenced documents, Section 2). The techniques utilized for specific tests ensure the accuracy, tolerance, precision, traceability and/or uncertainty required for the tests and are within the applicable State administrative guidelines and associated safety and cost-effective considerations.

3.3	Independence

Management ensures that the laboratory is independent from any commercial, financial, or other pressures which might adversely affect the quality of tests and resulting reports.  State policy provides guidelines to ensure laboratory independence.  NOTE: The laboratory should reference the appropriate State policy; i.e. where is it found?

3.4	Accreditation

The laboratory maintains accreditation with the NIST Office of Weights and Measures (OWM)(and the NIST National Voluntary Laboratory Accreditation Program (NVLAP), if accredited by NVLAP) to demonstrate conformance to ISO/IEC Guide 25 (through NIST HB 143 and NIST HB 150).  A current accreditation certificate is prominently displayed and maintained on the wall in the laboratory.   The laboratory does not issue calibration reports or other reports for nonaccredited parameters.

3.5	Confidentiality

The laboratory maintains the confidentiality and proprietary rights of all information including type of work performed and results of tests to the extent allowable by State law (NOTE: the laboratory should include the specific State law in this statement and document the law in Section 2 of this Quality Manual).  All laboratory personnel and staff are informed of this policy.  (See Appendix H, AP No. 1, Procedures of Client Confidentiality and Proprietary Rights.)

| Quality Policy | SECTION 3 Page 2 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

3.6	Document Control

3.6.1	General

A detailed list of controlled documents with revision dates, retention periods, and locations is in Appendix N.  The procedures for document control include information on document control

numbers and designated responsibility.  (See procedures list in Appendix H, AP No. 3.)  Section 12, “Records,”  lists the records maintained by the laboratory, the location of the records and retention time.

3.6.2	Authority

The quality manager has the designated authority to modify or update the Quality Manual. The quality manual is annually reviewed and updated as needed by the month of September, so that it has been reviewed prior to the accreditation cycle.   The Laboratory Director is responsible for final approval of all changes made to the quality manual and the revised document takes effect when signed and dated by the Laboratory Director.

This quality manual (along with associated appendices and references) is available to all laboratory staff and management.  Management is responsible for providing the documented quality system and ensuring that all staff familiarize themselves and comply with the policies and procedures established in the manual and associated documentation.

3.6.3	Controlled Copies

Controlled copies of this quality manual are issued to the director, program manager, and NIST OWM and made available to all laboratory personnel. All controlled copies are numbered and updated by the quality manager whenever changes are made.  Recipients of controlled copies are issued the revised quality manual with a cover sheet identifying updates made to the manual.  It is the responsibility of the quality manager to ensure that the most current quality manual is issued and followed by all laboratory and administrative staff.  A list of the names, control numbers, and locations of all controlled copies is maintained in the laboratory files.

3.6.4	Uncontrolled Copies

Uncontrolled copies of the quality manual are issued upon request, and are not  updated unless requests are made.

| Organization and Management | SECTION 4 Page 1 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

4.0	Organization and Management

4.1	Legal Status

The State metrology laboratory is authorized under State statute XYZ (NOTE: enter State statute title, number, and/or article number).  NIST HB 130, Uniform Laws and Regulations, Uniform Weights and Measures Law, Section 12, Powers and Duties of the Director, has been adopted by State statute.  Section 12(a), which is specific to State metrology laboratories, states, "The director maintains traceability of the state standards to the national standards in the possession of the National Institute of Standards and Technology" (Revisions to NIST HB 130 will include requirements for accreditation of State metrology laboratories as a demonstration of traceability).

4.2	Organization

The State metrology laboratory is part of the State (NOTE: enter State and department of the State that the laboratory is a part of).  Authority, interrelation, and responsibilities of all laboratory personnel are on file in the form of job descriptions contained in Appendix M and organizational charts provided in Appendix B.  The responsibilities of Quality Manager and Technical Manager are assigned to designated staff as shown in the organizational chart.

4.3	Responsibility

NOTE: The following is an example of  laboratory personnel responsibilities.  The State laboratories should document the responsibilities of laboratory personnel in this section.

4.3.1	Director

The Director is responsible for the overall compliance of the laboratory to this Quality Manual and is the direct supervisor of the State Metrologist.

4.3.2	Management (Program Manager, however named )

The management of the laboratory:

-implements and enforces the applicable good laboratory practices described in reference documents;

-provides resources, adjusts workloads, and provides training opportunities for laboratory staff to facilitate completion of assigned tasks in a safe work environment consistent with test requirements and personnel capabilities; and

-assigns deputies for both the technical and quality managers in the case of an absence.

| Organization and Management | SECTION 4 Page 2 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

4.3.3	Technical Manager

The technical manager :

-	is a metrologist who has completed the required level of NIST OWM training as specified in accreditation requirements for the level at which the laboratory maintains accreditation;

-is responsible for the overall administrative and technical operations of the laboratory;

-specifies and/or approves all methodologies used;

-implements good laboratory practices by providing instruction and training as needed, develops work plans and procedures, and requires that these be followed in all day-to-day operations;

-assigns only competent personnel to complete tests;

-attests, by signature, to the validity of all laboratory tests and reports (a list of Approved Signatories is maintained in the laboratory (see Section 12, “Records”); and

-ensures continued accreditation of the laboratory.

4.3.4	Quality Manager

The quality manager :

-is a metrologist who has completed the required level of NIST OWM training as specified in accreditation requirements for the level at which the laboratory maintains accreditation;

-coordinates internal audits of the laboratory in accordance with Section 5 of this quality manual;

-maintains, analyzes, and updates statistical data and/or control charts;

-participates in available and relevant proficiency tests, round-robins, and/or interlaboratory collaborative studies;

-where necessary, identifies, develops, and implements improvement of the laboratory measurement capability to meet the requirements of NIST, department programs, and laboratory clients;

-maintains the quality manual;

-has direct access to management and to the technical manager.

NOTE:	Many state laboratories are limited in staff.  These laboratories are often operated by one person or a part-time person.  In these cases, the responsibilities of both technical and quality manager fall on that person.  Special care and precaution must be taken and documented to ensure that the quality system and measurements are not adversely affected by limited laboratory staff.

| Quality System, Audit and Review | SECTION 5 Page 1 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

5.0	Quality System, Audit and Review

5.1	The laboratory has established and maintains a quality system supporting the tests conducted by the laboratory.  The quality system is described in this quality manual, appendices, and applicable sections of the references named therein. These documents are readily available to all laboratory staff and serve as the basis for evaluating the measurement integrity of tests and associated reports.  The laboratory conducts internal audits or periodic checks carried out by or on behalf of management to ensure that the laboratory's policies and procedures as set out in the quality manual are being followed.

5.2	Quality System

5.2.1	The basic elements of the quality system include the use of:

-Qualified personnel for each measurement (see Section 6; Appendices L, M);

-Supervision and review by management and senior personnel (see Section 4; Appendix B);

-Approved methodology including standard operating procedures, good laboratory practices,  and good measurement practices (see Section 10; Appendix H);

- 	Appropriately maintained and calibrated standards, equipment, and associated apparatus (see Section 8; Appendices G and F); and

-Environmentally controlled facilities and/or proper accounting of relevant environmental factors (see Section 7; Appendices D, E).

5.3	Quality Audits and Review

5.3.1	The basic elements of the quality audit and review program include:

-Identification of problems which arise as a result of any client-discovered errors and/or, discrepant results from the analysis of the laboratory test data (see Section 16);

-Evidence from internal audits and statistical control data and/or charts.  Control charts or other measurement control methods are maintained for each of the regular measurement practices (see Appendices J, N); and

-Evidence from external audits, participation in measurement assurance programs, proficiency tests,  round-robins, interlaboratory collaborative experiments, and NIST calibrations (see Appendices J, K,  N).

| Quality System, Audit and Review | SECTION 5 Page 2 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

5.3.3	Review of Accreditation Material

The laboratory submits updated accreditation material annually between October 1 and November 15 to NIST OWM for review.   Accreditation material submitted for review is dependent upon OWM requests and may consist of :

-Management Review checklist;

-Uncertainties charts for each parameter, if updated (see Appendix I);

-Weighing Equipment Assessment, if updated (see Appendix O);

-Proficiency Testing Results, if updated (see Appendix K);

-Personnel Training and Competency Log, if updated (see Appendix L);

-NIST HB 143 Appendix C, Self-Assessment Checklist;

-Measurement Control Data: control charts, LAP 26/27/28, and surveillance tests (see Appendices J, N); and

-Quality Manual, if updated.

5.3.4	Internal Audits

Internal audits (self-assessments) are conducted every 6 months (March and September) to verify that operations continue to comply with the quality system.  Auditors are trained in auditing techniques, have technical insight concerning the laboratory's functions, and are (wherever possible) independent of the activity to be audited.  When audit findings cast doubt on the correctness or validity of the laboratory's calibration or test results, the laboratory manager investigates further and, if warranted, takes immediate action to notify (in writing) any clients whose items may have been affected (see Appendix N, Document Control).

5.3.5	External Audits

External audits (on-site assessments) are performed by NIST OWM at variable intervals to verify that the laboratory's operations, facilities, equipment, standards, and staff continue to comply with the requirements of NIST HB 143 and this Quality Manual.  All external and internal audit and accreditation review findings, and any corrective actions that arise from them, are promptly settled within the agreed time, documented by the quality manager, and maintained in the laboratory files.

| Personnel | SECTION 6 Page 1 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

6.0	Personnel

6.1	Laboratory staff are selected for employment based on professional qualifications, including education and relevant experience. Staffing is sufficient to maintain the timely processing of the client workload, laboratory internal monitoring, quality control, and traceability activities required for NIST accreditation. Job descriptions for laboratory personnel are contained in Appendix M (or are on file in the laboratory).

6.2	Adequately trained staff are a key factor in good measurement. Laboratory metrologists have the necessary background in the physical sciences to ensure comprehension of the laboratory tests and operations.  Laboratory management and supervision provide formal training and on-the-job training for the laboratory staff.  NIST training requirements are noted in Section 6.4.  Training is documented and maintained in Appendix L.

6.3	The laboratory supervisor(s), utilizing staff resources to meet policy goals:

6.3.1	Implements and applies the procedures contained in the referenced documents as listed in Section 2;

6.3.2	Provides ongoing training to ensure proficiency in calibration and testing;

6.3.3	Develops work plan schedules and requires that the staff follow the procedures in day-to-day operations; and

6.3.4	Assigns tasks based on personnel training and verified competence.

| Personnel | SECTION 6 Page 2 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

6.4	NIST OWM Training Requirements

NIST OWM requires and provides training to all State metrology laboratories.  Laboratory staff meet the training requirements of  OWM and all training is documented and maintained in the laboratory (see Appendix L).  Laboratory staff conducting measurements in the following specific areas have successfully completed the noted courses and assignments:

6.4.1	Legal Metrology Activities (Tolerance Testing)

-Basic Laboratory Metrology Seminar (2 weeks);

-Basic Laboratory Auditing Program (LAP) problems; and

-Attendance at Regional Measurement Assurance Program meeting annually.

6.4.2	Calibration Testing

-All of the above (6.4.1);

-Intermediate Laboratory Metrology Seminar (1 week); and

-Intermediate LAP problems.

6.4.3	Advanced Mass or Volume Calibration Testing

-All of the above (6.4.1, 6.4.2);

-Advanced Mass and/or Advanced Volume Measurements Seminar; and

-Advanced LAP problems (consist of advanced measurement control program).

6.4.4	Noncompliance to NIST Training Requirements

Note: In the event that NIST training requirements are not met, or trained staff is not available, the laboratory management and supervision, as a minimum, ensure that the appropriate seminar is scheduled and attended as soon as possible and LAP problems are completed within 6 months of attendance.  Laboratory management will request in writing from OWM a conditional accreditation for legal metrology activities.  The laboratory does not issue calibration reports or other reports for nonaccredited parameters.

| Laboratory Facilities and Environment | SECTION 7 Page 1 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

7.0	Laboratory Facilities and Environment

7.1	Facilities and Conditions

7.1.1	The laboratory facilities are maintained, where applicable, in accordance with NCSL Recommended Practice No. 7.  The design of the laboratory does not adversely affect the measurement integrity and supports good laboratory practices.  See Appendix D for a diagram of the laboratory facilities.

7.1.2	The laboratory facilities, calibration and test areas, energy sources, lighting, heating, and ventilation facilitate proper performance of calibrations and tests.  The laboratory ensures that dust, electromagnetic interference, humidity, line voltage, temperature, sound and vibration levels (i.e., limit use of computer types which cause vibration or temperature changes that adversely affect the measurement) are appropriate for specific measurement results and associated uncertainties.

7.1.3	Environmental conditions maintained by the laboratory, listed in Appendix E, are within the limits recommended in NIST HB 143.  The environment in the laboratory where testing is performed does not invalidate results nor adversely affect the accuracy or uncertainty of the measurement. Measurements are not made if environmental conditions deviate from those stated.  Laboratory staff ensure adequate conditions in the facility by items listed below.

7.1.3.1	Verify that air conditioning, lighting, heating, and ventilation are controlled and monitored, to the level needed for each type of test.

7.1.3.2	Maintain good housekeeping practices to promote a clean, uncluttered laboratory according to procedures listed in Appendix H, AP No. 11.

7.1.3.3	Have sufficient space to minimize the risk of injury to staff and/or damage to standards or equipment due to activities around test setup.

7.1.3.4	Maintain a convenient and efficient work environment with effective separation of incompatible activities.

7.1.3.5	Limit the amount of paper products used or stored in sensitive and/or clean areas to prevent dust contamination.

| Laboratory Facilities and Environment | SECTION 7 Page 2 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

7.2	Environmental Records

7.2.1	Small Mass Calibration Laboratory:

Environmental conditions are recorded continuously with the use of a strip chart recording device which monitors the temperature and humidity of the laboratory.

7.2.2	Mass Tolerance, Volume, Length, Temperature, Frequency, and Density Testing Laboratory:

Environmental conditions are recorded continuously with the use of a strip chart recording device which monitors the temperature and humidity of the laboratory, or

The laboratory maintains a log of environmental conditions completed during all testing periods (see  Appendix O, Environmental Log form), or

Environmental conditions are recorded on all data sheets whenever appropriate.  Procedures state exceptions to this requirement.

7.2.3	The laboratory documents deviations and corrective actions when environmental conditions are not within conditions specified by the laboratory in Appendix E.  (See Appendix O, Complaints/ Corrective Actions form.)

NOTE:  Monitoring devices are periodically verified against accurate and traceable standards, but are not used to apply corrections that affect the quality of measurements.  Devices which do affect the quality of measurement are addressed in Section 9.3.3.

| Standards, Equipment and Associated Apparatus | SECTION 8 Page 1 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

8.0	Standards, Equipment, and Associated Apparatus

8.1	Laboratory standards, equipment, and associated apparatus are suitable for the correct  performance of calibrations and tests and are maintained in accordance with the requirements of NIST HB 145, equipment maintenance and operational manuals, and this quality manual to include protection from dirt, dust, corrosion, and other causes of deterioration.  The technical manager investigates any equipment or standards which are suspect in contributing to out-of-control conditions (see Appendix G, Standards List; Appendix F, Equipment List).  Records of corrective actions for discrepancies are maintained in the laboratory (see Section 12 “Records”; Appendix O, Forms).

Maintenance and calibration records for equipment and standards include (see Section 12, “Records”; Appendix O, Forms) the following as appropriate:

8.1.1	Item name and manufacturer; model, serial, and other identification numbers;

8.1.2	Date and condition of receipt, date placed in service, and current location;

8.1.3	History of calibration, maintenance, malfunction, modification, and repair;

8.1.4	Calibration status and recertification date;

8.1.5	Identification of any software affecting the calibration and quality assurance of the program; and

8.1.6	Copy of manufacturer's instructions, where available.

8.2	Operation and Maintenance

8.2.1	Equipment and Associated Apparatus

8.2.1.1	Laboratory equipment is properly maintained in accordance with  procedures for calibration, verification, and maintenance (see Appendix H, AP No. 14).  These procedures are located in the laboratory files.

8.2.1.2	Any item of the equipment which has been subjected to overloading or mishandling or which gives suspect results or has been shown by verification or otherwise to be defective is taken out of service, clearly identified, and whenever possible stored at a specified place until it has been repaired and shown by calibration, verification, or test to perform satisfactorily.  The laboratory examines the possible effect of defected equipment on any previous calibrations.

| Standards, Equipment and Associated Apparatus | SECTION 8 Page 2 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

8.2.1.3	Operation manuals and instructions for proper maintenance of equipment are available to the staff and located in the laboratory.

8.2.1.4	Newly installed equipment and software programs are tested to verify that they perform satisfactorily before they are placed into service.  Documentation of this verification is maintained in the laboratory (see Section 12, “Records,” and Appendix O, Forms).  The laboratory maintains procedures for testing newly installed equipment (see Appendix H, AP No. 14).

8.2.1.5	Equipment is used or operated only when in a safe and reliable condition, by personnel who have been trained and are qualified.

8.2.2	Standards (see Section 9, “Measurement Traceability and Calibration”)

8.2.2.1	To maintain integrity of the standards, all maintenance operations are performed according to documented procedures (see Appendix H, AP No. 13) and the laboratory standards are:

-Selected for use according to the level of precision, accuracy, and uncertainty required;

-Limited in access and use to trained and authorized laboratory staff only;

-Handled and safely stored according to good laboratory practices (see Appendix H); and

-Used in round-robin exercises annually, for verification purposes.

8.2.2.2	Primary standards are calibrated by the National Institute of Standards and Technology or by a NIST accredited laboratory and NIST calibration reports are maintained in the laboratory files.

8.2.2.3	Secondary standards are calibrated by comparison to primary standards, using  calibration procedures of  NIST HB 145.  Calibration reports are maintained in the laboratory files.

8.2.2.4	Working standards are calibrated by comparison to secondary standards using calibration procedures of NIST HB 145.  Calibration reports are maintained in the laboratory files.

| Measurement Traceability and Calibration | SECTION 9 Page 1 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

9.0	Measurement Traceability and Calibration

9.1	Policy

9.1.1	Standards and measuring and test equipment significantly affecting the measurement integrity of tests conducted by the laboratory are monitored for stability as part of the measurement control program.  Standards and equipment are calibrated and/or verified before use to ensure the recall or removal from service of any equipment or standards that are unreliable or that have exceeded the calibration interval if established.

9.2	Measurement Traceability

9.2.1	The established program of calibration and verification of measuring and test equipment is designed to ensure that the measurements made by the laboratory are traceable to national standards of measurement.  All laboratory reports contain a statement that the standards used are traceable to the National Institute of Standards and Technology.

9.2.2	To provide external evidence of traceability, the laboratory participates in measurement control programs, proficiency tests, and other interlaboratory and collaborative experiments (see Appendices L and K).

9.3	Calibration/Verification	(See Procedure for Calibration Intervals, AP No. 13)

9.3.1	Standards (Calibration)

9.3.1.1	Primary standards are calibrated by the National Institute of Standards and Technology or by a NIST-accredited laboratory.

9.3.1.2	Primary standards are continuously monitored by the measurement control programs established in the laboratory and are under the custody of trained metrologists (Note: According to State requirements, if any, document here who maintains custody).  These standards are generally stable and not subject to change.  Any changes that occur are generally identified by the laboratory and the NIST OWM State audit program in ample time for corrective action.

9.3.1.3	Standards are recalibrated if there is damage to the standards or any significant change is observed in the measurement control program.

| Measurement Traceability and Calibration | SECTION 9 Page 2 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

9.3.2	Standards (Verification)

9.3.2.1	Continuous verification of standards, through the measurement control program, ensures required measurement integrity of test and includes:

-Statistical data from check standards and/or control charts (see Appendix J); and

-Results from interlaboratory comparisons and/or proficiency tests (see Appendix K).

9.3.2.2	Measurement assurance procedures for verification of standards are maintained in the laboratory files (see Appendix H, AP No. 13)

9.3.3	Measuring and Test Equipment (M &TE) (Calibration)

9.3.3.1	Equipment used with nominal values and corrections is calibrated by NIST or by a calibration laboratory whose traceability to NIST has been validated through an accreditation process.  A calibration interval is established for the equipment (i.e., environmental equipment, balances).

9.3.3.2	When appropriate, each item of equipment is labeled, marked, or otherwise identified to indicate its calibration status.

-All equipment used with nominal values and corrections is labeled indicating the calibration status.  Examples of this equipment include thermometers, barometers, hygrometers, and balances.

-All equipment used to facilitate comparison between a standard artifact and an unknown is labeled "Used for Comparisons Only - No Calibration Required."

9.3.3.3	Procedures for setting and changing M&TE calibration intervals are maintained in the laboratory files (see Appendix H, AP No. 14).

9.3.3.4	Calibration of equipment is conducted at a frequency to ensure that the equipment remains in tolerance during its use in the laboratory.  Frequency of calibration is based on a review of calibration, maintenance, and repair history.  Reviews are conducted by the technical manager and records are maintained with internal audit records in the laboratory files.

9.4	Uncertainty of Measurement

9.4.1	Where applicable, the ISO Guide to the Expression of Uncertainty  in Measurement, 1993; the G. Harris paper, Implementation of the Guide to the Expression of Uncertainty in Measurement for Calibration Laboratories, 1994; and NIST HB 145, 1986, are used as the basis for expression of uncertainty in measurement (see Appendix I, Assessment of Uncertainties). Note: NIST HB 145 is being revised to meet the new ISO requirements for the expression of uncertainty in measurement.  The revised document will be adopted by the laboratory upon completion.

| Calibration Test Methods and Procedures | SECTION 10 Page 1 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

10.0	Calibration Test Methods and Procedures

10.1	The laboratory maintains administrative and measurement-related procedures in the laboratory files.  These procedures are available to the laboratory staff and are followed to ensure the integrity of administrative duties, calibrations, and test results (see Appendix H, Procedures List).  Equipment manuals, operating instructions, reference data, specifications, and tolerance tables relevant to laboratory calibrations and tests are maintained in an up-to-date file in the laboratory, and are readily available.

10.2	When NIST-documented procedures are unavailable, the laboratory uses procedures published in relevant scientific texts or journals from reputable technical organizations.   When documented or published procedures are unavailable or when deviations from documented procedures occur, procedures for a specific test are developed, validated, and agreed to by the client and the laboratory manager.  The calibration or test report states the procedure used.  Records regarding departures from documented policies and procedures or from standard specifications are initiated by laboratory management and maintained in the laboratory files (see Section 12, “Records”; Appendix O, Forms).  Procedures for departure from documented policies and procedures are maintained in the laboratory (see Appendix H, AP No. 15).

10.3	Measurement-Related Procedures

10.3.1	The laboratory follows the procedures in NIST HB 145 (procedures manual) and other documented procedures relevant to laboratory tests and related activities.  All measurement processes are monitored for statistical control (see Appendix H, Measurement Procedures List).

10.3.2	Calibration procedures contain the measurement range and tolerance or uncertainty of each item being calibrated or tested.  The procedures also contain the following:

10.3.2.1	Generic description of the measurement standards; and

10.3.2.2	Equipment needed with the required parameter, range, tolerances, or uncertainties and specifications.

10.3.3	Appropriate checks of data, calculations, and test results are made by the laboratory staff to ensure that  procedures are followed and to ensure measurement integrity.  All measurement processes are monitored for statistical control.  Records are maintained regarding feedback and corrective action, whenever testing discrepancies are detected. (See Section 12, “Records”; Appendix O, Forms.)   Procedures for feedback and corrective actions are maintained in the laboratory (see Appendix H, AP No. 6).

| Calibration Test Methods and Procedures | SECTION 10 Page 2 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

10.4	Administrative-Related Procedures

10.4.1	Administrative procedures developed by the laboratory comply with the documented standard operating procedures of NIST HB 145 and are maintained in the laboratory.  The administrative procedures ensure that the overall operations of the laboratory promote the quality and integrity of calibration and/or test results and test items.

10.4.2	Procedures are maintained by the technical manager for the purchase, reception and storage of consumable materials used for the technical operations of the laboratory (see Appendix H, AP No. 9).

10.5	Computer Software Procedures

10.5.1	Where computers are involved in data recording, retrieval, processing, calculation, analysis, or reporting, the laboratory ensures that:

-	The requirements of this manual are maintained; and

-	Computer software has been documented and verified for use (see Section 12, “Records”; Appendix O, Forms).

10.5.2	Procedures are established to:

-Protect the integrity of stored data;

-Provide limited access to maintain security of the programs in use;

-	Back-up programs and records; and

-Provide information for revising the software if updates occur.

10.5.3	Procedures for software documentation and verification are maintained by the technical manager and located in the laboratory files (see Appendix H, AP No. 8).

| Handling and Storage of Calibration and Test Items | SECTION 11 Page 1 of 1 Rev. 1 Date: February 1996 |
| --- | --- |

11.0	Handling and Storage of Calibration and Test Items

11.1	Items received for test or calibration are recorded in a laboratory work log and assigned a number which uniquely identifies the item during its stay in the laboratory.   Work logs are maintained in the laboratory.  A work order is completed which includes: the item or items received for test, name of company submitting the test items, and date of receipt.  Work orders are attached to and, if possible, are kept with the test item during its stay in the laboratory .  (See Appendix O, Forms, and Section 12, “Records.”)

11.2	Incoming test items are evaluated by appropriate laboratory staff to ensure that the appropriate standards, equipment, staff, facilities, and procedures necessary to perform testing are available.  Procedures for the review of all incoming work are maintained in the laboratory files (see Appendix H, AP No. 5).

11.3	Prior to testing incoming items, the laboratory communicates to the client any significant abnormalities including:

11.3.1	Departures from required standard conditions and necessary preparations;

11.3.2	Doubt as to the item's suitability for calibration or test;

11.3.3	Nonconformance of the test item with the description provided by the client; and

11.3.4	Unspecified calibration or test requirements.

11.4	The laboratory handles, prepares, and stores test items in its custody in a safe manner to protect them from loss, deterioration, damage, and destruction of required chains of evidence.  Documented procedures for the receipt and retention of the calibration items are maintained in the laboratory files (see Appendix H, AP No. 5).

11.5	Upon completion of calibration or testing, the test items are safely returned to the client to ensure that the integrity of the laboratory is preserved at all times (see Appendix H, AP No. 5, Procedures for Returning Completed Test Items, Including Shipping).

11.6	Calibration or test items to be held secure (for any reason, including safety, value, to enable check calibrations or test to be performed later) are stored and secured to protect item condition and integrity.

| Records | SECTION 12 Page 1 of 3 Rev. 1 Date: February 1996 |
| --- | --- |

12.0	Records

12.1	The laboratory ensures the safety and security of records.  Records include those with information required by regulation, or associated with original test observations, calculations, and reported results.  Measurement data is recorded in permanent form, in bound notebooks, or on standard forms on file (see Appendix O, Forms).  Permanent ink is used to record the actual data and no erasures or white-outs are made.  Corrections are made to data by drawing a single line through the entry and initialing the change, with a note as to why the change was made.  Measurement records contain sufficient detail to, if necessary, permit the repetition of measurements.  Records of original data include the following:

12.1.1	Standard Operating Procedure (SOP) used;

12.1.2	Description  of, and reason for, any deviation from the SOP;

12.1.3	Identity of the personnel making the measurement;

12.1.4	Identity and description of objects under test;

12.1.5	Identity of equipment or apparatus used;

12.1.6	Identity of standards used and reference to traceability;

12.1.7	Date of test;

12.1.8	State test number;

12.1.9	Environmental data during test, when applicable (see Section 7 “Laboratory Facilities and Environment”);

12.1.10	Work order; and

12.1.11	Purchase order number, if received.

12.2	Records, including those in computer files, are accessible to authorized personnel only.    Computer files are backed-up for protection against loss.

12.3	Records maintained by the laboratory are in three categories:  administrative, measurement-related, and process measurement assurance.  The laboratory maintains and retains the following records in the locations stated for the specified amount of time  (Note: State laboratories should include a statement addressing the specific retention time of records according to State or laboratory policy):

| Records | SECTION 12 Page 2 of 3 Rev. 1 Date: February 1996 |
| --- | --- |

| Administrative |  |  |
| --- | --- | --- |
| List of Records | Location | Retention Time |
| Audit |  |  |
| Complaints/ Feedback/Corrective Action |  |  |
| Deviations from Accepted Procedure |  |  |
| Management Review |  |  |
| List of Approved Signatories |  |  |
| Subcontractors and Outside Suppliers |  |  |
| Personnel Training and Competency |  |  |
| Controlled Document Distribution List |  |  |

| Measurement-Related |  |  |
| --- | --- | --- |
| List of Records | Location | Retention Time |
| Calibration and Test Reports/Supplements to Calibration and Test Reports |  |  |
| Original Calibration/Test Data |  |  |
| Environmental Conditions/Deviations Log |  |  |
| Calibration and Maintenance (Standards and Equipment) |  |  |
| Software Verification |  |  |
| [NIST] Primary Standards Calibration Reports |  |  |
| Weighing Equipment Assessment |  |  |
| Assessment of Uncertainties |  |  |
| Proficiency Test Results |  |  |

| Records | SECTION 12 Page 3 of 3 Rev. 1 Date: February 1996 |
| --- | --- |

| Process Measurement Assurance |  |  |
| --- | --- | --- |
| List of Records | Location | Retention Time |
| LAP 26/27 Surveillance |  |  |
| LAP 28 Surveillance |  |  |
| Mass Code Surveillance |  |  |
| Control Charts |  |  |
| Range Charts |  |  |

| Calibration Certificates and Test Reports | SECTION 13 Page 1 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

13.0	Calibration Certificates and Test Reports

13.1	The laboratory provides the client with reports of tests that, prior to final signing by the approved signatories responsible for the validity of the results, are reviewed to correct inconsistencies in the report, supporting data, and calculations (see Section 12, “Records” for location of calibration and test reports).

13.2	The laboratory follows NIST HB 150, Sec. 285.8 for policy regarding the use of the NVLAP logo, if accredited by NVLAP.  The NVLAP logo is used on calibration and test reports and is accompanied by "Accredited by the National Voluntary Laboratory Accreditation Program for the specific scope of accreditation under Lab Code XXXX," if the laboratory is accredited for that parameter, range, and scope.

13.3	Calibration certificates and test reports include the following information:

13.3.1	A report title;

13.3.2	Name and address of the laboratory, and location where the test was conducted if different from the laboratory;

13.3.3	Unique report number on every page of each report which shows the total number of pages of the report;

13.3.4	Name and address of client;

13.3.5	Item identification including: description, manufacturer, model, and serial number (where available);

13.3.6	Calibration date;

13.3.7	Condition and characterization of the item (where relevant);

13.3.8	Identification of the test method used;

13.3.9	Deviations, additions, or exclusions to the test method and other relevant information including environmental conditions existing during test (when applicable);

13.3.10	Tables, graphs, and other supporting information when necessary for the interpretation of the report;

13.3.11	Measurement results, calibration values, a statement of uncertainty, and, where appropriate, accuracy and tolerance conformity;

13.3.12	Date of issue and signature of the technical manager, metrologist, or other official who accepts responsibility for the validity of the results and the content of the report;

| Calibration Certificates and Test Reports | SECTION 13 Page 2 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

13.3.13	where relevant, a statement that the report relates only to the items listed in the report "at the time of test";

13.3.14	Clear identification of reported results of calibrations or test if performed by subcontractors;

13.3.15	Reference to sampling procedures where relevant;

13.3.16	A statement that the certificate shall not be reproduced except in full, without the written approval of the laboratory;

13.3.17	Statement that the client shall not use the report to claim endorsement by NIST, OWM,  NVLAP or any agency of the U.S. Government;

13.3.18	Signature of an approved signatory for all test and calibration reports endorsed with the accreditation status or NVLAP logo (see Section 12 “Records”, List of Approved Signatories);

13.3.19	Special limitations of use; and

13.3.20	Traceability statement.

13.4	The laboratory notifies its customers in writing of any events which cast doubt on the validity of the results given in any calibration report or amendment to a report.

13.5	Amendments to calibration certificates and test reports after issue are made in the form of a further document or data transfer that includes the statement "Supplement to Calibration Certificate [or Test Report]".  Records of these documents are maintained by the laboratory staff and located in the laboratory files (see Section 12, "Records", Calibration and Test Reports/Supplements to Calibration and Test Reports).

| Subcontracting | SECTION 14 Page 1 of 1 Rev. 1 Date: February 1996 |
| --- | --- |

14.0	Subcontracting

14.1	The laboratory performs calibrations or tests within its documented capability.  The laboratory does not subcontract.

OR

14.2	The laboratory subcontracts in the special circumstances where technical, safety, or efficiency issues dictate. Subcontracting is only conducted with OWM and/or NVLAP accredited laboratories, which meet the requirements of NIST HB 143 and/or NIST HB 150.  The laboratory maintains a list of all subcontracting laboratories and work.

14.3	If the laboratory is NVLAP-accredited and the subcontractor is NVLAP-accredited, all reports which contain data from subcontracted work include the following statement:

"This report contains data which were produced by a subcontracted laboratory ACCREDITED (NVLAP LAB CODE) for the calibration or test methods performed.”

| Outside Support Services and Supplies | SECTION 15 Page 1 of 1 Rev. 1 Date: February 1996 |
| --- | --- |

15.0	Outside Support Services and Supplies

15.1	The laboratory, where specifications of outside services and supplies are relevant to the measurement integrity of tests, uses services and supplies of adequate quality.  The laboratory maintains procedures for the purchase, storage, and evaluation of supplies and services (see Appendix H, AP No. 9).

15.2	Where assurance of the quality of outside support services or supplies is unavailable, the laboratory uses these items only after they have been inspected or otherwise verified for adequate quality.  Records of inspections, verifications, and suppliers are maintained in the laboratory (see Section 12, “Records”).

| Complaints and Corrective Action | SECTION 16 Page 1 of 1 Rev. 1 Date: February 1996 |
| --- | --- |

16.0	Complaints and Corrective Action

16.1	In the event of a complaint, adverse findings during audits, or any other circumstance which raises doubts concerning the laboratory's competence or compliance with required procedures, the laboratory ensures that  those areas of activity and responsibility involved are promptly investigated.  A resolution of the adverse situation is promptly sought and, where necessary, retesting is conducted.  Procedures for handling complaints are maintained in the laboratory (see Appendix H, AP No. 16).

16.2	The laboratory quality manager examines all documents and records associated with complaints and the laboratory manager investigates adverse audit findings and other circumstances.  This investigation seeks to identify specific root causes and initiate necessary corrective action.

16.3	Records of adverse circumstances and the actions taken by the laboratory to resolve the problem to prevent future occurrences are maintained (see Section 12, “Records” and Appendix O, Forms).

| Site Security | SECTION 17 Page 1 of 1 Rev. 1 Date: February 1996 |
| --- | --- |

17	Site Security

17.1	The laboratory is located within facilities of the State.  Security of the State facilities is the responsibility of the building manager and State contracted security and is subject to the security requirements of the weights and Measures department. The laboratory manager is responsible for security directly related to the laboratory and designates the specific duties of on-site security to the laboratory staff.  Security of the laboratory premises includes the following:

17.1.1	Locking laboratory doors, in specific areas, when not in use;

17.1.2	Securing all doors and perimeter at the close of the day;

17.1.3	Notifying building security of disturbances and suspicious activity as appropriate;

17.1.4	Securing entrances to laboratory when disturbance during testing affects the measurement integrity;  and

17.1.5	Securing all areas where standards and equipment are stored or maintained.

17.2	Access

17.2.1	Access to and use of all measurement areas is controlled and defined by the technical manager.  The laboratory maintains the current access list as follows: (NOTE: Replace Area 1 and Area 2 with a specific measurement area and list the staff (laboratory or support) that has access to the specific area).

| Access |  |  |  |
| --- | --- | --- | --- |
| Staff | Area 1 | Area 2 | Computers |
| 1 | X | X | X |
| 2 |  | X | X |
| 3 | X | X |  |

17.2.2	Laboratory building keys are given to administrative staff members and laboratory personnel.  Laboratory room keys are only given to laboratory personnel.

17.2.3	Cleaning staff are only allowed supervised access to the laboratory during normal working hours.

| Safety | SECTION 18 Page 1 of 1 Rev. 1 Date: February 1996 |
| --- | --- |

18.0	Safety

18.1	Safe working conditions are prerequisite to good laboratory practices.  Laboratory personnel are instructed in safe working practices and are encouraged to look for hazardous conditions as well as recommend and implement accident prevention.

18.2	The laboratory maintains a safety manual on file in the laboratory.  The safety manual is available to all  laboratory staff and management and contains all safety regulations associated with the overall laboratory operations (see Appendix N, Document Control).

18.3	Management provides safe working conditions, complies with safety regulations and, along with supervisors, assures that the staff comply with these regulations.

18.4	It is the responsibility of all staff to be familiar with and comply with all safety guidelines and requirements.  The laboratory staff takes proper precautions in the laboratory as described in the safety manual.

Appendices

| Definitions | Appendix A Page 1 of 3 Rev. 1 Date: February 1996 |
| --- | --- |

Accreditation - a formal recognition that a laboratory is competent to carry out specific tests or calibrations or types of tests or calibrations.

Calibration - a set of operations which establish, under specified conditions, the relationship between values indicated by a measuring instrument or measuring system, or values represented by material measure, and the corresponding known values of a measurand.  Also:  comparison of a measurement standard or instrument with another standard or instrument to detect, correlate, report, or eliminate by adjustment any inaccuracy of the compared.

Corrective Action - an action taken to eliminate the causes of an existing deficiency or other undesirable situation in order to prevent recurrence.

Deficiency - the nonfulfillment of  NIST OWM (if appropriate NVLAP) conditions and/or criteria for accreditation.

External Audit - Appraisal of a calibration or testing laboratory by an outside body, using specified criteria and checklists to evaluate compliance for accreditation.

Good Laboratory Practices (GLP) - an acceptable way to perform some basic operation or activity in a laboratory that is known or believed to influence the quality of its outputs.  GLPs ordinarily are essentially independent of the measurement techniques used.

Good Measurement Practices (GMP) - an acceptable way to perform some operation associated with a specific measurement technique that is known or believed to influence the quality of the measurement.

Internal Audit - the process of self appraisal of a calibration or testing laboratory using specified criteria and checklists to evaluate compliance for accreditation; may be used as a quality management review as well.

Interlaboratory Comparisons - Organization, performance, and evaluation of calibrations or tests on the same or similar items or materials by two or more laboratories in accordance with predetermined conditions.

Length Laboratory - a specific area, within a metrology laboratory, that is used solely for calibration or tolerance testing of length standards.

Mass Laboratory - a specific area, within a metrology laboratory, that is used solely for calibration or tolerance testing of mass standards or test weights; generally divided into small mass (# 10 kg) and large mass (> 10 kg) areas.

Measurement Assurance - a process to ensure adequate measurement results that may include, but is not limited to:  1) use of good experimental design principles so that the entire measurement process, its components, and relevant influence factors can be well characterized, monitored, and controlled; 2) complete experimental characterization of the measurement process uncertainty including statistical variations, contributions from all known or suspected influence factors, imported uncertainties, and the propagation of uncertainties throughout the measurement process; and 3) continuously monitoring the performance and state of statistical control of the measurement process with proven statistical process control techniques including the measurement of well-characterized check standards along with the normal workload and the use of appropriate control charts.

NVLAP - the NIST National Voluntary Laboratory Accreditation Program

| Definitions | Appendix A Page 2 of 3 Rev. 1 Date: February 1996 |
| --- | --- |

On-Site Assessment - a formal examination or official inspection of a calibration or testing laboratory to evaluate its compliance with specific laboratory accreditation criteria.

OWM - the NIST Office of Weights and Measures (Weights and Measures Program).

Preventive Action - an action taken to eliminate the causes of a potential deficiency or other undesirable situation in order to prevent occurrence.

Proficiency Testing - the determination of laboratory performance by means of comparing and evaluating calibrations or tests on the same or similar items or materials by two or more laboratories in accordance with predetermined conditions.

Standard, Primary - a standard that is designated or widely acknowledged as having the highest metrological qualities in a specified field; a substance, object, or scale, the value of which has been established by a reliable source, and which can be accepted (within specified limits) without question when used to establish the value of the same or related property of another similar object or scale.  Note that the primary standard for one user may be a secondary standard for another.

Quality - the totality of features and characteristics of a product or service that bear on its ability to satisfy stated or implied needs.

Quality Audit - a systematic examination or evaluation of the extent to which an entity is capable of fulfilling specified requirements; a systematic and independent examination to determine whether quality activities and related results comply with planned arrangements, whether these arrangements are implemented effectively and are suitable to achieve objectives, and whether the quality control system is operating within acceptable limits.

Standard, Reference - A standard, generally of the highest metrological quality available at a given location, from which measurements made at that location are derived.

RMAP - Regional Measurement Assurance Program jointly operated by OWM and States.

Standard, Secondary  - a standard whose value is assigned by comparison with a primary standard of the same quantity.

Standard, Working - a standard, usually calibrated against a reference standard, which is used routinely to calibrate or check material measures or measuring instruments.

Standard Operating Procedure (SOP) - a procedure adopted for repetitive use when performing a specific measurement or sampling operation.  It may be a standard method or one developed by the user.

| Definitions | Appendix A Page 3 of 3 Rev. 1 Date: February 1996 |
| --- | --- |

Subcontracting - (State metrology laboratories) use of an accredited State metrology laboratory to perform testing on an artifact that was received by another State metrology laboratory, where the results of that testing are included on the report of test issued by the receiving laboratory.

Tolerance Testing - a measurement operation performed to determine whether the actual value and uncertainty of a standard, artifact, or instrument is within a permitted tolerance of its nominal value.

Traceability - the property of a result of a measurement or the value of a standard whereby it can be related to stated references, usually national or international standards, through an unbroken chain of comparisons all having stated

uncertainties.  A measurement assurance process is essential to ensure that the accuracy of the measurement is within the stated limits of uncertainty.

Uncertainty of measurement - parameter, associated with the result of a measurement, that characterizes the dispersion of the values that could reasonably be attributed to the measurand.

Uncertainty, Type A (evaluation of) - Method of evaluation of uncertainty by the statistical analysis of the measurement process.

Uncertainty, Type B (evaluation of) - Method of evaluation of uncertainty by means other than the statistical analysis of the measurement process.

Verification - Confirmation by examination and provision of evidence that specified requirements have been met.

Volume Laboratory - a specific area, within a metrology laboratory, that is used solely for calibration or tolerance testing of  volume standards; generally divided into small volume (# 20 L) and large volume (> 20 L) areas.

| Organizational Chart | Appendix B Page 1 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

| Organizational Chart | Appendix B Page 2 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

| Measurement Parameters | Appendix C Page 1 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

| Parameter | Range/Application | Uncertainty |
| --- | --- | --- |
| Mass | laboratory to specify nominal ranges: e.g., 30 kg to 1 mg; Echelon II |  |
| Echelon I, Extra-fine accuracy | ASTM Class 1, 1.1, OIML Class E1, E2 | 0.25 at 1 kg, and 1 to 5 ppm (< tolerance) |
| Echelon II, Fine accuracy | ASTM Class 2, 3, OIML Class F1, F2 | 1 to 20 ppm ( < ¼ tolerance) |
| Echelon III, Medium accuracy | ASTM Class 4, 5, 6, OIML Class M1, M2, M3 NIST Class F, legal/regulatory enforcement | 20 to 1000 ppm (0.1%) (< 1/10 tolerances) |
| Length | laboratory to specify ranges: e.g., 10 cm to 20 meter tapes |  |
| Tapes, bench method | up to 25 meters (100 ft) | 3 to 5 ppm |
| Tapes, tape method | up to 25 meters (100 ft) | 5 to 10 ppm |
| Rules, direct comparison | up to 45 cm (18 in) | < 100 ppm |
| Volume | laboratory to specify ranges: e.g., 2 Fliter micropipettes to 2000 ml glass flasks |  |
| Gravimetric | syringes, micropipettes, glass standards, and metal provers | 10 to 100 ppm |
| Volume transfer | glass standards and metal provers | < 1000 ppm (0.1%) |
| Temperature | laboratory to specify type and range: e.g., -10 °C to 200 °C; Echelon II |  |
| I | SPRTs | # " 0.005 EC |
| II | Thermistors, Thermocouples | > " 0.005 EC to # " 0.05 EC |
| III | Liquid-in-glass thermometers | > " 0.05 EC to # " 0.20 EC |
| IV | Liquid-in-glass, dial type, pyrometers | > " 0.20 EC to # " 1.0 EC |
| V | infrared sensors, thermographs | > " 1.0 EC to # " 5.0 EC |
| Tuning Forks | as used for law enforcement | 0.05 mph, estimate based on interlaboratory tests |
| Hydrometers | sugar, syrup, petroleum | 0.05 mph, estimate based on interlaboratory test |
| Time | stopwatches used for law enforcement | significantly less than tolerances (2 sec.); estimated at 0.1 ppm for a 24 hr test |

| Measurement Parameters | Appendix C Page 2 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

| Grain Moisture Testing |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameters (types of grain tested) | Method | Low Moisture | Range __% to __% | High Moisture | Range __% to __% | Repeatability |
| Wheat |  |  |  |  |  |  |
| Barley |  |  |  |  |  |  |
| Oats (Including Wild oats) |  |  |  |  |  |  |
| Rye |  |  |  |  |  |  |
| Triticale |  |  |  |  |  |  |
| sorghum |  |  |  |  |  |  |
| soybean |  |  |  |  |  |  |
| mixed grain |  |  |  |  |  |  |
| peas |  |  |  |  |  |  |
| lentils |  |  |  |  |  |  |
| rice |  |  |  |  |  |  |
| corn |  |  |  |  |  |  |
| beans |  |  |  |  |  |  |
| flaxseed |  |  |  |  |  |  |
| safflower |  |  |  |  |  |  |
| sunflower |  |  |  |  |  |  |
| X: Tested N: Not Tested O: Oven Method C: Chemical Method |  |  |  |  |  |  |

| Additional Tests or Laboratory Functions |  |  |
| --- | --- | --- |
| Specific Test(s) | Range of Test | Uncertainty |

| Diagram of Facilities | Appendix D Page 1 of 1 Rev. 1 Date: February 1996 |
| --- | --- |

NOTE:	The above diagram is for demonstration only.  The State laboratory should include a diagram of the laboratory in this appendix.

| Laboratory Dimensions |  |
| --- | --- |
| Laboratory | Dimensions (Length and Width) |

| Environmental Conditions | Appendix E Page 1 of 1 Rev. 1 Date: February 1996 |
| --- | --- |

| Location | Environmental Conditions |  |  |  |
| --- | --- | --- | --- | --- |
|  | Temperature EC | Within " EC | Humidity % | Within " % |

| Equipment and Materials | Appendix F Page 1 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

| Measuring Equipment List |  |  |
| --- | --- | --- |
| Equipment | Model/Serial # | Location |

| Environmental Equipment List |  |  |
| --- | --- | --- |
| Equipment | Model/Serial # | Location |

| Equipment and Materials | Appendix F Page 2 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

| Associated Equipment and Materials List |  |  |
| --- | --- | --- |
| Equipment | Model/Serial # | Location |

| Standards and Reference Materials | Appendix G Page 1 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

| Primary Standards List |  |  |  |
| --- | --- | --- | --- |
| Type | Item | Metric/Avoirdupois | Location |

| Secondary Standards List |  |  |  |
| --- | --- | --- | --- |
| Type | Item | Metric/Avoirdupois | Location |

| Working Standards List |  |  |  |
| --- | --- | --- | --- |
| Type | Item | Metric/Avoirdupois | Location |

| Standards and Reference Materials | Appendix G Page 2 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

| Reference Materials List |  |
| --- | --- |
| Item | Location |

| Procedures List | Appendix H Page 1 of 4 Rev. 1 Date: February 1996 |
| --- | --- |

| Good Laboratory Practices (NIST HB 145, Handbook for the Quality Assurance of Metrological Measurements) |  |
| --- | --- |
| No. | Measurement-Related Procedure |
| GLP No. 1 | Measurement Control for Mass Calibration |
| GLP No. 2 | Minimum Requirement for the Physical Facilities for Weights and Measures Laboratory |
| GLP No. 3 | Care of State Standards |
| GLP No. 4 | Periodic Recalibration of State Standards |
| GLP No. 5 | Laboratory Administration |
| GLP No. 6 | Prerequisites for Testing Measurement Standards and Equipment |
| GLP No. 7 | Calibration Systems Requirements |
| GLP No. 8 | Sealing of Equipment |
| GLP No. 9 | Reporting Mass Values and Uncertainties |
| GLP No. 10 | Purity of Water |
| GLP No. 11 | Painting Weights |
| GLP No. 12 | Laboratory Records |
| GLP No. 13 | Drying Containers |

| Procedures List | Appendix H Page 2 of 4 Rev. 1 Date: February 1996 |
| --- | --- |

| Good Measurement Practices (NIST HB 145, Handbook for the Quality Assurance of Metrological Measurements) |  |
| --- | --- |
| No. | Measurement-Related Procedure |
| GMP No. 1 | Reading Turning Points on an Equal Arm Balance |
| GMP No. 2 | Reading the Center of the Graduations When Using a Microscope |
| GMP No. 3 | Method of Reading a Meniscus |
| GMP No. 4 | Adjusting the Optical Scale on a Single Pan Mechanical Balance |
| GMP No. 5 | Weight Cleaning Procedures |
| GMP No. 6 | Cleaning Metal Volumetric Measures |
| GMP No. 7 | Cleaning Precision Glassware |
| GMP No. 8 | Recommended Form For Reporting Tape Calibrations |
| GMP No. 9 | Equations of Metallic Tapes |
| GMP No. 10 | Weighing Operations |

| Procedures List | Appendix H Page 3 of 4 Rev. 1 Date: February 1996 |
| --- | --- |

| Standard Operating Procedures (NIST HB 145, Handbook for the Quality Assurance of Metrological Measurements) |  |
| --- | --- |
| No. | Measurement-Related Procedures |
| SOP No. 1 | Recommended SOP for Preparation of Calibration/Test Reports |
| SOP No. 2 | Recommended SOP for Applying Air Buoyancy Corrections |
| SOP No. 3 | Recommended SOP for Weighing by Double Substitution Using an Equal-Arm Balance |
| SOP No. 4 | Recommended SOP for Weighing by Double Substitution Using a Single Pan Mechanical Balance, Full Electronic Balance, or a Balance with Digital Indications and Built-in Weights |
| SOP No. 5 | Recommended SOP for using 3-1 Weighing Design (Double Substitution) |
| SOP No. 6 | Recommended SOP for Weighing by Transposition |
| SOP No. 7 | Recommended SOP for Weighing by Single Substitution Using a Single Pan Mechanical Balance, a Full Electronic Balance, or a Balance with Digital Indications and Built-in Weights |
| SOP No. 8 | Recommended SOP for Tolerance Testing of Mass Standards by Modified Substitution |
| SOP No. 9 | Recommended SOP for Control Charts for Calibration of Mass Standards |
| SOP No. 10 | Recommended SOP for Calibration of Rigid Rules |
| SOP No. 11 | Recommended SOP for Calibration of Metal Tapes, Bench Method |
| SOP No. 12 | Recommended SOP for Calibration of Steel Tapes, Tape to Tape Method |
| SOP No. 13 | Recommended SOP for Calibration of Volumetric Ware Using an Electronic Balance |
| SOP No. 14 | Recommended SOP for Gravimetric Calibration of Volumetric Ware Using an Electronic Balance |
| SOP No. 15 | Recommended SOP for Calibration of Intermediate and Large Volume Standards, Gravimetric Method |
| SOP No. 16 | Recommended SOP for Calibration of Measuring Flask, Volume Transfer Method |
| SOP No. 17 | Recommended SOP for Control Charts for Calibration of Measuring Flask, Volume Transfer Method |
| SOP No. 18 | Recommended SOP for Calibration of Graduated Neck Volumetric Field Standards, Volumetric Transfer Method |
| SOP No. 19 | Recommended SOP for Calibration of Large Metal Provers, Volumetric Method |
| SOP No. 20 | Recommended SOP for Control Charts for Calibration of Neck-Type Provers, Volumetric Method |
| SOP No. 21 | Recommended SOP for Calibration of LPG Provers |

| Procedures List | Appendix H Page 4 of 4 Rev. 1 Date: February 1996 |
| --- | --- |

| Laboratory Administrative Procedures |  |
| --- | --- |
| No. | Procedure |
| AP No. 1 | Protection of Client Confidentiality and Proprietary Rights |
| AP No. 2 | Impartial Service |
| AP No. 3 | Document Control |
| AP No. 4 | Ensuring Traceability |
| AP No. 5 | Handling Calibration and Test Items (Incoming inspection and review; Review of new incoming work; Receipt, retention, and return to include work order and work log instructions and packing and shipping instructions ) |
| AP No. 6 | Feedback and Corrective Actions |
| AP No. 7 | Audits and Reviews (Client notification regarding adverse findings) |
| AP No. 8 | Software Data Integrity (Security, access, verification of new software and protection of stored data) |
| AP No. 9 | Purchase, Storage, and Evaluation of Supplies and Services |
| AP No. 10 | Complaints |
| AP No. 11 | Laboratory Housekeeping |
| AP No. 12 | Review and Maintenance of Control Charts (Covered in SOP 9, 17, 20) |
| AP No. 13 | Calibration, Verification, and Maintenance of Standards |
| AP No. 14 | Calibration, Verification, and Maintenance of M&TE (New equipment and calibration intervals) |
| AP No. 15 | Departure from Documented Policies and Procedures |
| AP No. 16 | Investigation of Complaints, Adverse Audit Findings or Discrepancies, and Notifying Clients when Test Results are Affected |

| Assessment of Uncertainties | Appendix I Page 1 of 1 Rev. 1 Date: February 1996 |
| --- | --- |

| Uncertainties Chart for Measurement Parameter________________________ |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Range | Type A Uncertainty | Type B Uncertainty | Combined Uncertainty | Expanded Standard Uncertainty (k = 2) | Comments |

| Control Charts List | Appendix J Page 1 of 1 Rev. 1 Date: February 1996 |
| --- | --- |

| No. | Mass, Length, or Volume Standards Used | Nominal Value | Comments |
| --- | --- | --- | --- |
| ST96CC1 |  |  |  |
| ST96CC2 |  |  |  |
| ST96CC3 |  |  |  |

| Proficiency Tests | Appendix K Page 1 of 1 Rev. 1 Date: February 1996 |
| --- | --- |

| Proficiency Test Results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Range | Accuracy Class | Coordinating Organization | Procedure Used | Operator | Results |  |  |  |
|  |  |  |  |  |  | Passed | Passed with Concerns | Failed | Corrective Action Follow-Up |

| Personnel Training And Competency | Appendix L Page 1 of 1 Rev. 1 Date: February 1996 |
| --- | --- |

| Personnel Training and Competency |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name of Staff Position |  |  |  |  |  |  |  |  |  |
| Training Provider Subject/Topics | Training Dates |  | Hours | Completed |  | Staff |  | Supervisor |  |
|  | From | To |  | Yes | No | Initial | Date | Initial | Date |

| Job Descriptions and/or Duty Statements | Appendix M Page 1 of 3 Rev. 1 Date: February 1996 |
| --- | --- |

Job Description: Metrologist

Note:	Because the responsibilities of the metrologist vary from State to State, this job description should only be used as a guide.  Specific objectives and tasks should be selected from this list or added as appropriate.

Under the direction of the State Director (or Laboratory Supervisor) the Metrologist is directed to meet the following objectives and perform the associated tasks.

Objective - Maintain the State Primary Standards and Measuring Equipment

Tasks:	1.	Take charge of the primary standards and assure their safekeeping.

2.	Perform periodic maintenance on the primary standards including cleaning.

3.	Perform periodic maintenance on the measuring equipment including cleaning and minor adjustments.  Assure that instruments such as precision balances receive routine preventative maintenance by qualified technicians.

4.	Perform measurements intercomparing the primary standards to provide data for control charts.  Analyze the control chart data to establish repeatability limits for measurements using the measuring equipment.  The data is also used to identify significant changes in the primary standards.

5.	Arrange for calibration as needed.

Objective - Maintain the State Working Standards

Tasks:	1.	Take charge of the working standards and assure their safekeeping.

2.	Calibrate (or recalibrate) the working standards against the primary standards to maintain their traceability to the NIST.

3.	Perform periodic maintenance on the working standards including cleaning.

4.	Perform measurements intercomparing the working standards (and primary standards) to provide data for control charts.  Analyze the control chart data to establish repeatability limits for measurements using the measuring equipment.  The data is also used to identify significant changes in the working standards.

| Job Descriptions and/or Duty Statements | Appendix M Page 2 of 3 Rev. 1 Date: February 1996 |
| --- | --- |

Objective - Test and Calibrate Weights and Measures Field Standards

Tasks:	1.	Schedule the workload for the laboratory.  Make assignments for the work performed by the technicians.

2.	Receive field standards submitted for verification.  Identify them and ensure their safekeeping while in the custody of the laboratory.

3.	Select the appropriate working standard, test equipment, and test method appropriate for the field standards submitted.

4.	Perform the measurements necessary to certify the standards submitted for test.  This will also include measurements on check standards used in control charts to monitor system performance.

5. 	Document the tests through test worksheets and a formal Calibration Report or Report of Test. Ensure that all paperwork is correct and ensure that copies are safely maintained for a time period defined in the laboratory's Quality Manual.

6.	Ensure that the field standards are safely returned to the custody of the person or firm that submitted them.

Objective - Test and Calibrate Industry Standards

Tasks:	1.	Schedule the workload for the laboratory.  Make assignments for the work performed by the technicians.

2.	Receive standards submitted for test.  Identify them and ensure safekeeping while in the laboratory custody.

3.	Select the appropriate working or primary standard, test equipment, and test method to test the standards submitted.

4.	Perform the measurements necessary to test the standards submitted for test.  This will also include measurements on check standards used in control charts to monitor system performance.

5. 	Document the tests through test worksheets and a formal Calibration Report or Report of Test. Ensure that all paperwork is correct and ensure that copies are safely maintained for a time period defined in the laboratory's Quality Manual.

6.	Ensure that the standards are safely returned to the custody of the person or firm that submitted them.

| Job Descriptions and/or Duty Statements | Appendix M Page 3 of 3 Rev. 1 Date: February 1996 |
| --- | --- |

Objective - Maintain the Measurement Assurance Programs

Tasks:	1.	Select and calibrate check standards for use in measurement assurance programs.

2.	Perform repeated measurements using the check standards to gather data for control charts as needed.

3.	Construct control charts for each measurement process used in the laboratory.  Evaluate control limits for estimating measurement uncertainties.

4.	Compare new data points to these control limits to monitor the laboratory output.

5.	Document, investigate, and correct any out-of-control condition.  Evaluate the potential impact on clients that may have been affected by the condition.  Notify clients of significant out-of-control conditions so that they may take appropriate action.

Objective - Perform Type Evaluation Examinations (for NTEP)

Tasks:	1.	Schedule the workload of the laboratory.

2.	Receive the devices submitted for test.  Identify them and ensure their safekeeping while in the custody of the laboratory.

3.	Select and perform the appropriate tests on the device submitted following the procedures in the Type Evaluation Checklists.

4. 	Document the tests through test worksheets and prepare the formal Certificate of Conformance document.  Ensure that all paperwork is correct and ensure that copies are safely maintained for a time period defined in the laboratory's Quality Manual.

5.	Ensure that the devices are safely returned to the custody of the person or firm that submitted them.

Objective - Correspondence

Tasks:	1.	Correspond with potential clients regarding the capabilities, schedules, and requirements of the laboratory.

2.	Answer questions regarding metrology, specifications and tolerances of standards and measuring equipment, and other related activities.

3.	Provide technical assistance to enforcement officials and other clients regarding proper use and maintenance of standards.

| Document Control | Appendix N Page 1 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

| No. | Manuals | Responsible for Review | Revision Date |
| --- | --- | --- | --- |
| ST96SM-01 | Safety Manual |  |  |
| ST96QM-01 | Quality Manual |  |  |
| ST96OM-01 | Operations Manual |  |  |

NOTE:  The first 2 digits of the manual control numbers represent the State; the second 2 digits represent the revision year;  the third 2 digits represent the type of manual, e.g., QM = Quality Manual; and the fourth 2 digits represent the revision number.  Controlled documents that are distributed contain an additional digit representing the distribution number.  Records of the distribution numbers are located in the laboratory files.  Manual numbers are examples only.

| No. | Administrative Forms | Responsible for Review | Revision Date |
| --- | --- | --- | --- |
| ST96AF1-01 | Management Review |  |  |
| ST96AF2-01 | Complaints |  |  |
| ST96AF3-01 | Work Orders |  |  |
| ST96AF4-01 | Work Logs |  |  |
| ST96AF5-01 | Subcontractors & Outside Suppliers |  |  |
| ST96AF6-01 | Personnel Training and Competency Log |  |  |
| ST96AF7-01 | Control Document Distribution List |  |  |

NOTE:  The first 2 digits of the form control numbers represent the State; the second 2 digits represent the revision year;  the third 2 digits represent the type of form, e.g., AF = Administrative Form; the next digit identifies the specific form; and the last two digits represent the revision number.  Form numbers are examples only.

| Document Control | Appendix N Page 2 of 2 Rev. 1 Date: February 1996 |
| --- | --- |

| No. | Measurement-Related Forms | Responsible for Review | Revision Date |
| --- | --- | --- | --- |
| ST96MR1-01 | Environmental Conditions/Deviations Log |  |  |
| ST96MR2-01 | Calibration, Maintenance, Verification Log Standards and Equipment |  |  |
| ST96MR3-01 | Weighing Equipment Assessment |  |  |
| ST96MR4-01 | Assessment of Mass Calibration Uncertainties |  |  |
| ST96MR5-01 | Software Verification |  |  |
| ST96MR6-01 | Audits |  |  |
| ST96MR7-01 | Observation and Data Sheet |  |  |
| ST96MR8-01 | Calibration Report |  |  |
| ST96MR9-01 | Test Report |  |  |

| No. | Process Measurement Assurance Forms | Responsible for Review | Revision Date |
| --- | --- | --- | --- |
| ST96MA1-01 | LAP 26/27 |  |  |
| ST96MA2-01 | LAP 28 |  |  |
| ST96MA3-01 | Proficiency Testing Results |  |  |

Forms

Note: The following are examples of forms that can be used in the laboratory.

| Forms | Appendix O Page 1 of 10 Rev. 1 Date: February 1996 |
| --- | --- |

| Personnel Training and Competency |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name of Staff Position |  |  |  |  |  |  |  |  |  |
| Training Provider Subject/Topics | Training Dates |  | Hours | Completed |  | Staff |  | Supervisor |  |
|  | From | To |  | Yes | No | Initial | Date | Initial | Date |

| Form No. ST96AF6-01 | Rev. Date: February 1996 |  |
| --- | --- | --- |

| Forms | Appendix O Page 2 of 10 Rev. 1 Date: February 1996 |
| --- | --- |

| Calibration And Maintenance Log (Standards And Equipment) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Standard or Equipment | Date |  | Condition on Receipt | Manufacturer | Model/Serial | Cal. Status/ Maintenance Date | Current Location | Comments: (Maintenance, Malfunction, Modification, Repair) |
|  | Received | In-Service |  |  |  |  |  |  |

| Form No. ST96MR2-01 | Rev. Date: February 1996 |
| --- | --- |

| Forms | Appendix O Page 3 of 10 Rev. 1 Date: February 1996 |
| --- | --- |

| Software Program Verification |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Software Program/Version | File Name / File size | Manufacturer / Developer | Verification: Data Set Runs |  | Verification Date | Verified by (Initials) | Comments |
|  |  |  | Pass | Fail |  |  |  |

| Form No. ST96MR5-01 | Rev. Date: February 1996 |
| --- | --- |

| Forms | Appendix O Page 4 of 10 Rev. 1 Date: February 1996 |
| --- | --- |

| Weighing Equipment Assessment |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Item Information |  |  |  |  | Accepted |  |
| Range | Manufacturer | Model | Method | Standard Deviation | Yes | Comments Attached |

| Form No. ST96MR3-01 | Rev. Date: February 1996 |
| --- | --- |

| Forms | Appendix O Page 5 of 10 Rev. 1 Date: February 1996 |
| --- | --- |

| Proficiency Test Results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Range | Accuracy Class | Coordinating Organization | Procedure Used | Operator | Results |  |  |  |
|  |  |  |  |  |  | Passed | Passed with Concerns | Failed | Corrective Action Follow-Up |

| Form No. ST96MA3-01 | Rev. Date: February 1996 |
| --- | --- |

| Forms | Appendix O Page 6 of 10 Rev. 1 Date: February 1996 |
| --- | --- |

| Audit Log |  |  |  |
| --- | --- | --- | --- |
| Audit Description | Date Conducted | Location of Audit Report | Audit Findings / Corrective Actions |

| Form No. ST96MR6-01 | Rev. Date: February 1996 |
| --- | --- |

| Forms | Appendix O Page 7 of 10 Rev. 1 Date: February 1996 |
| --- | --- |

| Work Log |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test No. | Date Received | Item (s) Received | Test Requested | Requested By | Date Tested | Date Completed | Date Returned | Comments |

| Form No. ST96AF4-01 | Rev. Date: February 1996 |
| --- | --- |

| Forms | Appendix O Page 8 of 10 Rev. 1 Date: February 1996 |
| --- | --- |

| Complaints / Corrective Action |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subject of Complaint / Discrepancy | Date | Response Date | Reviewed By | Actions Taken / Results |

| Form No. ST96AF2-01 | Rev. Date: February 1996 |
| --- | --- |

| Forms | Appendix O Page 9 of 10 Rev. 1 Date: February 1996 |
| --- | --- |

| Environmental Conditions/Deviations Log |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location | Date | Time | Temperature EC | Within " EC | Humidity % | Within " % | Pressure mm Hg | Within " mm Hg |

| Form No. ST96MR1-01 | Rev. Date: February 1996 |
| --- | --- |

| Forms | Appendix O Page 10 of 10 Rev. 1 Date: February 1996 |
| --- | --- |

| Subcontractors and Outside Suppliers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| State Metrology Laboratory Performing Test | Test Item(s) | Date | Laboratory Accredited |  | Supplier | Type of Supply | Verification |  |  |
|  |  |  | Yes | No |  |  | Type of Verification performed | Approved |  |
|  |  |  |  |  |  |  |  | Yes | No |

| Form No. ST96AF5-01 | Rev. Date: February 1996 |
| --- | --- |